3GPP TSG RAN WG4 #112bis													     R4-2416475

Hefei, CN, October 14th – 18th, 2024

Agenda Item:		5.17.1

Source:				Inmarsat, Viasat

Title:	Discussion on UE-to-UE Coexistence

Document for:	Discussion and Decision

# Background

The intent of this paper is to provide information on the coexistence study to be carried out for S-band, and addressing some of the existing coexistence requirements.

# Coexistence Parameters

For coexistence studies carries out in TR 38.863, the minimum distance between NTN UEs and TN UEs has been a minimum of 2*ISD. At RAN4 #112 Maastricht a point was raised which suggested that there may be scenarios in which a TN UE will be much, much closer to an NTN UE, in which case these coexistence scenario assumptions are not correct.

Observation 1: The TR 38.863 for NTN-TN coexistence analysis, the TN-NTN separation may be far too high for scenarios that may practically be seen with a real system. A much lower value should be derived.

## 2.1 TN Network UE Density

As per TR 38.863, the 19 cell cluster has a per-cell density with a certain number of NR and NB-IoT UEs per cell.

NB-IoT

- 3 multi tone UE

- 12 single tone 15kHz UE

- 48 single tone 3.75kHz UE

NR

- 3 NR UEs per cell

Total 66 UEs per cell.

This is applicable for both the urban and rural ISD, 750 and 7500m respectively. This equates to 37.3 UE/km^2 for urban and 0.374 UE/km^2 for rural.

Observation 2: The TR 38.863 for NTN-TN coexistence analysis, the TN aggregate TN user density yields 66 UEs per cell in a 19 cell cluster.

## 2.2 NTN Network UE Density

In TR 38.863 NTN UEs are distributed at a minimum distance from the edge of the TN cell cluster, at a minimum distance of 2*ISD. There are scenarios in which it is expected that an NTN UE will be much closer to a TN UE. There is no guarantee that two operators of a TN network will provide coverage to the same area in certain edge of coverage scenarios.

As a result there may be a scenario in which a TN UE is in very close proximity to an NTN UE, which makes the requirements for a coexistence scenario much more strict.

In order to assess the probability an NTN UE is below a certain distance from a TN UE, an NTN UE density must be defined. Using the SA1 TS 22.261 scenario parameters for handheld NTN, an assumed 100 UE/km^2 is used, with an active rate of 1.5%.

For a 50km satellite cell, this would result in 11781 UEs. This is similar to the ITU report M.2292-0 which specifies a rural UE density which is very similar to that of the 22.261 assumption.

Proposal 1: An NTN UE density of 100/km^2 with an active rate of 1.5% shall be used for the coexistence study for scenarios in which TN-NTN UE separation distance will be less than the TR 38.863 2*ISD separation.

## 2.3 Probability Study

The probability that the NTN-TN UE separation distance is below 5m and 10m was calculated with varying levels of NTN-TN cell overlap (e.g. 0% overlap means no NTN UEs are within the TN cell cluster, 100% overlap means NTN UEs are distributed with the given NTN UE density throughout the entire TN cell cluster). Each test was run 100 times to create a meaningful average.

### 2.3.1 TN Victim, NTN Aggressor

For the case of TN being the victim and NTN being the aggressor, for each TN UE, the shortest distance to an NTN UE was recorded and several parameters were produced: total average separation distance, average minimum separation distance and the probability that a UE is below the 5m/10m separation distance.

| Scenario | Total Average Separation Distance [m] | Average Minimum Separation Distance [m] | Probability UE is Below Given Separation Distance |  |
| --- | --- | --- | --- | --- |
| Minimum Separation [m] | - | - | 5 | 10 |
| 750m ISD, 100% overlap | 408 | 11 | 0.000665 | 0.002537 |
| 750m ISD, 75% overlap | 432 | 12 | 0.000399 | 0.002517 |
| 750m ISD, 50% overlap | 566 | 14 | 0.00048 | 0.002036 |
| 750m ISD, 25% overlap | 925 | 18 | 0.000353 | 0.001234 |
| 750m ISD, 0% overlap | 1596 | 106 | 0 | 0 |
| 7500m ISD, 100% overlap | 408 | 12 | 0.000096 | 0.000431 |
| 7500m ISD, 75% overlap | 601 | 12 | 0.000056 | 0.000407 |
| 7500m ISD, 50% overlap | 1929 | 15 | 0.000112 | 0.000343 |
| 7500m ISD, 25% overlap | 5577 | 19 | 0.000040 | 0.000191 |
| 7500m ISD, 0% overlap | 12688 | 221 | 0 | 0 |

For both ISD scenarios where the TN UEs are the victims, the probability the UE is below a given separation distance is constant for overlaps above 25%. For overlaps of 25% a reduction in probability can be seen when compared to 50% overlap, and for 0% overlap, no UEs were below the given separation distance.

Observation  3:  For scenarios where TN UE is the victim, the probability of a separation of 5m or lower, never exceeds 0.000665

### 2.3.2 NTN Victim, TN Aggressor

For the case of NTN being the victim and TN being the aggressor, for each NTN UE, the shortest distance to a TN UE was recorded and several parameters were produced: total average separation distance, average minimum separation distance and the probability that a UE is below the 5m/10m separation distance.

| Scenario | Total Average Separation Distance [m] | Average Minimum Separation Distance [m] | Probability UE is Below Given Separation Distance |  |
| --- | --- | --- | --- | --- |
| Minimum Separation [m] | - | - | 5 | 10 |
| 750m ISD, 100% overlap | 29634 | 11 | 0.000588 | 0.002242 |
| 750m ISD, 75% overlap | 29627 | 12 | 0.000674 | 0.002082 |
| 750m ISD, 50% overlap | 29671 | 14 | 0.000430 | 0.001738 |
| 750m ISD, 25% overlap | 29723 | 18 | 0.000213 | 0.001011 |
| 750m ISD, 0% overlap | 29806 | 109 | 0 | 0 |
| 7500m ISD, 100% overlap | 3746 | 10 | 0.000832 | 0.002938 |
| 7500m ISD, 75% overlap | 3844 | 11 | 0.000511 | 0.002195 |
| 7500m ISD, 50% overlap | 4207 | 13 | 0.000699 | 0.002348 |
| 7500m ISD, 25% overlap | 5041 | 18 | 0.000345 | 0.001493 |
| 7500m ISD, 0% overlap | 7349 | 221 | 0 | 0 |

Observation  4:  For scenarios where NTN UE is the victim, the probability of a separation of 5m or lower, never exceeds 0.000832

# Conclusions

Observation 1: The TR 38.863 for NTN-TN coexistence analysis, the TN-NTN separation may be far too high for scenarios that may practically be seen with a real system. A much lower value should be derived.

Observation 2: The TR 38.863 for NTN-TN coexistence analysis, the TN aggregate TN user density yields 66 UEs per cell in a 19 cell cluster.

Proposal 1: An NTN UE density of 100/km^2 with an active rate of 1.5% shall be used for the coexistence study for scenarios in which TN-NTN UE separation distance will be less than the TR 38.863 2*ISD separation.

Observation  3:  For scenarios where TN UE is the victim, the probability of a separation of 5m or lower, never exceeds 0.000665

Observation  4:  For scenarios where NTN UE is the victim, the probability of a separation of 5m or lower, never exceeds 0.000832