3GPP TSG-RAN4 Meeting #112bis	R4-2416476

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.13.2

Source:	Keysight Technologies UK Ltd

Title:	On absolute probe locations

Document for:	Approval

# Introduction

This contribution is briefly addressing discussions held in RAN4#112 regarding the absolute probe locations.

# Discussion

During the Multi-Rx SI discussions, the initial finding was that absolute probe locations are required for the baseline test system [1], [2].

Issue 1-2-2: Probe locations for UE RF testing - Proposals - Option 1 (Keysight): For measurement setup option 2a, absolute probe locations must be defined to guarantee different system vendors yield the same UE RF test results - Agreement: Option 1 is agreed

Later, however, based on new agreements related to positioner sweeps/ranges and probes located in the xz plane, the need for absolute probe locations for the Multi-Rx test system, targeted to be the baseline system for STxMP as well, was considered unnecessary, e.g., [3]

Proposal 1: Consider the previous agreement for absolute probe locations for UE RF testing unnecessary for multi-Rx UE RF testing using the proposed test system and testing methodology. Proposal 2: Consider any 2 AoA RRM test system with probes in the xz plane and the required relative angular offsets for Multi-Rx testing suitable for Multi-Rx OTA spherical coverage test cases.

and [4]

Issue 1-4-1: Absolute probe locations - Proposals - Option 1 (R4-2309469): Consider the previous agreement for absolute probe locations for UE RF testing unnecessary for multi-Rx UE RF testing using the proposed test system and testing methodology. - Option 2: TBA - Agreements: - Option 1 is agreed. - FFS on whether the probe location should be on the measurement grid.

Unfortunately, this agreement and the corresponding analyses were never properly captured in [5] which until today states that absolute probe locations are required, specifically in Clause 5.3

The plots of the perceived DL directions from different, fixed probes demonstrate the need to define the absolute probe locations/directions for UE RF systems so that different test systems yield the same results.

The annex of this contribution has a draft of a CR with the proposed changes to update [5] with regards to absolute probe locations; this CR will be submitted to RAN4#113.

During RAN4#112, the need for absolute probe locations was clarified in online and offline discussions and one action was captured in the WF [6].

Issue 1-2-1: AoA separation and UE orientation for EIRP PUMAX,f,c,k testing - Agreement - Take below proposal as the starting point: - For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enabled (i.e., from Beam Peak Search for MOP in 6.2 of TS 38.101-2, the other AoA is at the direction decided by UE declared orientation (listed in TS38.101-2) and AoA pair selecting from set of {30deg, 60deg, 90deg, 120deg, 150deg}. - FFS whether a test direction other than the beam peak direction of single CC operation without STxMP enabled can be used for first AoA - Due to lack of absolute probe location definition of the multi-AoA UE RF test system, defined in Clause 5.2.2 of TR38.871, FFS on whether or how such test system can guarantee the same relative angular orientations between specific UE test directions and probes.

Given the rotational symmetry, two systems with different absolute probe locations should still be able to guarantee the same relative angular orientations between specific UE test directions and probes provided the baseline system with probes in the xz plane and angular sweeps of 0≤q<360° and 0≤f<180° are supported.

Proposal 1: Consider the baseline multi-RX/STxMP system suitable to guarantee the same relative angular orientations between specific UE test directions and probes.

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1: Consider the baseline multi-RX/STxMP system suitable to guarantee the same relative angular orientations between specific UE test directions and probes.

# References

- R4-2219851, Testing Considerations for Multi AoA Rx Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #105, November 2022

- R4-2220264, WF for FR2 OTA test SI, Qualcomm, 3GPP TSG-RAN WG4 Meeting #105, November 2022

- R4-2309469, On Absolute Probe Locations for Multi-Rx Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #107, May 2023

- TR 38.871, Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements

- R4-2309811, WF for FR2 OTA test SI, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting # 107, May 2023

- R4-2413534, WF on Rel-19 NR_FR2_OTA, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting # 112, August 2024

# Annex: Draft CR to 38.871 to update the need of absolute probe locations

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

## 5.1	General

This clause describes the UE RF testing methodology for multi-Rx chain DL reception for FR2. The following candidate measurement setups are investigated:

-	Measurement Setup with Full Degree of Rotation Freedom for Each AoA

-	Measurement Setup with Full Degrees of Freedom for AoA1 with Fixed Angular Offset(s) Between AoA1 and 		AoA2

-	Measurement Setup with Full Degrees of Freedom for AoA1 with Variable Angular Offset(s) between AoA1 and 		AoA2

-	Measurement Setup with Full Degrees of Freedom for AoA1 with Partial Freedom of Variable Angular Offset(s) 		between AoA1 and AoA2

-	Measurement Setup with Full Degrees of Freedom for AoA1 with Fixed AoA2/Anchor in NF

-	Measurement Setup with Test Modes

-	Measurement Setup to reduce sensitivity to UE orientation in holder with full degrees of freedom for AoA1 with 		fixed angular offset(s) between AoA1 and AoA2

Through the analysis of system complexity, chamber footprint, upgradeability of existing system, development lead time, measurement uncertainty, and test time aspects, the measurement setup with full degrees of freedom for AoA1 with fixed angular Offset(s) between AoA1 and AoA2 and with all probes located on the xz plane is selected as the baseline in Rel-18.

<<< Skip Unchanged Sections >>>

## 5.3	Test System Aspects

### 5.3.1	Absolute Probe Locations

A sample system with three different probe locations as shown in Figure 5.3.1-1 is investigated to provide detailed justification to define absolute probe locations/directions for Multi-AoA UE RF testing. The sample implementation has a 2-axis positioning system for the DUT to implement a 3D scan and three probes in the x, y, and z axes with the assumption that AoA1 is along z and AoA2 along x or y. The DL directions perceived by the DUT from each probe are illustrated with red, green, and red spheres on the DUT grid in the x, y, and z directions, respectively, for this initial test point of (q, f) = (0°, 0°).

Figure 5.3.1-1: Sample measurement setup with three fixed probe locations for initial test point of (0°, 0°)

For the sample multi-AoA system investigated, the angular difference between AoA1 (z direction) and AoA2 (x direction) and between AoA1 (z direction) and AoA2 (y direction) is 90° in both cases. One could seemingly argue that the roll motor of the DUT (rotation around the DUT z axis) makes it irrelevant whether AoA2 is along the y or the x axis. However, once the device is rotated in 3D, i.e., q and f, this argument is no longer applicable which is further analysed and illustrated below. For a second sample device orientation of (q, f) = (45°, -45°), the perceived DL directions are illustrated on the rotated DUT grid in Figure 5.3.1-2. Clearly, a 90° rotation of the device around its z axis would no longer map the green sphere onto the red sphere.

Figure 5.3.1-2: Sample measurement setup with three fixed probe locations for test point of (45°, -45°)

It is now assumed that the test points in 3D are based on a constant-step size grid with 266 unique grid points (Dq=Df=15°), i.e., as defined for PC1 & PC3 spherical coverage test cases for 1-DL in Clause 5.3 of [6], or on a constant density grid with 200 unique grid points, i.e., as defined for PC1 spherical coverage test cases for 1-DL in Clause 5.3 of [6]. These test/grid points are visualized in Table 5.3.1-1.

Table 5.3.1-1: Visualization of spherical coverage test/grid points

| Constant-step size grid with 266 unique grid points (Dq=Df=15°) | Constant density grid with 200 unique grid points |
| --- | --- |

When all spherical coverage grid points are sampled with 0°≤q≤180° and 0°≤f≤360°, the DL directions perceived by the UE from the various probes are plotted in Table 5.3.1-2. Clearly, these results show vastly different results.

-	For the probe aligned with the z axis, the perceived DL directions match the test points.

-	For the probe aligned with the y axis, the DUT only perceives the DL in a single plane only, i.e., the DUT’s xy plane. This is due to the turntable axis of the OTA positioner aligned with the y axis, i.e., the turntable rotation merely changes the polarization but not the DL direction with this probe. The roll motor, i.e., the rotation of the DUT around its z axis, only adjusts the DL direction within the xy plane.

-	For the AoA2 probe aligned with the x axis, the DL directions are perceived from different directions in (q, f) but limited to a single hemisphere only.

The plots of the perceived DL directions from different, fixed probes demonstrate the need to define the absolute probe locations/directions for UE RF systems so that different test systems yield the same results if the probes are placed arbitrarily and not within the xz plane.

Table 5.3.1-2: DL directions perceived by the DUT from probes in the x, y, and z axes.

| Probe Along Direction | DL Directions perceived by DUT with constant-step size grid with 266 unique grid points (Dq=Df=15°) | DL Directions perceived by DUT with constant density grid with 200 unique grid points |
| --- | --- | --- |
| z |  |  |
| y |  |  |
| x |  |  |

When probes are arbitrarily aligned in the xz plane while meeting the required relative angular offsets, absolute probe locations are no longer required for multi-Rx chain DL reception testing.

The effect of the 2 AoA DL polarizations perceived by the UE using orientation vectors will be demonstrated next; additional details regarding the orientation vectors can be found in Clause 5.3.5. The AoA1-AoA2 DL orientation vectors are the vectors between the DL orientations perceived by the UE from AoA1 to AoA2 and are visualized by plotting the AoA1 DL directions on a sphere with the black sphere while the direction towards the corresponding offset AoA2 DL direction is visualized with a single arrow (the length of the arrow corresponds to the value of the offset), e.g.,

Three different probe placement examples with probes on the xz plane are used next together with a 15° grid spacing. In the first example, a sample 4-probe system supporting {30°, 60°, 90°, 120°, and 150°} relative angular offsets is used as shown in Figure 5.3.1-3.

Figure 5.3.1-3: Example system implementation with P0 placed at q=0°

In the second example, as illustrated in Figure 5.3.1-4, the P0 probe is no longer aligned with the z axis but rotated by 30° towards the -x axis.

Figure 5.3.1-4: Example system implementation with P0 placed at q=-30°

In the third example, the AoA1&AoA2 probes are placed so that they are symmetric to the z axis as illustrated in Figure 5.3.1-5.

Figure 5.3.1-5: Example system implementation with probes symmetric to the z axis: left: P0 & P60, centre: P0 & P120, right: P0 & P150.

The AoA1-AoA2 DL orientation vectors for the two system/probe configurations are further illustrated in Table 5.3.1-3. Clearly, the vectors are all aligned, i.e., absolute probe locations are not necessary as long as the probes are placed on the xz plane.

Table 5.3.1-3: Vectors of DL orientations between AoA1 (P0) and AoA2 from three different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant-step size grid with Dq=Df=15°). Full rotation in q and half rotation in f.

|  | Probes in the xz plane with P0 at q=0° | Probes in the xz plane with P0 at q=-30° | Probes in the xz plane symmetric to z axis |
| --- | --- | --- | --- |
| System Configuration |  |  |  |
| Orientation between Probes P0 and P60 |  |  |  |
| Orientation between Probes P0 and P120 |  |  |  |
| Orientation between Probes P0 and P150 |  |  |  |

<<< END OF CHANGES >>>