3GPP TSG-RAN4 Meeting #112bis	R4-2416480

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.12.2.2

Source:	Keysight Technologies UK Ltd

Title:	On NTN OTA Testing

Document for:	Approval

# Introduction

This contribution addresses NTN OTA test metrics and on how to resolve the UE antenna polarization based on the Rel-19 OTA WI in [1]

# Discussion

A number of suitable performance metrics were proposed by various companies in RAN4#100-bis as captured in the WF [2]:

Issue 3-2-2: Proper performance metric for NR-NTN Way forward: Consider the following initial input as a starting point for further discussions: - Option 1: consider the following aspects - Option 1: integrated power/sensitivity within declared half sphere - Option 2: peak EIRP/EIS only - Option 3: peak EIRP/EIS + X%-tile spherical coverage within declared half sphere - Option 4: peak EIRP/EIS + Y%-tile spherical coverage from whole sphere - Option 2: consider the following aspects - Consider a general framework for each UE type, to specify performance metric for different use cases / power classes, e.g., a set of metric for Handheld, and other set for FR1 VSAT-like UE. - Consider the assumption: UE elevation angles for NTN are typically >20°, in majority of cases. - Consider TRP, TRS, EIRP, and EIS as starting point. Further discuss other performance metric based on NTN usage scenarios, e.g., directivity requirements, Antenna Gain. - Option 3: consider the following aspects - consider WRP and WRS defined in R4-2404278 as performance metrics for NTN devices - consider EIRP and EIS CDF percentile thresholds as performance metrics for NTN devices - Option 4: Conical TRP/EIRP for Tx and conical TRS/EIS for Rx could be considered as the starting point for the metric of NTN device OTA testing. FFS on the range of q and f. - Option 5: Consider the following aspects for NTN performance metric - Adopting a directional antenna pattern as the starting point for the antenna characteristics of an FR1 NTN device. - Take UIRP and UHIS as the as the starting point for the performance metric of devices using directional antenna. - Further discuss whether a smaller angle of test scan can be used or whether a EIRP/EIS CDF-like performance metric and be considered. - Option 6: other aspects are not precluded

In RAN4#111, additional discussions on the metrics were held with additional down selections made in the WF [3], i.e.,

Issue 3-2-1: Categorized performance metric for NR-NTN handheld Agreements - RAN4 should not adopt direction 1, other directions are FFS - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS - Direction 2: Single point + measured full sphere (integrated or CDF), e.g., - D2a, Peak EIRP/EIS+TRP/TRS, - D2b, Peak EIRP/EIS+ full Spherical coverage CDF - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles), - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting, - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere) - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

No progress was made in the last meeting. This contribution intends to analyse the select test metrics for simulated and synthesized antenna patterns. The test metric portion of this contribution is largely from [4].

# NTN Antenna Patterns

This contribution is investigating sample antenna patterns for smartphone UEs, i.e., no VSAT-like devices. The first set of patterns were simulated in CST Studio Suite. Here, a full smartphone model including most components was investigated first with the device held in the hand and is shown in Figure 1

Figure 1: CST Studio Suite smartphone model in hand right (HR) talk condition

Two different antenna locations for NTN antennas were investigated, i.e., one at the top (with frequency ~1.6 GHz, n255) and one on the side (with frequency range ~2.1 GHz, n256) as further illustrated in Figure 2.

Figure 2: NTN antennas investigated on top (left) and on side (right)

The simulated antenna patterns are shown in Figure 3.

Figure 3: NTN antennas patterns for antenna location on top (left) and on side (right)

From the patterns shown in Figure 3, it can be observed that the simulated antennas yield fairly omnidirectional patterns even in the HR talk condition. In the remainder of the contribution, the simulated antenna patterns had to be adjusted/rotated to comply with the conventional smartphone UE, e.g., Figure N.1-2 of [6].

Figure N.1-2: DUT default alignment of example smartphone UE to coordinate system

In order to analyse more directive patterns, two additional patterns with higher directivity were synthesized, generally using the sinc(theta) pattern shape. One has a 120° HPBW and the other a 60° HPBW, both with a 15 dB FTBR. All 4 analysed patterns are visualised in Figure 4.

Figure 4: Simulated and synthesized NTN antennas patterns. Top left: simulated smartphone antenna on top, top right: simulated smartphone antenna on right, bottom left: directive antenna with 120° HPBW, bottom right: directive antenna with 60° HPBW

# Analysis of Metrics

This section is investigating the following performance metrics discussed in the last couple meetings:

- Peak EIRP: Maximum total component of EIRP measured on the respective measurement grid

- TRP: Total radiated power, i.e., spatially weighted surface integral of the EIRPs measured on the respective measurement grid

- UHRP: Upper hemisphere radiated power, i.e., spatially weighted surface integral of the EIRPs measured on the respective measurement grid in the upper hemisphere (θ≤°90)

- WRP: weighted radiated power per [8][9], definition in Annex of [4]. Parameters used here: Wi=0.5 and threshold of 15dB.

- Weighted TRP: weighted TRP based on the following equation 2/3*UHRP + 1/3*LHRP

- CDF85%: 85th-percentile CDF of EIRPs measured on the respective measurement grid points with spatially normalized PDFs.

The following investigations take into account various device rotations and thus antenna pattern rotations around the y-axis of the smartphone. Select device rotations of the smartphone in the hand right talk condition are visualized in Figure 5.

Figure 5: Visualization of various device orientations/rotations around the y-axis.

It is clearly understood that for certification testing, just a single orientation is selected, i.e., any variability w.r.t. device orientation is irrelevant. OEMs commented in the RAN4#111 that “NTN communication is important for emergency and life critical scenarios” and specifically that there is a “software assistant function to guide user how to hold the UE” [10] or “for NTN devices it could be feasible to assume that a connection manager assists the user with positioning the device to obtain a higher quality signal” [12]. In other words, OEMs suggest that in emergency situations, the optimal device orientation could be suggested to the user using device prompts which supports the idea that any performance metric should be independent on device orientation as the optimal device orientation could be declared by the OEMs.

Observation 1: OEMs suggest that in emergency situations, the optimal device orientation could be suggested to the user using device prompts

On the other hand, satellite operators suggested that any device orientation could be possible [13], i.e.,

- For handheld/smartphone UE – UE may be upside-down when a call is initiated (e.g. in pocket): - If we assume only synchronization and paging typically occur when UE is upside down, we can still sacrifice some antenna performance on the bottom hemisphere, due to lower SNR requirements for the physical channels associated with these procedures. - When data transfer is occurring– e.g. PUSCH/PDSCH and the UE is pointing upwards, it is advantageous to have a higher antenna performance in the upper hemisphere to allow for a higher link margin. - This would also reduce the antenna noise temperature, meaning the system noise is less significant, which increases G/T, increasing the DL link margin, while increasing the UL margin due to higher gain towards the satellite orbital arc. - Assessing the directivity of the antenna can be done with a similar effort level to obtaining the TRP, TRS and EIS values, and would allow for more detailed link level analysis.

Observation 2: Satellite operators suggest that in emergency situations, the device orientation could be arbitrary

Given these contrary views, the intent of the following performance metric analyses is to determine the variability of these metrics as a function of various device/pattern rotations and two different measurement grids, i.e., a constant-step size grid with Dq=Df=5° (fine) and 15° (coarse, default). Ideally, the selected metric should show rather limited dependence on the device orientations for different NTN antenna patterns so that the metric demonstrates general applicability.

Observation 3: Ideally, the selected metric should show rather limited dependence on the device orientations

The results for the different metrics as a function of pattern rotation angles and measurement grids are tabulated in Table 1 through Table 6.

Table 1: Deviation of Peak EIRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | -0.02 | 0.00 | -0.07 | 0.00 | 0.00 | 0.00 | 0.00 |
| 15 | -0.01 | -0.02 | -0.01 | -0.10 | 0.00 | 0.00 | 0.00 | 0.00 |
| 30 | 0.00 | -0.01 | 0.00 | -0.07 | 0.00 | 0.00 | 0.00 | 0.00 |
| 45 | 0.00 | -0.03 | -0.01 | -0.06 | 0.00 | 0.00 | 0.00 | 0.00 |
| 90 | 0.00 | -0.02 | -0.01 | -0.04 | 0.00 | 0.00 | 0.00 | 0.00 |
| 135 | 0.00 | -0.07 | -0.01 | -0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
| 180 | 0.00 | -0.02 | 0.00 | -0.07 | 0.00 | 0.00 | 0.00 | 0.00 |

Table 2: Deviation of TRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.00 |
| 15 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 30 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 45 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 90 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 135 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 180 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.00 |

Table 3: Deviation of UHRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | 0.01 | 0.00 | -0.01 | 0.00 | -0.02 | 0.00 | 0.01 |
| 15 | -0.05 | -0.05 | -0.30 | -0.30 | -0.07 | -0.09 | 0.03 | 0.04 |
| 30 | -0.09 | -0.09 | -0.65 | -0.66 | -0.26 | -0.28 | 0.09 | 0.08 |
| 45 | -0.16 | -0.16 | -1.08 | -1.08 | -0.60 | -0.62 | 0.05 | 0.03 |
| 90 | -0.51 | -0.50 | -2.50 | -2.48 | -2.53 | -2.53 | -2.37 | -2.37 |
| 135 | -0.29 | -0.29 | -3.39 | -3.35 | -6.08 | -6.02 | -8.33 | -8.18 |
| 180 | 0.08 | 0.07 | -2.50 | -2.49 | -9.29 | -9.15 | -8.00 | -8.09 |

Table 4: Deviation of WRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | 0.00 | 0.00 | 0.00 | -0.03 | -0.17 | -0.04 | 0.05 |
| 15 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.04 | 0.00 | 0.01 |
| 30 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.03 | 0.01 | 0.01 |
| 45 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.04 | 0.00 | 0.03 |
| 90 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.05 | -0.01 | 0.01 |
| 135 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.04 | 0.00 | 0.03 |
| 180 | 0.00 | 0.00 | 0.00 | 0.00 | -0.03 | -0.17 | -0.04 | 0.05 |

Table 5: Deviation of Weighted TRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | -0.03 |
| 15 | -0.02 | -0.02 | -0.04 | -0.04 | 0.13 | 0.16 | -0.05 | -0.07 |
| 30 | -0.03 | -0.03 | -0.11 | -0.12 | 0.41 | 0.43 | -0.15 | -0.14 |
| 45 | -0.06 | -0.06 | -0.24 | -0.24 | 0.67 | 0.68 | -0.08 | -0.05 |
| 90 | -0.19 | -0.19 | -0.83 | -0.82 | 0.57 | 0.57 | 0.29 | 0.29 |
| 135 | -0.11 | -0.11 | -1.28 | -1.26 | -1.16 | -1.12 | -2.87 | -2.78 |
| 180 | 0.03 | 0.02 | -0.83 | -0.83 | -3.09 | -3.01 | -2.67 | -2.73 |

Table 6: Deviation of 85th-percentile CDF for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.01 | 0.03 | 0.06 | 0.11 | 0.06 | 0.07 | 0.31 | 0.41 |
| 15 | 0.00 | 0.07 | 0.06 | 0.18 | 0.06 | 0.06 | 0.35 | 0.32 |
| 30 | 0.00 | 0.01 | 0.06 | 0.28 | 0.07 | 0.08 | 0.41 | 0.41 |
| 45 | 0.00 | 0.06 | 0.05 | 0.19 | 0.08 | 0.08 | 0.44 | 0.43 |
| 90 | -0.01 | 0.04 | 0.02 | 0.13 | 0.09 | 0.06 | 0.48 | 0.35 |
| 135 | -0.01 | 0.03 | -0.02 | -0.04 | 0.08 | 0.08 | 0.44 | 0.43 |
| 180 | 0.00 | 0.01 | -0.04 | -0.09 | 0.05 | 0.06 | 0.29 | 0.36 |

From these results, the following observations can be made:

- The peak EIRP and TRP, as expected, have very little variability in terms of pattern rotations and measurement grids

- Similarly, the WRP metric shows very little variability regardless of orientations, grids, and patterns

- The UHRP and weighted TRP metric generally shows strong variability as a function of device orientation and pattern, especially for directive patterns

- The CDF metric shows relatively small variability for realistic (simulated) NTN antenna patterns with different device/pattern rotations

Observation 4: While the peak EIRP, TRP, and WRP metrics generally show no variability in terms of device orientations and NTN antenna patterns, the CDF metric shows generally small variability and the UHRP and weighted TRP metrics show strong variability.

For metrics not to be too dependent on device orientations, it is proposed to no longer focus on partial radiated powers and/or weighted TRPs.

Proposal 1: No longer consider partial radiated power and/or weighted TRPs but focus on TRP, WRP, or 85th %-ile CDF metrics with optional peak EIRP. Consider similar metrics for the RX measurements.

# Use of Circular Polarization

The RAN4#112 WF [5] included a discussion topic for test equipment and test system vendors related to polarization measurements, i.e.,

Issue 3-3-4: How to resolve UE antenna circular polarization issue Agreements: - FFS RAN4 how to accommodate different UE polarization implementations, i.e., either circular polarization or cross polarization.

The use of circular polarization in OTA test systems for NR NTN, specifically related to conformance testing, seems to be considered as an alternative to conventional linear polarization using in traditional OTA conformance test systems for SISO and MIMO. As illustrated in Figure 6, circular polarization (CP) requires the definition of the handedness, i.e., clock vs counter-clock wise or left-handed circular polarization (LHCP) vs right-handed circular polarization (RHCP).

Figure 6: Left and Right Handedness of Circular Polarization

Leaving aside the test system implementation to introduce circularly polarized DL signals for now, this section is focused on whether a circularly polarized antenna (RHCP or LHCP) used for both UL and DL can be used as measurement antenna. It should be noted that circular polarization in wireless telecommunications is very untypical on the UE side; the use of linear polarizations is much more predominant. While satellite vehicles could easily use circularly polarized antennas, the UEs will likely leverage highly integrated antennas similar to GNSS antennas that are commonly linearly polarized as the integration of CP antennas into handsets generally require more space and/or real estate not readily available for antennas.

Observation 5: The NTN antennas in handheld devices are most likely linearly polarized.

To assess the UE DL performance for circular polarization reception, the UE has to be exposed to the corresponding CP (LHCP, RHCP) polarization, i.e., the performance cannot be assessed with post-processing of two linear-polarization (LP) measurements, i.e., q and f. On the other hand, to assess the UE UL performance for CP transmission, the UE does not have to be exposed to the corresponding CP polarization as the performance could be assessed with post-processing of two LP measurements (q and f) provided that the phase difference between the LP measurement paths has been measured.

Observation 6: To assess the UE DL performance for reception of a circularly polarized wave, the UE has to be exposed to the corresponding CP (LHCP, RHCP) polarization

Observation 7: To assess the UE UL performance for circular polarization reception, the UE does not have to be exposed to the corresponding CP (LHCP, RHCP) polarization as the performance could be assessed with post-processing of two linear-polarization (q and f) measurements

Introducing circular polarization in the DL can be achieved by either

- Using circularly polarized antennas, such as (conical) spiral antennas

- Using dual linearly (orthogonal) polarized antennas with input signals that are in quadrature

Native, CP antennas generally support just a single polarization (LHCP or RHCP) which is problematic as all other NR FR1 OTA conformance test cases [14], [15] require the use of linear polarizations to be introduced in the DL. Such native, CP antennas cannot readily support switching between CP and LP polarizations, i.e., conformance test systems supporting NR FR1 OTA test cases would have to rely on mechanical switching between CP and LP measurement antennas which is possible but highly undesirable.

Observation 8: The use of antennas with native circular polarization is not suitable for FR2 OTA conformance test cases.

The use of dual linearly polarized antennas with orthogonal polarizations seems the more practical approach if CP had to be introduced for NR NTN conformance test cases. Suitable implementations would be dual polarized horn or Vivaldi antennas. Either would require the input signals to be controlled individually in amplitude (due to antenna gain differences in the two respective polarizations) and in phase, i.e., achieve the relative +90o or -90o phase differences.

Observation 9: The use of dual, linearly polarized with orthogonal polarizations, e.g., horn or Vivaldi antennas seems more suitable for conformance test cases that use CP

If we assume that the gain difference between two polarizations is insignificant, the DL signal could be split in front of the antenna inputs and a ±90o phase shifter needs to be inserted at one input to support both LHCP and RHCP. Adjustable 90o and -90o phase shifters are readily, commercially available. However, such phase shifters would introduce additional insertion loss and reduction of SNR. Additionally, the operation of CP and LP polarization in the same test system would require further complexities in the switching network and thus impact dynamic range even more.

Observation 10: The use of discrete 90o phase shifters/polarizers inserted in front of dual-linearly polarized antennas to generate CP is not recommended.

An alternate approach would be to individually adjust the magnitudes and phases of the DL signals connected to the dual-linearly polarized antenna at the output of the gNB emulator. In order to feed the antenna in quadrature, the phase differences between gNB emulator ports have to be controllable individually and accurately. However, such adjustability has never been made a requirement in 3GPP for UE RF testing and therefore not considered in gNB emulators architectures so far. Performing phase calibrations before and/or during each UE RF test is impractical as well.

Observation 11: The need for fixed phase differences between gNB emulator ports has not been made a requirement in 3GPP for UE RF testing and therefore has not yet been taken into account for gNB emulator architectures.

Observation 12: Performing phase calibrations before or during each UE RF test is impractical

If we assumed that the generation of signals in quadrature at the input of the antenna was feasible in principle, some related aspects need to be considered. The deviation from the ideal ±90o phase differences at the gNB emulator outputs need to be considered together with the axial ratio, i.e., deviation from the ideal ±90o phase differences, in the far field of the antenna (assuming the antenna was fed in perfect quadrature). Such axial ratio assessment might potentially also be necessary in off-axis directions. More importantly, the relative phase differences for two orthogonal polarizations inside the quiet zone need to be determined at various discrete locations caused by reflections/standing waves from support structures, positioners, absorbers, the deviation in AR of the feed antenna (especially off-axis directions), etc. Additionally, the relative amplitude differences for two orthogonal polarizations inside the quiet zone need to be determined as well. All of the above degradations and coupling between polarizations need to be considered with an additional MU element related to UEs receiving downlink signals with an axial ratio of less than 1 (an axial ratio of 1 which corresponds to circular polarization). These three phase deviation terms (output phase of the gNB emulator, antenna axial ratio variation, and the quality of the quiet zone) could change the intended circular polarization to elliptical polarization. The change from circular to elliptical and even to linear polarization due to changes in phase is illustrated in the Poincare sphere in Figure 7.

Observation 13: New and modified quality of quiet zone procedures are needed for relative phase and amplitude

Observation 14: Additional MU element(s) are needed to account for the relative phase variation (due to gNB emulator output phase variation, antenna axial ratio, quality of quiet zone).

Observation 15: An intended circularly polarized signal could become elliptically or even linearly polarized due to cumulative relative phase difference changes anywhere within the quiet zone.

Figure 7: Poincare Sphere and Changes in Polarization due to relative phase differences

Additionally, the intention of the OTA testing is to determine the overall/total performance of the UE which captured appropriately with TRP and TRS metrics for the transmitter and receiver. These metrics or partial metrics, as discussed in the earlier section, still seem to be appropriate based on the OEM, chipset vendor, operator, and TE/TS vendor feedback. The CP performance of the UE is specific to the antenna only which is typically not assessed exclusively in OTA conformance tests which focus on the entire TX/RX transceiver chains all the way to the baseband. As such, the assessment of the CP performance of the antennas should be left to in-house R&D tests rather than certification tests.

Observation 16: The assessment of the CP performance of the antennas should be left to in-house R&D tests rather than certification tests

Given the many observations and impact on test equipment and MUs, it is proposed not to consider the introduction of circular polarization for NR NTN measurements.

Proposal 2: The introduction of circular polarization for NR NTN measurements is not considered given the findings in this contribution, specifically Observation 5 through Observation 16.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: OEMs suggest that in emergency situations, the optimal device orientation could be suggested to the user using device prompts

Observation 2: Satellite operators suggest that in emergency situations, the device orientation could be arbitrary

Observation 3: Ideally, the selected metric should show rather limited dependence on the device orientations

Observation 4: While the peak EIRP, TRP, and WRP metrics generally show no variability in terms of device orientations and NTN antenna patterns, the CDF metric shows generally small variability and the UHRP and weighted TRP metrics show strong variability.

Observation 5: The NTN antennas in handheld devices are most likely linearly polarized.

Observation 6: To assess the UE DL performance for reception of a circularly polarized wave, the UE has to be exposed to the corresponding CP (LHCP, RHCP) polarization

Observation 7: To assess the UE UL performance for circular polarization reception, the UE does not have to be exposed to the corresponding CP (LHCP, RHCP) polarization as the performance could be assessed with post-processing of two linear-polarization (q and f) measurements

Observation 8: The use of antennas with native circular polarization is not suitable for FR2 OTA conformance test cases.

Observation 9: The use of dual, linearly polarized with orthogonal polarizations, e.g., horn or Vivaldi antennas seems more suitable for conformance test cases that use CP

Observation 10: The use of discrete 90o phase shifters/polarizers inserted in front of dual-linearly polarized antennas to generate CP is not recommended.

Observation 11: The need for fixed phase differences between gNB emulator ports has not been made a requirement in 3GPP for UE RF testing and therefore has not yet been taken into account for gNB emulator architectures.

Observation 12: Performing phase calibrations before or during each UE RF test is impractical

Observation 13: New and modified quality of quiet zone procedures are needed for relative phase and amplitude

Observation 14: Additional MU element(s) are needed to account for the relative phase variation (due to gNB emulator output phase variation, antenna axial ratio, quality of quiet zone).

Observation 15: An intended circularly polarized signal could become elliptically or even linearly polarized due to cumulative relative phase difference changes anywhere within the quiet zone.

Observation 16: The assessment of the CP performance of the antennas should be left to in-house R&D tests rather than certification tests

Proposal 1: No longer consider partial radiated power and/or weighted TRPs but focus on TRP, WRP, or 85th %-ile CDF metrics with optional peak EIRP. Consider similar metrics for the RX measurements.

Proposal 2: The introduction of circular polarization for NR NTN measurements is not considered given the findings in this contribution, specifically Observation 5 through Observation 16.

# References

- RP‑240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, RAN meeting #103, Mar 2024

- R4-2406086, Way Forward for [110bis][338] TRP_TRS_MIMO_OTA, vivo, CAICT, 3GPP TSG-RAN WG4 Meeting #110bis, April 2024

- R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA, vivo, 3GPP TSG-RAN WG4 Meeting #111, May 2024

- R4-2419771, NTN OTA Test Metrics, Keysight Technologies, 3GPP TSG-RAN4 Meeting #112, August 2024

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo, 3GPP TSG-RAN WG4 Meeting #112, August 2024

- TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

- TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

- R4-2402875, WF for [110][331] NR_FR1_TRP_TRS_enh, vivo, 3GPP TSG-RAN WG4 Meeting #110, March 2024

- R4-2404278, on test methodology and radiated performance metric for FR1 NTN devices, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting # 110bis, April 2024

- R4-2409771, NTN OTA Testability Aspects, Keysight Technologies, 3GPP TSG-RAN4 Meeting #111, May 2024

- R4-2407897, Discussion on NTN UE OTA test, Samsung, 3GPP TSG-RAN WG4 Meeting #111, May 2024

- R4-2407060, On NTN TRP TRS methodology, Apple, 3GPP RAN4 Meeting #111, May 2024

- RP-240615, Satellite Operator views on OTA requirements for NTN, Inmarsat, Viasat, RAN#103

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

- TS 38.161, User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios