3GPP TSG-RAN WG4 Meeting #112-bis									                    R4-2416483

Hefei, China, October 14 – 18, 2024

Agenda item:			6.8.2.2.1

Source:	Ericsson

Title:	On UE RF Tx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Tx requirements for NR-NTN HPUE from the approved WF in RAN4#112 [2].

# Discussion

## General considerations

Regarding the scope of this WI, and particularly the power class support, there was a following agreement in the previous meeting [2]:

Issue 3-1-3: Consideration of power classes

- Agreement

  - All power classes in WID, including Power class 1/1.5/2 for NR-NTN and Power class 1/2 for IoT-NTN, will be further discussed

According to the WID objectives [1], the main goal of this WI is to specify the power class 2 (PC2, 26dBm) that applies to both handheld and non-handheld UEs based on the co-existence study. Support of other power classes will be based on co-existence and feasibility studies, which in our understanding means that their support is not guaranteed as it will depend on the outcome of the two studies. Therefore, until the feasibility of other power classes is confirmed, we propose that RAN4 should focus on specifying power class 2.

Proposal 1: Until the feasibility of other power classes is confirmed based on co-existence and feasibility studies, we propose that RAN4 should focus on specifying power class 2.

On the HPUE architecture issue, the following was agreed [2]:

Issue 3-1-5: Consideration of HPUE Architecture

- Agreement:

  - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements.

  - For PC2 IoT NTN, at least 1Tx shall be considered

  - For other PCs (both IoT NTN and NR NTN), FFS

In one of the objectives for the coexistence analysis part, particularly PC1.5 and PC1 are mentioned for NR-NTN. In the UE RF requirements for TN [2], in the UE maximum output power sub-clause, PC1.5 is specified for TDD bands only, which is achieved using dual Tx only. If PC1.5 were going to be introduced for FR1-NTN bands n256, n255 and n254 (which are all FDD bands), its feasibility should be confirmed, including an investigation into why it is not specified for TN FDD bands. In our understanding, the current UEs in the field are not equipped with power amplifiers (PA) of 29dBm. Hence, if feasibility of PC1.5 is confirmed for this WI, only dual Tx architecture should be considered.

Proposal 2: Since PC1.5 is specified for TN TDD bands only, the feasibility for PC1.5 for NTN FDD bands should be confirmed first, including an investigation into why it is not specified for TN FDD bands. If its feasibility is confirmed for this WI, only dual Tx architecture should be considered.

## Feasibility

Regarding the NR-NTN HPUE feasibility for power classes other than PC2, the following was agreed [2]:

Issue 3-2-1: NR-NTN HPUE feasibility for different power classes

Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes

Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes

- Agreement of Issue 3-2-1 to Issue 3-2-3:

  - For PC2, it is feasible to support both handheld and non-handheld devices.

  - FFS other power classes for applicable device type.

When it comes to UE type, according to the WID objectives both handheld and non-handheld UEs are in the scope, where for other than PC2 high power classes “non-handheld UE is prioritized”. In TS38.101-1 [3] “Table 6.2-1: UE Power Class”, there is a note that “Generally, PC1 UE is not targeted for smartphone form factor.” As the starting point, the same assumption should hold for NR-NTN in FR1-NTN operating bands. Thus, unless NTN handheld UEs prove to be able to support PC1 and there is a specific need to introduce it, we propose to target PC1 for non-handheld UEs only.

Proposal 3: Similar to FR1 TN, for NR-NTN in FR1-NTN PC1 should be targeted for non-handheld UEs only.

## Detailed requirements

On the nominal MOP and the tolerance, the following was agreed as a starting point [2]:

Issue 3-3-1: Maximum output power

- Agreement

  - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN.

In our view, +2/-3 tolerance can be confirmed for PC2 for NTN HPUE, which is typically the case for TN PC2. Once the feasibility for PC1.5/PC1 is confirmed, the RAN4 can come back to this issue and check whether the same requirements can be applied.

Proposal 4: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN HPUE.

Regarding the SAR requirements issue, the following was agreed [2]:

Issue 3-3-4: SAR for handheld

Agreement:

- Using P-MPR as the starting point

- The other solutions are not precluded

  - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

In TS38.101-1 [3] clause “6.2.1 UE maximum output power”, Specific Absorption Rate restriction (SAR) requirements are specified for PC2 and PC1.5. The solution relying on the “UL duty cycle” is optional. For FDD bands it is not specified how exactly the UE implements such a solution, since it is only stated that “the percentage of uplink symbols transmitted in a certain evaluation period is larger than X% (The exact evaluation period is no less than one radio frame);” This period could be anything from one radio frame to many radio frames, and the network is not aware of how the UE makes that choice.

On another hand, as part of the discussion on the scope of this WI, there were some ideas shared that perhaps UEs do not have to comply with SAR requirements at least in some extreme use cases. For example, one could argue that in an emergency situation in a very remote area where a TN is not available, the ability to establish a reliable communication with a FR1-NTN high power class outweighs the risk to the health by exposing to more than recommended strength of the EMF. However, in our view, since SAR requirements are regulatory ones which always need to be respected, these requirements should be specified for handheld UEs and there should be no exceptions when it comes to their applicability.

Proposal 5: For the compliance with SAR requirements for handheld UEs, only P-MPR mechanism shall be used since the “UL duty cycle” based solutions have turned out to be inefficient for TN FDD bands.

On the SEM issue, there was a question whether to reuse the existing requirements from PC3:

Issue 3-3-5: SEM

- Agreement:

  - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

  - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

Proposal 6: Since for NR-TN the SEM requirements do not depend on the PC, the existing SEM requirements from PC3 can be applied to other PCs.

# Conclusion

Based on the discussion in the previous sections we have the following proposals:

Proposal 1: Until the feasibility of other power classes is confirmed based on co-existence and feasibility studies, we propose that RAN4 should focus on specifying power class 2.

Proposal 2: Since PC1.5 is specified for TN TDD bands only, the feasibility for PC1.5 for NTN FDD bands should be confirmed first, including an investigation into why it is not specified for TN FDD bands. If its feasibility is confirmed for this WI, only dual Tx architecture should be considered.

Proposal 3: Similar to FR1 TN, for NR-NTN in FR1-NTN PC1 should be targeted for non-handheld UEs only.

Proposal 4: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN HPUE.

Proposal 5: For the compliance with SAR requirements for handheld UEs, only P-MPR mechanism shall be used since the “UL duty cycle” based solutions have turned out to be inefficient for TN FDD bands.

Proposal 6: Since for NR-TN the SEM requirements do not depend on the PC, the existing SEM requirements from PC3 can be applied to other PCs.

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2414452, “WF on UE RF requirements for NTN HPUE”, Samsung, RAN4#112.

- 3GPP TS 38.101-1 V18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”