3GPP TSG-RAN WG4 Meeting #112-bis									                    R4-2416485

Hefei, China, October 14 – 18, 2024

Agenda item:			6.8.2.3.1

Source:	Ericsson

Title:	On UE RF Tx requirements for IoT-NTN HPUE

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for IoT-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Tx requirements for IoT-NTN HPUE from the approved WF in RAN4#112 [2].

# Discussion

## General considerations

Regarding the scope of this WI, and particularly the power class support, there was a following agreement in the previous meeting [2]:

Issue 3-1-3: Consideration of power classes

- Agreement

  - All power classes in WID, including Power class 1/1.5/2 for NR-NTN and Power class 1/2 for IoT-NTN, will be further discussed

According to the WID objectives [1], the main goal of this WI is to specify the power class 2 (PC2, 26dBm) that applies to both handheld and non-handheld UEs based on the co-existence study. Support of other power classes will be based on co-existence and feasibility studies, which in our understanding means that their support is not guaranteed as it will depend on the two studies. Therefore, until the feasibility of PC1 is confirmed, especially taking into account that it is not specified for TN IoT devices, we propose that RAN4 should focus on specifying power class 2.

Proposal 1: Until the feasibility of PC1 is confirmed based on co-existence and feasibility studies, we propose that RAN4 should focus on specifying power class 2.

On the HPUE architecture issue, the following was agreed [2]:

Issue 3-1-5: Consideration of HPUE Architecture

- Agreement:

  - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements.

  - For PC2 IoT NTN, at least 1Tx shall be considered

  - For other PCs (both IoT NTN and NR NTN), FFS

As stated for the previous issue, until the feasibility of other power classes (PC1) is confirmed, the RAN4 should focus on specifying power class 2. Both 1Tx and 2Tx architectures should be feasible.

Proposal 2: For PC2, both 1Tx and 2Tx architectures should be feasible.

## Feasibility

Regarding the IoT-NTN HPUE feasibility for power classes other than PC2, the following was agreed [2]:

Issue 3-2-1: NR-NTN HPUE feasibility for different power classes

Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes

Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes

- Agreement of Issue 3-2-1 to Issue 3-2-3:

  - For PC2, it is feasible to support both handheld and non-handheld devices.

  - FFS other power classes for applicable device type.

When it comes to UE type, according to the WID objectives both handheld and non-handheld UEs are in the scope, where for other than PC2 high power classes “non-handheld UE is prioritized”. Since category NB1, NB2 and M1 UEs for TN [3] currently do not support PC2 or higher power class, in our view it would make sense only to focus on PC2 in Rel-19.

On another hand, similar to NR, in TS36.101 there is a note that “Generally, PC1 UE is not targeted for smartphone form factor.” [3] Moreover, to our knowledge, category NB1 and NB2 UEs are typically stationary devices which are not handheld. Thus, if PC1 is being studied for IoT-NTN, PC1 should be targeted for non-handheld UEs only.

Proposal 3: Since category NB1, NB2 and M1 UEs for TN currently do not support PC2 or higher power class, for both NB-IoT and eMTC based IoT-NTN HPUE, only focus on PC2 in Rel-19.

## Detailed requirements

On the nominal MOP and the tolerance, the following was agreed as a starting point [2]:

Issue 3-3-1: Maximum output power

- Agreement

  - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN.

Since for band n14, which is the only FDD band in LTE for which PC2 is specified, the tolerance is ±2 (which is also the case for all TDD bands except for one), the same could be used as the starting point for IoT-NTN.

Proposal 4: Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance can be considered as starting point (same as for band n14) for PC2 for IoT-NTN HPUE.

Regarding the SAR requirements issue, the following was agreed [2]:

Issue 3-3-4: SAR for handheld

Agreement:

- Using P-MPR as the starting point

- The other solutions are not precluded

  - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

In LTE, a solution similar to TN duty cycle based do not apply for the only FDD band for which PC2 is specified (n14). In our understanding, SAR requirements only rely on P-MPR mechanism.

Proposal 5: For SAR requirements for handheld UEs, only P-MPR mechanism shall be used, as it is the case for band n14 in LTE TN.

On the SEM issue, there was a question whether to reuse the existing requirements from PC3:

Issue 3-3-5: SEM

- Agreement:

  - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

  - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

Proposal 6: Since for LTE the SEM requirements do not depend on the PC, the existing SEM requirements from PC3 can be applied to other PCs.

# Conclusion

Based on the discussion in the previous sections we have the following proposals:

Proposal 1: Until the feasibility of PC1 is confirmed based on co-existence and feasibility studies, we propose that RAN4 should focus on specifying power class 2.

Proposal 2: For PC2, both 1Tx and 2Tx architectures should be feasible.

Proposal 3: Since category NB1, NB2 and M1 UEs for TN currently do not support PC2 or higher power class, for both NB-IoT and eMTC based IoT-NTN HPUE, only focus on PC2 in Rel-19.

Proposal 4: Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance can be considered as starting point (same as for band n14) for PC2 for IoT-NTN HPUE.

Proposal 5: For SAR requirements for handheld UEs, only P-MPR mechanism shall be used, as it is the case for band n14 in LTE TN.

Proposal 6: Since for LTE the SEM requirements do not depend on the PC, the existing SEM requirements from PC3 can be applied to other PCs.

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2414452, “WF on UE RF requirements for NTN HPUE”, Samsung, RAN4#112.

- 3GPP TS 36.101 V18.7.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”