3GPP TSG-RAN WG4 Meeting #112-bis									                    R4-2416487

Hefei, China, October 14 – 18, 2024

Agenda item:			6.8.3.1

Source:	Ericsson

Title:	On system parameters for less than 5 MHz CBW for NTN

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In this contribution, we focus on the system parameters for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands and we provide our view for some of the remaining open issues from the approved WF [2].

# Discussion

In the approved WF [2], the two following issues are FFS:

Issue 1-1-6: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

- FFS

In TN, 3MHz channel bandwidth is optional for some (n26, n28, n85 and n100) and mandatory for other (n31, n72 and n106) bands, where all bands which support it are <1GHz FDD bands. n31, n72 and n106 are all narrow bands with only 5 MHz available bandwidth on DL/UL, which is not the case for FR1-NTN bands. Since the usual practice is to make the support for a new channel bandwidth optional in the Release where it is introduced unless there is a specific request from an operator, we propose to do the same for 3 MHz CBW for FR1-NTN bands:

Proposal 1: 3MHz channel bandwidth for NR-NTN in FR1-NTN bands should be optional in this Release unless there is a specific request from an operator for some band.

Issue 1-2-6: Signalling of SS raster

- FFS

For both TN and NTN in NR, synchronization raster is never being signaled, and the same shall apply for 3 MHz CBW in NTN.

Proposal 2: Synchronization raster shall not be signaled for 3MHz channel bandwidth for NR-NTN.

Our guess is that this issue actually refers to the question on whether UEs which support 3 MHz channel bandwidth for NR-NTN in FR1-NTN bands should use the Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 which were introduced for 3 MHz UE channel bandwidth support for TN. In general, we don’t see any issue if the same capabilities were used. However, their description in TS38.306 should be modified – other than TS38.101-1, TS38.101-5 should also be referenced, and in case of support3MHz-ChannelBW-Symmetric-r18 capability the correct table reference should be used for “Applicable SS raster entries per operating band for 3 MHz channel bandwidth” table.

Observation 1: UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 which were introduced in Rel-18 for TN could be reused for NR-NTN in FR1-NTN operating bands with certain small modifications.

# Conclusion

Based on the discussion in the previous sections we have made the following observation:

Observation 1: UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 which were introduced in Rel-18 for TN could be reused for NR-NTN in FR1-NTN operating bands with certain small modifications.

Based on the discussion in the previous sections we propose the following:

Proposal 1: 3MHz channel bandwidth for NR-NTN in FR1-NTN bands should be optional in this Release unless there is a specific request from an operator for some band.

Proposal 2: Synchronization raster shall not be signaled for 3MHz channel bandwidth for NR-NTN.

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2413528, “WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF”, Xiaomi, RAN4#112.