3GPP TSG-RAN WG4 Meeting #112-bis									                    R4-2416488

Hefei, China, October 14 – 18, 2024

Agenda item:			6.8.3.2

Source:	Ericsson

Title:	On UE RF requirements for less than 5 MHz for NTN

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In RAN4#112, the WF has been approved in [2]. In this contribution, we focus on the UE RF requirements for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands and we provide our view on the open issues.

# Discussion

## Tx requirements

Issue 2-1-1: UE maximum output power

- FFS on how to define the additional requirements for transmit power density for 3MHz channel bandwidth in n254

Since the ETSI requirements [3] for the EIRP density within the operation band apply for the whole range 1610 MHz – 1626.5 MHz and for any frequency sub-band, it means that it shall also apply for 3 MHz channel bandwidth (CBW). Therefore, we propose to add 3 MHz CBW to both NS_04N and NS_05N for the additional requirements for transmit power density, i.e. to modify Table 6.2.1-2 from TS38.101-5 [4] in the following way:

Table 6.2.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| n254 | NS_04N | 3, 5 | 1610 - 1618.25 | 27dBm/4kHz (mean) 15dBm/4kHz (peak limit) |
|  | NS_05N | 3, 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |

Proposal 1: Add 3 MHz channel bandwidth to both NS_04N and NS_05N for the additional requirements for transmit power density.

On the A-MPR requirements impact, the following was captured in the approved WF [2]:

Issue 2-1-3: A-MPR

- Further check

  - Neither 3MHz channel bandwidth or A-MPR are needed define in Table 6.2.3.1-1 for N_100

  - Need introduce 3MHz channel bandwidth in Table 6.2.3.1-1, but no A-MPR need define for below NS values:

      - NS_01, NS_02N

  - Need introduce 3MHz channel bandwidth in Table 6.2.3.1-1 and define A-MPR for below NS values:

      - NS_03N: A-MPR need define for 3MHz channel bandwidth in Table 6.2.3.2-1/Table 6.2.3.2-1

      - NS_04N: A-MPR need define for 3MHz channel bandwidth in Table 6.2.3.3-1/ Table 6.2.3.3-2

      - NS_05N: A-MPR need define for 3MHz channel bandwidth in Table 6.2.3.4-1/ Table 6.2.3.4-2

      - NS_24: A-MPR need define for 3MHz channel bandwidth for NTN n256 in a new clause 6.2.3.X in NTN spec 38.101-5.

- Companies are encouraged to provide the related A-MPR values

A-MPR requirements associated with NS_100 have the purpose of UTRA protection, to meet the UTRA ACLR requirement. These requirements apply to all channel bandwidths and therefore 3 MHz CBW does not need to figure in Table 6.2.3.1-1 for N_100.

Proposal 2: 3 MHz channel bandwidth shall not figure in Table 6.2.3.1-1 for N_100.

Both NS_01 and NS_02N network signalling values are not related to A-MPR requirements, the later one being defined for the additional spurious emission requirements for band n255. Thus, we agree with the proposed WF:

Proposal 3: For NS_01 and NS_02N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, but no A-MPR is needed to be defined.

NS_03N is associated with the additional spurious emission requirements for band n254 to comply with FCC regulations, where the A-MPR requirements are specified per channel bandwidth. Similarly, NS_04N and NS_05N are associated with the additional spurious emission requirements for band n254 to comply with ETSI regulations [3]. For these network signalling values, we agree with the proposed WF:

Proposal 4: For NS_03N, NS_04N and NS_05N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, and A-MPR requirements for 3 MHz channel bandwidth need to be defined for each NS value in the corresponding clause.

NS_24 is associated with the additional spurious emission requirements for band n256 (also specified for band n65 for TN when the upper edge of the channel bandwidth frequency is greater than 1980 MHz). For TN, 3 MHz CBW has been added only for <1GHz operating bands and thus A-MPR requirements for band n65 for 3 MHz CBW are lacking. So, for NS_24, we agree with the proposed WF [2]:

Proposal 5: For NS_24, specify A-MPR requirements for 3 MHz channel bandwidth for band n256 in a new clause 6.2.3.X in NTN spec 38.101-5. Also, for NS_24, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1.

After an additional checking, for the following group of issues we propose to confirm the proposed WF from the previous meeting [2]:

Issue 2-1-4: Minimum output power/ Transmit OFF power

- Further check

  - Whether reuse the 3MHz channel bandwidth requirements of NR TN FR1 UE

    - Minimum output power: -40dBm with the measurement bandwidth 2.715 MHz

    - Transmit OFF power: -50dBm with the measurement bandwidth 2.715 MHz

Issue 2-1-5: Occupied bandwidth

- Further check

  - Addition of 3 MHz into Table 6.5.1-1 in TS 38.101-5

Issue 2-1-6: Spectrum emissions mask

- Further check

  - Whether reuse the 3MHz channel bandwidth requirements of NR TN FR1 UE

Issue 2-1-8: NR ACLR

- Further check

  - Addition of 3 MHz into Table 6.5.2.4.1-1 in TS 38.101-5

Issue 2-1-9: General spurious emission

- Further check

  - Whether reuse the TN UE approach for 3 MHz channel bandwidth correspondingly FOOB (6 MHz)

Proposal 6: For the following Tx requirements, confirm the proposed WF from the previous meeting:

- Minimum output power: reuse 3MHz CBW requirements of NR TN FR1 UE, i.e. -40dBm with the measurement bandwidth 2.715 MHz (add it to Table 6.3.1-1 in TS38.101-5);

- Transmit OFF power: reuse 3MHz CBW requirements of NR TN FR1 UE, i.e. -50dBm with the measurement bandwidth 2.715 MHz (add it to Table 6.3.2-1);

- Occupied bandwidth: add 3 MHz CBW into Table 6.5.1-1;

- Spectrum emission mask: reuse the 3MHz CBW requirements of NR TN FR1 UE, i.e. 3 MHz shall be added to Table 6.5.2.2-1 and the same values as for TN should be specified;

- NR ACLR: add 3 MHz into Table 6.5.2.4.1-1 in TS 38.101-5;

- General spurious emissions: reuse the TN UE approach for 3 MHz channel bandwidth, i.e. define OOB boundary FOOB to be 6 MHz.

Additional spurious emissions requirements are FFS in the approved WF [2]:

Issue 2-1-10: Additional spurious emissions

- FFS

As discussed above, each of NS_02N, NS_03N, NS_04N and NS_05N is associated with the additional spurious emissions requirements (originating from ETSI or FCC regulatory requirements), which are common for all channel bandwidths since those regulatory requirements are defined for frequency ranges. 3 MHz CBW can also fit into the same ranges, and thus we propose:

Proposal 7: For the additional spurious emissions requirements, add 3 MHz CBW into:

- Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

- Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

- Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N".

For the non-impacted Tx requirements, the following was captured [2]:

Issue 2-1-12: Other Tx requirements

- Further check

  - Whether there are no specification impact for below Tx requirements.

6.2.4 Configured transmitted power

6.3.3 Transmit ON/OFF time mask

6.3.4 Power control

6.4.1 Frequency error

6.4.2 Transmit modulation quality

6.5.2.4.2 UTRA ACLR

6.5.4 Transmit intermodulation

After a further check, we would like to confirm the proposal from the WF:

Proposal 8: There is no specification impact in TS38.101-5 for the following Tx requirements: Configured transmitted power, Transmit ON/OFF time mask, Power Control, Frequency error, Transmit modulation quality, UTRA ACLR and Transmit intermodulation.

## Rx requirements

Regarding the REFSENS requirements, the following was captured in the approved WF [2]:

Issue 2-2-1: Reference sensitivity

- FFS

For the reference sensitivity requirements (REFSENS), 3 MHz CBW shall be added to Table 7.3.2-1, where the exact requirement should be evaluated. As the starting point, 2.2dB improvement compared with 5 MHz CBW could be considered due to 10log(BW) factor in the REFSENS equation difference between 3 MHz and 5 MHz, which was also adopted for most of TN bands. Regarding the uplink configuration for REFSENS, 3 MHz should be added to Table 7.3.2-2, where 15RBs could be considered as the starting point.

Proposal 9: For the REFSENS requirements, 3 MHz CBW shall be added to Table 7.3.2-1:

- Use 2.2dB improvement compared with 5 MHz CBW as the starting point;

- For the uplink configuration for REFSENS, 3 MHz should be added to Table 7.3.2-2, where 15RBs could be considered as the starting point;

- TX-RX carrier center frequency separation for REFSENS verification should be further studied.

Regarding the other Rx requirements, the following was captured in the approved WF [2]:

Issue 2-2-2: Maximum input level

- FFS

Issue 2-2-3: ACS

- FFS

Issue 2-2-4: In-band blocking

- FFS

Issue 2-2-5: Out-of-band blocking

- Further check

  - Define out of band blocking requirement similar to TN for 3MHz  (REFSENS + 6.0 dB)

Issue 2-2-6: Narrow band blocking

- Further check

  - Define narrow band blocking requirement similar to TN for 3MHz  (-18dB Pw)

Issue 2-2-7: Spurious response

- Further check

  - 3 MHz shall be added in table 7.7-1, same column as for 5 and 10 MHz channel bandwidth (REFSENS + 6.0 dB)

Issue 2-2-8: Intermodulation characteristics

- Further check

  - There is no specification impact.

Issue 2-2-9: Spurious emissions

- Further check

  - There is no specification impact.

After a further check, for the following Rx requirements we would like to propose to re-use the existing requirements for 3 MHz CBW for TN wherever applicable:

Proposal 10: For the following Rx requirements, re-use the existing requirements for 3 MHz CBW for TN wherever applicable:

- Maximum input level: 3 MHz shall be added in table 7.4-1 into the same column as for 5, 10, 15, 20 MHz channel BW;

- ACS: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset;

- In-band blocking: 3 MHz shall be added to table 7.6.2-1 with a 3 MHz interferer and with the same interference offsets as for TN (4.5 MHz and 7.5 MHz);

- Out of band blocking: 3 MHz shall be added to table 7.6.3-1 where the same value as for 5 and 10 MHz channel BW should be adopted: REFSENS + 6dB;

- Narrow band blocking: 3 MHz shall be added to table 7.6.4-1 with 18dB Pw;

- Spurious response: 3 MHz shall be added to table 7.7-1 into the same column as for 5 and 10 MHz channel BW: REFSENS + 6 dB;

- Intermodulation characteristics: no specification impact;

- Spurious emissions: no specification impact.

# Conclusion

Based on the discussion in the previous sections we have the following proposals:

Proposal 1: Add 3 MHz channel bandwidth to both NS_04N and NS_05N for the additional requirements for transmit power density.

Proposal 2: 3 MHz channel bandwidth shall not figure in Table 6.2.3.1-1 for N_100.

Proposal 3: For NS_01 and NS_02N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, but no A-MPR is needed to be defined.

Proposal 4: For NS_03N, NS_04N and NS_05N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, and A-MPR requirements for 3 MHz channel bandwidth need to be defined for each NS value in the corresponding clause.

Proposal 5: For NS_24, specify A-MPR requirements for 3 MHz channel bandwidth for band n256 in a new clause 6.2.3.X in NTN spec 38.101-5. Also, for NS_24, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1.

Proposal 6: For the following Tx requirements, confirm the proposed WF from the previous meeting:

- Minimum output power: reuse 3MHz CBW requirements of NR TN FR1 UE, i.e. -40dBm with the measurement bandwidth 2.715 MHz (add it to Table 6.3.1-1 in TS38.101-5);

- Transmit OFF power: reuse 3MHz CBW requirements of NR TN FR1 UE, i.e. -50dBm with the measurement bandwidth 2.715 MHz (add it to Table 6.3.2-1);

- Occupied bandwidth: add 3 MHz CBW into Table 6.5.1-1;

- Spectrum emission mask: reuse the 3MHz CBW requirements of NR TN FR1 UE, i.e. 3 MHz shall be added to Table 6.5.2.2-1 and the same values as for TN should be specified;

- NR ACLR: add 3 MHz into Table 6.5.2.4.1-1 in TS 38.101-5;

- General spurious emissions: reuse the TN UE approach for 3 MHz channel bandwidth, i.e. define OOB boundary FOOB to be 6 MHz.

Proposal 7: For the additional spurious emissions requirements, add 3 MHz CBW into:

- Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

- Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

- Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N".

Proposal 8: There is no specification impact in TS38.101-5 for the following Tx requirements: Configured transmitted power, Transmit ON/OFF time mask, Power Control, Frequency error, Transmit modulation quality, UTRA ACLR and Transmit intermodulation.

Proposal 9: For the REFSENS requirements, 3 MHz CBW shall be added to Table 7.3.2-1:

- Use 2.2dB improvement compared with 5 MHz CBW as the starting point;

- For the uplink configuration for REFSENS, 3 MHz should be added to Table 7.3.2-2, where 15RBs could be considered as the starting point;

- TX-RX carrier center frequency separation for REFSENS verification should be further studied.

Proposal 10: For the following Rx requirements, re-use the existing requirements for 3 MHz CBW for TN wherever applicable:

- Maximum input level: 3 MHz shall be added in table 7.4-1 into the same column as for 5, 10, 15, 20 MHz channel BW;

- ACS: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset;

- In-band blocking: 3 MHz shall be added to table 7.6.2-1 with a 3 MHz interferer and with the same interference offsets as for TN (4.5 MHz and 7.5 MHz);

- Out of band blocking: 3 MHz shall be added to table 7.6.3-1 where the same value as for 5 and 10 MHz channel BW should be adopted: REFSENS + 6dB;

- Narrow band blocking: 3 MHz shall be added to table 7.6.4-1 with 18dB Pw;

- Spurious response: 3 MHz shall be added to table 7.7-1 into the same column as for 5 and 10 MHz channel BW: REFSENS + 6 dB;

- Intermodulation characteristics: no specification impact;

- Spurious emissions: no specification impact.

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2413528, “WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF”, Xiaomi, RAN4#112.

- ETSI EN 301 441, "Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES), including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) operating in the 1,6 GHz/2,4 GHz frequency band under the Mobile Satellite Service (MSS) covering the essential requirements of article 3.2 of the Directive 2014/53/EU", v2.1.1, June 2016

- 3GPP TS 38.101-5 V18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”