3GPP TSG-RAN WG4 Meeting #112-bis									                    R4-2416490

Hefei, China, October 14 – 18, 2024

Agenda item:			6.19.2

Source:	Ericsson

Title:	On UE RF requirements for NR MIMO Phase 5

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for the RAN1-led WI on “NR MIMO Phase 5” WI [1]. In RAN4#112, a WF has been approved [2], where the objectives of the WI with the identified potential RAN4 UE RF impacts are the following:

RAN1:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Acronyms:

- sTRP: single TRP (transmit-receive point)

- mTRP: multiple TRP (transmit-receive point)

- CJT: coherent joint transmission

- DU: distributed unit

- CRI: CSI-RS resource indicator

RAN2:

Specify higher layer support of the enhancements listed above.

RAN4:

Specify necessary core requirements for the enhancements listed above.

In this contribution, we further discuss the impact of “Support for 3-antenna-port codebook-based transmissions” and “Asymmetric DL sTRP/UL mTRP” working areas on the UE RF requirements in RAN4.

# Discussion

In the approved WF [2], it was agreed that “RAN4 will analyse the impact of 3Tx on UE RF requirements”, while regarding the UE RF impact of asymmetric DL sTRP/UL mTRP, “companies are encouraged to provide clear view based on WID and RAN1 status”.

## 3Tx

In the following, the open issues from the approved WF [2] will be discussed.

### Applicable UE types

On applicable UE types issue, the following has been agreed in the previous meeting [2]:

Issue 2-1: Applicable UE types

- Proposals

  - Option 1: Handheld UE included

  - Option 2: Non-handheld UE only

  - Option 3: Depend on condition (e.g., ULFPTx)

- Agreement

  - Both handheld and non-handheld UEs will be considered for UE RF requirements

  - FFS on whether a single set or two sets of UE RF requirements, e.g., MPR, will be specified.

Two sets of UE RF requirements for handheld and non-handheld UEs should not be precluded. In the current specifications, the MPR requirements for PC1.5 with dual Tx are differentiated for the FWA UEs with large form factor with 20 dB or above antenna isolation and the other UEs (presumably including the handheld UEs). Similarly, for PC1.5 with 4 Tx, the MPR requirements are even differentiated between the non-handheld UEs: one set of requirements is targeted for “vehicular UE or other industrial device form factor with 10dB antenna isolation” and the other is targeted for “large FWA form factor with 20 dB or above antenna isolation”.

Proposal 1: Two sets of UE RF requirements for handheld and non-handheld UEs should not be precluded, as e.g. there are two sets of the MPR requirements for dual Tx and 4 Tx for PC1.5, depending on the achievable antenna isolation.

### Power classes and PA configuration for 3Tx UL MIMO

On “power classes and PA configuration for 3Tx UL MIMO” the following options were agreed in the previous meeting [2]:

Issue 2-3: Power classes and PA configuration for 3Tx UL MIMO

- Proposals

  - Option 1: PC3 and PC2

  - Option 2: PC1.5 only

  - Option 3: Others

- Way forward

  - Companies are encouraged to provide clear view on 3Tx power class and its PA assumptions

We don’t identify any issue in PC3 and PC2 support for 3Tx UL MIMO. Moreover, regarding the support of full-power Mode 0, in [3] there was a RAN1 agreement:

Agreement For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability.

Hence, at least the full-power Mode 0 could be supported for PC3 and PC2.

When it comes to PC1.5, since according to the WID [1] the UL full power transmission modes 1 and 2 are not supported, the only remaining candidate mode is the “Mode-full power” (ULFPTx mode 0). For PC1.5, the support for that mode implies that the UE would have to be equipped with a PA of 29dBm in every Tx branch. However, according to our understanding, there are currently no UEs in the field which are equipped with a PA of 29dBm. PC1.5 could potentially only be specified for a “non-full power” mode, i.e. the Rel-15 behavior, so the full support for PC1.5 should be further studied.

Proposal 2: Support at least PC3 and PC2 for 3Tx UL MIMO, since for PC1.5 to support “Mode-full power” (ULFPTx mode 0) the UE would have to be equipped with a PA of 29dBm in every Tx branch. According to our understanding, there are currently no UEs in the field which are equipped with a PA of 29dBm.

### ULFPTx mode 0

For “ULFPTx mode 0” issue, the following two options were agreed in the previous meeting [2]:

Issue 2-6: ULFPTx mode 0

- Proposals

  - Option 1: Add both 1-layer and 2-layer for 3Tx with

  - Option 2: Add 1-layer for 3Tx with W=13100,13010,13001

- Tentative agreement

  - Take 1-layer as baseline

  - Further discuss whether 2-layer is needed

Currently, the full-power Mode 0 is only specified for 2 Tx ports and 1 layer, and as stated in clause 2.1.2, in our understanding it implies that the UE would have to be equipped with a full-power PA in each Tx branch. To us, it is not clear what is the exact definition of full-power Mode 0 for 3 Tx ports and 2 layers. In general, Mode 0 implies that every precoder in the codebook delivers full power. Unless clearly defined, only 1-layer for 3 Tx ports shall be added for full-power Mode 0.

Observation 1: The full-power Mode 0 with 2 layers is not currently specified and it is not clear what it would imply regarding the PA configuration.

In [3], there were agreements on the supported precoders for single-layer and two-layer transmissions:

Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission. 13100,13010,13001

Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission. 13100100, 13100001, 13 001001

For the support of ULFPTx mode 0 with two-layer transmission and 3 Tx ports, RAN4 should discuss whether it is sufficient that the UE can achieve the full power with each of the following precoders:

13100100, 13100001, 13 001001.

For PC3 and PC2 we are fine with the tentative agreement from the WF, to take 1-layer as the baseline and to further discuss 2-layers support. For 1-layer, both DFT-s-OFDM and CP-OFDM waveforms should be supported. Hence, we make the following proposals:

Proposal 3: For ULFPTx mode 0, support 1-layer with 3 Tx ports case for both DFT-s-OFDM and CP-OFDM waveforms, i.e. add 1-layer for 3Tx with W=13100,13010,13001 and add both DFT-s-OFDM and CP-OFDM.

Proposal 4: For ULFPTx mode 0 support, 2-layer transmission with 3 Tx ports should be further discussed, e.g. what it would imply regarding the PA architecture. RAN4 should discuss whether it is sufficient that the UE can achieve the full power with each of the following precoders:

13100100, 13100001, 13 001001.

## Asymmetric DL sTRP/UL mTRP

In the approved WF [2], the only captured issue is the following one:

Issue 3-1: FR2 UL enhancement scenario

- FFS the following proposal (companies to check):

  - Option 1: UL TCI-state differs from DL TCI-state

  - Other options not precluded

According to the WID [1], “since the macro gNB and micro nodes differ in power rating, a UE may receive DL transmission from the macro gNB, but transmit UL to either the macro gNB or non-co-located micro nodes in order to maximize UL throughput.” Since the UL TRP node can be non-co-located to the DL TRP node, the UL-TCI state in that case is expected to be different to DL TCI-state, and thus we support the Option 1:

Proposal 5: For asymmetric DL sTRP/UL mTRP scenario, in general the UL TCI-state is different to the DL TCI-state when the UL TRP node is non-co-located to the DL TRP node.

# Conclusion

Based on the discussion in the previous sections we propose the following observation:

Observation 1: The full-power Mode 0 with 2 layers is not currently specified and it is not clear what it would imply regarding the PA configuration.

We also make the following proposals:

Proposal 1: Two sets of UE RF requirements for handheld and non-handheld UEs should not be precluded, as e.g. there are two sets of the MPR requirements for dual Tx and 4 Tx for PC1.5, depending on the achievable antenna isolation.

Proposal 2: Support at least PC3 and PC2 for 3Tx UL MIMO, since for PC1.5 to support “Mode-full power” (ULFPTx mode 0) the UE would have to be equipped with a PA of 29dBm in every Tx branch. According to our understanding, there are currently no UEs in the field which are equipped with a PA of 29dBm.

Proposal 3: For ULFPTx mode 0, support 1-layer with 3 Tx ports case for both DFT-s-OFDM and CP-OFDM waveforms, i.e. add 1-layer for 3Tx with W=13100,13010,13001 and add both DFT-s-OFDM and CP-OFDM.

Proposal 4: For ULFPTx mode 0 support, 2-layer transmission with 3 Tx ports should be further discussed, e.g. what it would imply regarding the PA architecture. RAN4 should discuss whether it is sufficient that the UE can achieve the full power with each of the following precoders:

13100100, 13100001, 13 001001.

Proposal 5: For asymmetric DL sTRP/UL mTRP scenario, in general the UL TCI-state is different to the DL TCI-state when the UL TRP node is non-co-located to the DL TRP node.

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105.

- R4-2414310, “WF on UE RF requirements for Rel-19 MIMO enhancement”, Samsung, RAN4#112.

- R1-2401937, “Final Report of 3GPP TSG RAN WG1 #116 v1.0.0”, MCC Support