3GPP TSG-RAN WG4 Meeting #112-bis                                       R4-2416491

# Hefei, China, 14th – 18th of October 2024

Title: 	Discussion on the Choice of Numerology for Ku-band

Agenda Item:	6.9.3

Source: 	THALES

Document for:	Discussion

# For information

As described in the approved Way Forward for [112][314] NR_NTN_Ku_Band_General (R4-2413519, Eutelsat Group), the following agreements have been made at the previous RAN4#112 meeting:

#### Issue 3-1: Frequency range definition

Following the scope of the WID, definition of a new frequency range e.g. FR3, is not in scope.

#### Issue 3-2: Numerology - Decision criteria

The following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology

- UL timing synchronization

- Phase noise

- Efficient channel utilization

- Beam hopping granularity

- Link budget UL/DL – as a function of SCS and channel bandwidth

#### Issue 3-3: Numerology – timing aspects

The impact of SCS on timing is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

#### Issue 3-4: Numerology – phase noise

The impact of SCS on susceptibility to phase noise is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

#### Issue 3-5: Numerology – efficient channel utilization

The ability to efficiently support in future the required aggregate bandwidths [list] of NTN services for different SCS choices based on existing FR1 and FR2 channel bandwidths needs to be fully studied.

#### Issue 3-6: Numerology– beam hopping granularity

An analysis of the impact of different SCS on performance, although not critical, can influence the decision on FR1 vs FR2 numerology.

# Proposals

For the time being, the following NTN Ku-band has been agreed in WF R4-2413519: 10.7 - 12.75 GHz (DL); 14.0 - 14.5 GHz (UL).

Moreover, the following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology (WF R4-2413519):

- UL timing synchronization: Impact of SCS on timing is critical to reliable performance

- Phase noise: Impact of SCS on susceptibility to phase noise is critical to reliable performance.

- Efficient channel utilization;

- Beam hopping granularity;

- Link budget UL/DL – as a function of SCS and channel bandwidth.

Observation 1. Decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis.

Some other aspects have to be considered, including satellite complexity (e.g. 800-1000 cells/satellite) and several operational constraints:

Observation 2: Satellite access networks operating in above 10 GHz are most likely to operate simultaneously in several NTN bands (i.e. Ku and Ka-band).

The Ka-band implementation already presents an obvious amount of complexity, with 800-1000 cells/satellite (see e.g. RAN1 working assumptions and R1-2401988 3GPP TSG RAN WG1 Meeting #116bis, THALES):

| LEO600km Set1-1 FR2 (i.e., Ka-band) |  |
| --- | --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Observation 3: The support of both FR1 and FR2 numerologies on board of the satellites operating above 10 GHz will lead to extra additional complexity.

Moreover, is detrimental to fragment the market of VSAT-like terminals operating in above 10 GHz, with the risk of having different numerology especially when the trend/future is to develop terminals able to operate in both bands (i.e. Ku and Ka-band).

Observation 4: Is detrimental to fragment the market of VSAT-like terminals operating in above 10 GHz.

Observation 5: Most chipset vendors do not support 60 kHz SCS configuration.

Proposal 1: A common numerology (i.e. common sub-carrier spacing) to be used in both Ka and Ku-bands should be defined.

Proposal 2: The common numerology used in both Ku and Ka-band should allow a nominal maximum Channel BW up to 400 MHz in DL and UL.

Observation 6: Advantages and disadvantages related to the potential common SCS:

- Option 1: 120 kHz SCS

      - Ku-band: Introduce 120 kHz SCS for both synchronization and UL/DL data transmission

      - Ka-band: No action

- Option 2: 60 kHz SCS

      - Ku & Ka-band: Introduce 60 kHz SCS for both synchronization and UL/DL data transmission

- Option 3: 30 kHz SCS

      - Ku & Ka-band: Introduce 30 kHz SCS for both synchronization and UL/DL data transmission

Proposal 3: RAN4 to select among the following options:

- Option 1: 120 kHz SCS

      - Ku-band: Introduce 120 kHz SCS for both synchronization and UL/DL data transmission

      - Ka-band: No action

- Option 2: 60 kHz SCS

      - Ku & Ka-band: Introduce 60 kHz SCS for both synchronization and UL/DL data transmission

- Option 3: 30 kHz SCS

      - Ku & Ka-band: Introduce 30 kHz SCS for both synchronization and UL/DL data transmission

Proposal 4: RAN4 shall adopt at least 120 kHz SCS for Ku-band for both synchronization and UL/DL data transmission.

Proposal 5: RAN4 to inform/discuss with RAN1 through an LS the choice of FR1/FR2 numerologies for Ku-band.

# Conclusion

Observation 1. Decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis.

Observation 2: Satellite access networks operating in above 10 GHz are most likely to operate simultaneously in several NTN bands (i.e. Ku and Ka-band).

Observation 3: The support of both FR1 and FR2 numerologies on board of the satellites operating above 10 GHz will lead to extra additional complexity.

Observation 4: Is detrimental to fragment the market of VSAT-like terminals operating in above 10 GHz.

Observation 5: Most chipset vendors do not support 60 kHz SCS configuration.

Proposal 1: A common numerology (i.e. common sub-carrier spacing) to be used in both Ka and Ku-bands should be defined.

Proposal 2: The common numerology used in both Ku and Ka-band should allow a nominal maximum Channel BW up to 400 MHz in DL and UL.

Observation 6: Advantages and disadvantages related to the potential common SCS:

- Option 1: 120 kHz SCS

      - Ku-band: Introduce 120 kHz SCS for both synchronization and UL/DL data transmission

      - Ka-band: no action

- Option 2: 60 kHz SCS

      - Ku & Ka-band: Introduce 60 kHz SCS for both synchronization and UL/DL data transmission

- Option 3: 30 kHz SCS

      - Ku & Ka-band: Introduce 30 kHz SCS for both synchronization and UL/DL data transmission

Proposal 3: RAN4 to select among the following options:

- Option 1: 120 kHz SCS

      - Ku-band: Introduce 120 kHz SCS for both synchronization and UL/DL data transmission

      - Ka-band: no action

- Option 2: 60 kHz SCS

      - Ku & Ka-band: Introduce 60 kHz SCS for both synchronization and UL/DL data transmission

- Option 3: 30 kHz SCS

      - Ku & Ka-band: Introduce 30 kHz SCS for both synchronization and UL/DL data transmission

Proposal 4: RAN4 shall adopt at least 120 kHz SCS for Ku-band for both synchronization and UL/DL data transmission.

Proposal 5: RAN4 to inform/discuss with RAN1 through an LS the choice of FR1/FR2 numerologies for Ku-band.

# Annex

Moreover, the following information with respect to current specifications should be taken into account (with the current definition FR1-NTN: 410 MHz – 7125 MHz, FR2-NTN: 17300 MHz – 30000 MHz).

Table 5.3.5-1: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 (NOTE) |
| n256 | 15 | 5 | 10 | 15 | 20 |  |
|  | 30 |  | 10 | 15 | 20 |  |
|  | 60 |  | 10 | 15 | 20 |  |
| n255 | 15 | 5 | 10 | 15 | 20 |  |
|  | 30 |  | 10 | 15 | 20 |  |
|  | 60 |  | 10 | 15 | 20 |  |
| n254 | 15 | 5 | 10 | 15 |  |  |
|  | 30 |  | 10 | 15 |  |  |
|  | 60 |  | 10 | 15 |  |  |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |

Table 5.3.5-2: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

And the NTN timing error requirements represented for both FR1-NTN and FR2-NTN:

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | Applicability |
| --- | --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc | Timing error limit applicable for FR1-like NTN configurations. |
|  |  | 30 | 24*64*Tc |  |
|  |  | 60 | N/A |  |
|  | 30 | 15 | 24*64*Tc |  |
|  |  | 30 | 22*64*Tc |  |
|  |  | 60 | N/A |  |
| FR2-NTN | 120 | 60 | 13*64*Tc | Timing error limit applicable for FR2-like NTN configurations with fixed VSAT served by GSO, and fixed VSAT served by NGSO. |
|  |  | 120 | 7.5*64*Tc |  |
|  | 240 | 60 | 13*64*Tc |  |
|  |  | 120 | 7.5*64*Tc |  |
| FR2-NTN | 120 | 60 | 13*64*Tc | Timing error limit applicable for FR2-like NTN configurations with mobile VSAT served by GSO. |
|  |  | 120 | 10*64*Tc |  |
|  | 240 | 60 | 13*64*Tc |  |
|  |  | 120 | 10*64*Tc |  |
| NOTE: Tc is the basic timing unit defined in TS 38.211 |  |  |  |  |