3GPP TSG-RAN WG4 Meeting #112                                                                R4-2416494

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Further study on testability and interoperability issues for AI-CSI prediction

Agenda item:			6.17.3.1

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two use cases (i.e., beam management and positioning accuracy enhancements) [2].

Furthermore, the study objectives with corresponding checkpoints in RAN#105 (Sep ‘24) particularly on the two sub-use cases of CSI feedback enhancements are provided [2]. From RAN4 perspective, it is tasked to further analyze the various testing options for two-sided models, i.e., particularly related to the use case of CSI compression among all (sub-)use cases discussed in Rel-18 study item.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

Based on the outcome of the study objectives captured in TR 38.843 [1], RAN4 continued the study in RAN4#110, RAN4#110bis and RAN4#111 with agreements and way forwards achieved in [4][6][8] respectively. In RAN#105, the WID has been revised by taking the AI-CSI prediction as part of normative work, which will be started from Q1 of 2025.

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

Until the start of normative work, the RAN4 shall still focus on testability and interoperability issues for AI-CSI prediction use case. Accordingly, in this contribution, we would like to continue the discussion on these aspects.

# 2. Discussion on CSI prediction

## 2.1 Testability on CSI prediction accuracy

In RAN4#110, the agreement was achieved on CSI prediction accuracy metrics, as follows [4]:

Issue 4-1: CSI Prediction Accuracy metrics - Proposals - Option 1: Prediction accuracy can be used as KPI/metric - Option 2: Prediction accuracy cannot be used because the “correct” value is not available - Option 3: Throughput should be the default metric, others should be discussed only if throughput is not feasible - Option 4: Others Agreement: - Agree option 3 for inference only. TBD whether we use relative or absolute throughput. - Monitoring will be discussed separately.

And in the follow-up discussion in RAN4#112, the following agreement is achieved:

Issue 4-5: CSI prediction accuracy metric Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to

Even by adopting the relative throughput as test metric for inference, the definition of the baseline denominator of relative throughput still needs clarification. In the existing PMI test case, in order to test UE reported PMI is accurate enough, RAN4 choose the random PMI as denominator, which is equivalent to the case of “no UE reporting”. To test the predicted PMI is accurate enough, we need to study what should be the baseline denominator:

Observation 1: For relative throughput as requirement metric for CSI prediction CSI prediction accuracy metric for inference, the definition of baseline throughput (as denominator):

- Option 1: Throughput achieved by following random PMI (with the same codebook used for the reported predicted PMI)

- Option 2: Throughput achieved by following UE’s last reported PMI given by UE measurement

For the above Option 1 by following random PMI for baseline throughput, the final relative throughput performance is determined by both measurement and prediction, since both could contribute to the inaccuracy of predicted PMI.

Option 1 is given by the fact that PMI prediction is performed by UE for the time slot without explicit CSI-RS configured, while UE shall still use the measured PMI for the time slot if it contains the CSI-RS configured for PMI reporting. Hence, we can let TE to follow UE’s last reported PMI which is given by UE measurement rather than prediction. By adopting Option 1-2 as baseline throughput, we could rule out the impact from measurement inaccuracy but make the test focus on the UE prediction accuracy.

Proposal 1: For relative throughput used as CSI prediction accuracy metric for inference, Option 2 (Throughput achieved by following UE’s last reported PMI given by UE measurement) is adopted for baseline throughput.

# 3. Conclusion

In this contribution, we provided our viewpoints on the on the interoperability and testability aspects for AI-CSI prediction use case, accordingly the following observations and proposals are obtained:

Observation 1: For relative throughput as requirement metric for CSI prediction CSI prediction accuracy metric for inference, the definition of baseline throughput (as denominator):

- Option 1: Throughput achieved by following random PMI (with the same codebook used for the reported predicted PMI)

- Option 2: Throughput achieved by following UE’s last reported PMI given by UE measurement

Proposal 1: For relative throughput used as CSI prediction accuracy metric for inference, Option 2 (Throughput achieved by following UE’s last reported PMI given by UE measurement) is adopted for baseline throughput.

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-242399 (revised from RP-234039, RP-240774), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402390, “Discussion on testability and interoperability issues for AI-CSI”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] R4-2405653, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#110bis, April 2024.

[6] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[7] R4-2409762, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#111, May 2024.

[8] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[9] https://en.wikipedia.org/wiki/Multiply%E2%80%93accumulate_operatio

[10] R4-2411636, “Further study on testability and interoperability issues for AI-CSI”, RAN#112, Aug. 2024.

[11] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.