3GPP TSG-RAN WG4 Meeting #112                                                                R4-2416495

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Further study on testability and interoperability issues for AI-CSI compression

Agenda item:			6.17.2.1

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two use cases (i.e., beam management and positioning accuracy enhancements) [2].

Furthermore, the study objectives with corresponding checkpoints in RAN#105 (Sep ‘24) particularly on the two sub-use cases of CSI feedback enhancements are provided [2]. From RAN4 perspective, it is tasked to further analyze the various testing options for two-sided models, i.e., particularly related to the use case of CSI compression among all (sub-)use cases discussed in Rel-18 study item.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

Following the findings detailed in TR 38.843 [1], RAN4 has progressed its research through RAN4#110 and RAN4#110bis, with consensus and future directions to further the 2-sided model feasibility by executing the standardization procedure as documented in way forwards [4] [6]. As further discussed in RAN4#112, RAN4 agreed the way forward [8] on how to proceed the evaluation on test decoder Option 3 (test decoder specified in RAN4 spec.), with the detailed agreements captured as:

Issue 4-1: Option 3 next steps Agreement: - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused Issue 4-2: Simulation results and alignment Agreement: - perform a round of alignment until RAN4#112-Bis - refined simulation parameters can be found in R4-2414447 Issue 4-4: Options for Option 4 Agreement: Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

As per the discussions in RAN#105, the exploration of testability and interoperability challenges related to AI-driven CSI compression is set to continue within the remaining timeframe of Release 19. In this contribution, we aim to further the discussion on the issues of testability and interoperability for the AI-CSI compression scenario.

# 2. Discussion

## 2.1 Standardization for Test decoder Option 3

In the context of Release-18 research, the objective for test decoder Option 3 is to establish a unified decoder specification for at least one test applicable to all DUTs. It's important to recognize that RAN1's Option 1 for facilitating inter-vendor training collaboration, which entails a fully standardized reference model encompassing structure and parameters, aligns with RAN4's test decoder Option 3 in terms of the standardization effort to define a reference model, or what RAN4 refers to as a test decoder. However, the intended applications of these standards diverge, as RAN4's Option 3 is designed for test decoders within the test environment, whereas RAN1's Option 1 pertains to reference decoders within the gNB.

As per the discussions in RAN4#110bis, RAN4 is examining the feasibility of this approach in terms of standardization processes. In our contribution to RAN4#110bis [5], a flowchart has been proposed that reflects a common understanding of the standardization procedure within 3GPP RAN4. During RAN4#111 and #112, RAN4 engaged in extensive discussions on how to deliver AI-CSI compression outcomes and how to establish the scenario parameters necessary for dataset generation.

#### 2.1.1 Purpose of performance alignment for CNN-based evaluation

Based on RAN4#111, the following test decoder derivation procedure Option 1 is agreed, in which companies are required to “bring encoder + decoder set based on agreed parameters” firstly.

Agreement from RAN4#111 [8]: The following test decoder derivation procedure options were discussed: - Option 1 companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria) - Option 2 companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria) RAN4 agrees to work on option 1. Interested companies are invited to bring further feasibility analysis for Option 2

In the post-meeting email discussion after RAN4#111, the baseline system-level parameters (captured in TR38.858 and originally for Rel-18 RAN1 study) are reused to generate the dataset for observed channel model (without labels), and during RAN4#112, further refined simulation parameters are concluded and can be found in [R4-2414447].

It should be noted that the model structure type, i.e., CNN, and the detailed model parameters for both encoders and decoders are “randomly” picked for verify the feasibility of Option 3 standardization in RAN4. In other words, RAN4 pretended to have completed the step 1-5 in Samsung proposed flow-chart [5], and study the feasibility of performance alignment based on specified CNN-structure and the dataset obtained by SLS under the specified scenario. For the on-going evaluation (which is based on selected CNN-structure and trained over the company SLS-generated dataset under the specified scenario), it should be considered as a feasibility study on standardization procedure, i.e., for Step-6 and 7 in our proposed flow-chart [5].

After clarifying the-above RAN4 conclusion, it is clarified that the following issues can be studied by conducting this CNN-based performance alignment (which is based on selected CNN-structure and trained over the company-wise SLS-generated dataset):

Proposal 1: For this round of performance alignment, at least the following issues shall be studied and concluded:

- Issue-1: Whether the performance alignment can be achieved, with the aligned CNN-structure and training procedure (both of which are aligned as much as possible), by leaving the following difference:

  - the company-wise SLS-generated dataset, with particular SLS implementation, randomness and the size of dataset.

- Issue-2: How performance results can be regarded as being “aligned”?

The above issue-1 is well recognized as the major purpose of this alignment, by minimizing the factors which could result in unaligned results as much as possible. For issue-2, although it is not a brand-new practice for performance alignment (Note: in demodulation requirement standardization, performance alignment on LLS results is frequently conducted), it is still new to practice performance alignment for model training, which still needs more study. Based on our practice, we suggest the following Proposal 2 to be adopted.

Proposal 2: On the issue of “How performance results can be regarded as being ‘aligned’”, the following steps are required to be studied:

- Alignment Step-a: On the evaluation dataset provided by individual company, what is the SGCS performance spread among companies?

If the alignment is not confirmed in Alignment Step-a, consider Alignment Step-b:

- Alignment Step-b: On a common evaluation dataset (agreed to be used by the group), what is the SGCS performance spread among companies?

#### 2.1.2 CNN-based evaluation for CSI compression

In the post-meeting email discussion after RAN4#111, the detailed CNN model parameters for both encoders and decoders are provided as follows. It should be noted that the CNN-based structures for encoder/decoder are given without strong technical justification and not yet favored by most companies, since the purpose of this evaluation is only for the feasibility study on standardization procedure.

Fig 1. Detailed model diagrams of encoder (left) and decoder (right).

Table 1. CNN model parameters for encoder.

| Model architecture parameters for encoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 47M (vs. 1.6M for eTypeII) |
|  | Model size | 448Kbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer after resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer before resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,32) |
|  | Size of message (feature map) passed across layers | (13,32) |

Table 2. CNN model parameters for decoder.

| Model architecture parameters for decoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 740M |
|  | Model size | 7.2Mbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer before resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer after resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,128) |
|  | Size of message (feature map) passed across layers | (13,32) |

Table 3. Other model related parameters:

| Activation function | ReLU |
| --- | --- |
| Latent message size | 32 float values |
| Payload size, bits | 64 per layer. |
| Quantization | 2 bits |

When we build up the CNN-based encoder and decoder, we examined the value of FLOPs given in Table 1 and 2, and found that both values of “FLOPs” given are actually for the number of Multiply-Accumulate (MAC) operations, which is defined as a common step that computes the product of two numbers and adds that product to an accumulator [9]. Strictly speaking, it is not the FLOPs (Floating Point Operations), which is used more commonly for neural network computation complexity, and usually 1 MACs can be interpreted as 2 FLOPs.

Proposal 3: The metric of FLOPs shall be adopted used to evaluate neural network complexity, and original CNN complexity values (47M and 740M for encoder/decoder respectively) are actually for MACs, which shall be corrected by FLOPs (9.39*107 and 1.48*109 for encoder/decoder respectively).

Based on the agreed system-level simulation parameters in the approved WF [8] and further aligned parameters in [R4-2414447], we have performed the system-level simulation to obtain the dataset, which is randomly split for training and validation. Based on the obtained dataset, the CNN-based model is trained, with the following parameters to generate dataset taken into account:

Table 4. Dataset information

| UE antenna element assumptions | Omni-directional antenna elements |
| --- | --- |
| Data collection assumptions | 10 Drops One drop 60K UE |
| Training dataset size | 600K samples |
| For every sample | 32 TX ports * 4 RX ports * (13 subbands * 4 RBs/subband) |
| Dataset quantization | Float32 |
| Scenario | Dense Urban (Macro only) |
| BS Tx power | 44dBm for 20MHz |
| MIMO scheme | SU-MIMO |
| Channel estimation | ideal channel estimation |

#### 2.1.3 Performance analysis for CNN-based evaluation

We firstly provide the evaluation results and complexity:

Observation 1: SGCS performance and complexity for CNN-based model trained and evaluated over Samsung-SLS generated dataset is provided as:

| Encoder | Decoder | SGCS |
| --- | --- | --- |
| Back-bone: CNN Number of model parameter: 125K FLOPS: 9.39*107 Depth: 6 Res blocks Feature maps: 32 | Back-bone: CNN Number of model parameter: 1.8M FLOPS: 1.48*109 Depth: 6 Res blocks Feature maps: 128 | 0.63 |

As practiced in last meeting’s contribution, we also perform the training and evaluation by using the external dataset publicly available:

- CAICT shared dataset (Link: 移动通信开放数据平台 (mobileai-dataset.com), which is the updated link containing the same dataset used in our last contribution)

- OPPO shared dataset (Link: www.wireless-intelligence.com, the dataset of WLI_1_1024_UMa_52RB_3GPP_R18AI)

This generalization procedure can be viewed as a procedure to practice Alignment Step-b mentioned in Proposal 2. Here are some general comparisons between three datasets:

Table 5. Comparison of datasets from three sources

|  | Samsung dataset | CAICT dataset | OPPO dataset |
| --- | --- | --- | --- |
| # of total samples | 25K | 100K | 600K |
| Simulation parameters | Carrier Frequency: 2GHz # of drops: 10 # of UEs per drop: 60K # of slots per UE: 1 | Carrier Frequency: 2.1GHz # of drops: 100 # of UEs per drop: 1000 # of slots per UE: 1 | Carrier Frequency: 4GHz # of drops: 1 # of UEs per drop: 3000 # of slots per UE: 200 slot interval: 100 |

Here, we present an evaluation of the models' ability of generalization, assessing their performance when trained on various datasets.

Table 6. Generalization performance for models trained in difference datasets

| Encoder | Decoder | Testing data Training data | Samsung dataset | CAICT dataset | OPPO dataset |  |
| --- | --- | --- | --- | --- | --- | --- |
| Back-bone: CNN Number of model parameter: 125K FLOPS: 9.39*107 Depth: 6 Res blocks Feature maps: 32 | Back-bone: CNN Number of model parameter: 1.8M FLOPS: 1.48*109 Depth: 6 Res blocks Feature maps: 128 | Samsung dataset | 0.670 | 0.730 | 0.731 |  |
|  |  | CAICT dataset | 0.646 | 0.760 | 0.718 |  |
|  |  | OPPO dataset | 0.655 | 0.713 | 0.736 |  |

Drawing from the data presented in Table 6, we can deduce the following insights regarding model generalization:

Observation 2: An increased number of UEs per drop leads to a greater dispersion in channel characteristics, which in turn can adversely affect the SGCS performance (Note: this is observed across all models evaluated on the Samsung dataset.)

Observation 3: The SGCS performance varies significantly across different SLS simulated datasets, which utilize varying numbers of UEs per drop.

Based on the above observations, we see the importance to further discuss the common dataset to be used for model training and evaluation.

Observation 4: The importance of common dataset for training and/or evaluation has been demonstrated.

## 2.2 Discussion on Test decoder Option 4

For test decoder Option 4, the following sub-options are agreed in RAN4#112:

-	4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder)

-	4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

FFS whether training dataset (channel information) needs to be specified

-	4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

Among the three sub-options presented, we initially observe a paradox with Option 4b. This option has been considered due to the challenges in developing a test decoder as per RAN4 requirements, yet it necessitates the process of documenting a reference encoder within the specifications. From this standpoint, the feasibility of Option 4b depends upon the confirmed feasibility of Option 3.

Observation 5: The feasibility of Option 4b depends upon the confirmed feasibility of Option 3.

With or without model architecture parameters (e.g., model structure) specified in RAN4 standard, to facilitate other vendors (DUT vendors and infra vendors) to be able to develop such a decoder that can deliver similar performance, the standardized data set for training shall be helpful. In other words, if the training data set (including enough amount of data for raw CSI and compressed bit strings) is available in 3GPP standard, Option 4a can be regarded as the standardized training data set, compared with Option 3 with standardized test decoder itself.

Observation 6: For Option 4a, if the training data set (including enough amount of data for raw CSI and compressed bit strings) is specified in 3GPP standard, Option 4a can be regarded as the standardized training dataset.

Accordingly, we would like to provide the following proposal to clarify option 4:

Proposal 4: Among three sub-options,  Option 4a is preferred by assuming:

- TE vendor will not share decoder directly to other vendors (DUT and/or infra vendors);

- Parameters that need to be specified for defining test decoder shall include:

Training data set for TE decoder training, including enough amount of data for raw CSI and compressed bit string.

# 3. Conclusion

In this contribution, we provided our viewpoints on the on the interoperability and testability aspects for AI-CSI compression use case, accordingly the following observations and proposals are obtained:

Standardization for Test decoder Option 3

Proposal 1: For this round of performance alignment, at least the following issues shall be studied and concluded:

- Issue-1: Whether the performance alignment can be achieved, with the aligned CNN-structure and training procedure (both of which are aligned as much as possible), by leaving the following difference:

  - the company-wise SLS-generated dataset, with particular SLS implementation, randomness and the size of dataset.

- Issue-2: How performance results can be regarded as being “aligned”?

Proposal 2: On the issue of “How performance results can be regarded as being ‘aligned’”, the following steps are required to be studied:

- Alignment Step-a: On the evaluation dataset provided by individual company, what is the SGCS performance spread among companies?

If the alignment is not confirmed in Alignment Step-a, consider Alignment Step-b:

- Alignment Step-b: On a common evaluation dataset (agreed to be used by the group), what is the SGCS performance spread among companies?

Proposal 3: The metric of FLOPs shall be adopted used to evaluate neural network complexity, and original CNN complexity values (47M and 740M for encoder/decoder respectively) are actually for MACs, which shall be corrected by FLOPs (9.39*107 and 1.48*109 for encoder/decoder respectively).

Observation 1: SGCS performance and complexity for CNN-based model trained and evaluated over Samsung-SLS generated dataset is provided as:

| Encoder | Decoder | SGCS |
| --- | --- | --- |
| Back-bone: CNN Number of model parameter: 125K FLOPS: 9.39*107 Depth: 6 Res blocks Feature maps: 32 | Back-bone: CNN Number of model parameter: 1.8M FLOPS: 1.48*109 Depth: 6 Res blocks Feature maps: 128 | 0.63 |

Observation 2: An increased number of UEs per drop leads to a greater dispersion in channel characteristics, which in turn can adversely affect the SGCS performance (Note: this is observed across all models evaluated on the Samsung dataset.)

Observation 3: The SGCS performance varies significantly across different SLS simulated datasets, which utilize varying numbers of UEs per drop.

Observation 4: The importance of common dataset for training and/or evaluation has been demonstrated.

Discussion on Test decoder Option 4

Observation 5: The feasibility of Option 4b depends upon the confirmed feasibility of Option 3.

Observation 6: For Option 4a, if the training data set (including enough amount of data for raw CSI and compressed bit strings) is specified in 3GPP standard, Option 4a can be regarded as the standardized training dataset.

Proposal 4: Among three sub-options,  Option 4a is preferred by assuming:

- TE vendor will not share decoder directly to other vendors (DUT and/or infra vendors);

- Parameters that need to be specified for defining test decoder shall include:

Training data set for TE decoder training, including enough amount of data for raw CSI and compressed bit string.

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402390, “Discussion on testability and interoperability issues for AI-CSI”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] R4-2405653, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#110bis, April 2024.

[6] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[7] R4-2409762, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#111, May 2024.

[8] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[9] https://en.wikipedia.org/wiki/Multiply%E2%80%93accumulate_operatio

[10] R4-2411636, “Further study on testability and interoperability issues for AI-CSI”, RAN#112, Aug. 2024.

[11] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.