3GPP TSG-RAN WG4 Meeting #112bis                                                          R4-2416500

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:	On the modification of existing RX requirements for SBFD-capable BS

Agenda item:			6.20.2.3

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. And in the RAN4#111 and #112, further agreements are achieved on the modification of existing requirements for SBFD-capable BS [6] [8]. In this contribution, accordingly, we would like to provide our analysis and viewpoints on the modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS.

# 2. Modification on Existing RX requirement for SBFD

### 2.1  OTA sensitivity

As the outcome from RAN4 study item, regarding reference sensitivity requirement for SBFD-capable BS, due to the self-interference caused internally to receiver side, RAN4 reached the following consensus [2]:

- For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no sensitivity degradation is allowed. - Otherwise, OTA sensitivity requirement could be derived based on the following equation: PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]-G - The candidate value [0.5~1.0]dB degradation and final value will be specified in the WI phase. - The following aspects need more discussion during a WI phase - The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot - If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference.

Based on the agreement achieved in RAN4#112 [8], OTA sensitivity is agreed to be measured with only self-interference:

Agreement: - OTA sensitivity is measured with only self-interference. FFS on the OTA sensitivity degradation within [0.5~1.0] dB.

From Rel-18 study, gNB residual self-interference is widely assumed to be corresponding to 1dB desense in the system-level simulation. The following assumption is captured in annex B.1 for gNB residual self-interference. Accordingly, for the OTA sensitivity degradation due to self-interference, we propose that [1.0] dB degradation is adopted.

Table 1: SLS assumptions for interference modelling (copied from Table B.1-6)

| Parameters | FR1 | FR2-1 |
| --- | --- | --- |
| gNB self-interference - αSI | Candidate values for αSI at least can be determined based on the assumption that UL receiver sensitivity degradation due to self-interference is 1dB. Companies shall report what values of the individual components are assumed in order to achieve the αSI value corresponding to 1 dB desense. Other approaches of determining values for αSI are not precluded and can be used and reported by companies. |  |

Proposal 1: [1.0] dB degradation due to self-interference is assumed for OTA sensitivity requirement.

### 2.2  Dynamic range

Dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received BS channel bandwidth. In this condition a throughput requirement shall be met for a specified reference measurement channel. For the requirement of dynamic range, the following agreements is achieved in the study phase [2]:

Regarding the dynamic range requirement, this requirement is still applicable for SBFD-capable BS. The IoT level and wanted signal power level could be further discussed in the WI phase.

In RAN4#112, the following WF is achieved for dynamic range:

WF: - FFS on the followings: - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work. - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations. - Option 3: for receiver dynamic requirement, both uplink signals and BS2BS CLI signal should be considered for IoT levels.

As proposed by other companies, both uplink signal and gNB-to-gNB CLI signal should be considered in the IoT level. The IoT level in the existing requirement is derived by assuming that: While measuring the receiver dynamic range requirement, uncertainty due to the receiver’s own thermal noise floor should be minimized. So the interference level should be increased by a certain amount of margin to mask the receiver’s own noise floor. From a scenario of view, it is reasonable to adopt the similar amount of interference signal margin for NR as for E-UTRA. The mean power of interfering signal is defined in the following method:

PIntf = -174dBm/Hz+10*log10(NRB*SCS*12) + NF + 20

Based on our understanding, since the interference is already defined by 20dB over thermal noise, we see no strong reason to further increase the level of IoT to consider the impact of gNB-to-gNB CLI to be introduced in RAN1.

Proposal 2: In the existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations, by considering gNB-to-gNB CLI handling mechanism to be introduced in RAN1.

### 2.3  In-band selectivity and blocking

For the requirement of in-band selectivity and blocking, the following agreements is achieved in the study phase [2]:

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement: - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level. - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - For in-band selectivity and blocking, the requirements shall be defined out of the BS channel bandwidth instead of uplink subband bandwidth.

As provided as the way forward in RAN4#111, companies are encouraged to provide the evaluation results for in-band blocking in this meeting. By following Scenario 1, 3, 5, 6, and 9, and Case 3 (TDD DL to SBFD UL subband) defined in TR 38.858, it is intended to evaluate the power level from the co-existence study to derive in-band blocking levels. Furthermore, in RAN4#112, the following way forward is approved [8]:

## 1.7. ACS requirement WF: - FFS on the followings: - Option 1: If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed. - Option 2: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement. ## 1.8. In-band blocking Agreement: - On contributing source for in-band blocking requirement, it is only the DL adjacent channel interference. - RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements. - For co-existence study, - The reference point for the power level which should be before array gain WF: - FFS on the followings: - RAN4 needs to discuss on how in-band blocking requirement are derived: - Option 1: MCL assumption for BS2BS CLI interference - Option 2: Co-existence study - Option 3: Both - For co-existence study, RAN4 need firstly agree on the open parameters for simulation assumption: - FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - 99% of the UL SBFD wideband received power CDF

Firstly, we further conduct the co-existence simulation to study the impact of grid-shift on the probability distributions of adjacent channel gNB DL power level, Scenario 1 FR1 UMa-UMa.

| Grid-shift (%) | 10 | 20 | 50 | 100 |
| --- | --- | --- | --- | --- |
| Adjacent channel gNB DL power level @99 percentile (dBm) | -4.39 | -11.01 | -21.48 | -27.76 |

Another observation is the percentage value of grid shifting plays an important role to determine the distributions of adjacent-channel and co-channel gNB DL received powers, since the increased percentage grid shifting leads to higher pathloss between the interfering gNB and victim gNB directly. By adopting different percentages of grid shifting to derive the in-band blocking levels, the resultant requirements could imply certain level of deployment restriction.

Observation 3: The percentage value of grid shifting plays an important role to determine the distributions of adjacent-channel and co-channel gNB DL received power, by translating the distance between interfering gNB and victim gNB into pathloss.

Since the discussion for TX ACLR and OBUE requirements could be separated into (1) when coexisting with legacy TDD system in adjacent channel and (2) when coexisting with new SBFD system in adjacent channel. For the latter one, if new tightened requirement introduced, the CLI from adjacent channel could be greatly mitigated.

Proposal 3: RAN4 shall define the in-band blocking requirement by:

- For FR1 MR and LA BS and for FR2 BS, a unified requirement is defined by assuming the worst case 10% grid-shifting to allow deployment flexibility.

- For FR1 WA BS,

  - when coexisting with legacy TDD system in adjacent channel, 100% grid-shifting is assumed and RAN4 study how to capture this assumption as a deployment restriction in the specification.

  - when coexisting with new SBFD system in adjacent channel, FFS the in-band blocking requirement if tightened ACLR and/or OBUE requirements are introduced.

# 3. Conclusion

In this contribution, we provided the viewpoints on the modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS, accordingly with the following observations and proposals obtained:

Proposal 1: [1.0] dB degradation due to self-interference is assumed for OTA sensitivity requirement.

Proposal 2: In the existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations, by considering gNB-to-gNB CLI handling mechanism to be introduced in RAN1.

Proposal 3: RAN4 shall define the in-band blocking requirement by:

- For FR1 MR and LA BS and for FR2 BS, a unified requirement is defined by assuming the worst case 10% grid-shifting to allow deployment flexibility.

- For FR1 WA BS,

  - when coexisting with legacy TDD system in adjacent channel, 100% grid-shifting is assumed and RAN4 study how to capture this assumption as a deployment restriction in the specification.

  - when coexisting with new SBFD system in adjacent channel, FFS the in-band blocking requirement if tightened ACLR and/or OBUE requirements are introduced.

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[5] R4-2409764, “On the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411639, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#112.

[8] R4-2413515, “Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, RAN4#112.