3GPP TSG-RAN WG4 Meeting #112bis                                                          R4-2416502

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:	Discussion on OTA spatial emission conformance testing

Agenda item:			6.11.4

Document for:	Approval

# 1. Introduction

In ITU-R world radiocommunication conference (WRC) in Nov. 20 - Dec. 15 2023,  the frequency band of 6 425-7 125 MHz was identified in every ITU region as in the RR Footnotes 5.6A12, 5.6B12 and 5.6C12 with associated technical condition of limits on the expected equivalent isotropically radiated power (EIRP) spectral density of IMT base-stations, for the purpose of protecting earth-to-space fixed satellite services (FSS) as in Resolution 220 (WRC-23). Based on the newly released ITU-R regulation, the expected EIRP limits have been established which require the harmonized specification for compliance tests to be developed as the task in 3GPP.

Specifically, the aim of this task is to include the new regulation in BS (and other nodes like IAB, Repeater and NCR) RF core specification (TS 38.104) and a corresponding test requirement in the conformance test specification (TS 38.141-2). Furthermore, having the requirement included in the 3GPP standard would also guarantee a harmonized terminology and conformance test method. Accordingly, the performance part objective related to expected EIRP mask for upper 6GHz is provided in WID [1] as:

Expected EIRP mask for upper 6GHz - Specify manufacturer declarations, conformance test descriptions, test tolerances and test requirements. - Note: target completion date of this objective is December 2024.

And much progress on performance requirement has been achieved in RAN4#112 [8], with the following relevant agreements obtained:

Agreement: To declare the supported mechanical tilt for CAR/beam steering direction sets for BS conformance testing Agreement: To focus on the single user beamforming for EEIRP measurement. Agreement: - Proposal 1: EEIRP calculation in discrete form in the conformance test specification. EIRPθn,φm=k=1KwkEIRPkθn,φm,αk,βk , where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam. EEIRPi=θHHi-θHLisinθHHi-sinθHLiMN∙n=nLinHim=1MEIRPθn,θmcosθn Where - M is the number of the equally divided intervals over the horizontal (azimuth) range of −180° to +180°; - N is the number of the equally divided intervals over the specified vertical (elevation) range of θL to θH. - nL is the lowest elevation sampling angle within the θL , θH bounding range - nH is the highest elevation sampling angle within the θL , θH bounding range - θn is the vertical angle measured within the n-th interval out of the vertical range of θL to θH, where the interval is from θL+n-1NθH-θL to θL+nNθH-θL, - φm is the horizontal angle measured within the m-th interval out of the horizontal range of −180° to +180°, where the interval is from -π+2π∙m-1M to -π+2π∙mM. i means different bin index. Agreement: Only normal condition is considered. Agreement: To agree the test tolerance as 0dB;

Furthermore, the following open issues are also summarized and provided [8]:

Issue 4-2: Test vectors for EEIRP mask Agreement: Companies are encourage to provide the inputs for test beams for EERIP measurement; Issue 4-3: Measurement grid over elevation angle and azimuth angle Agreement: Companies are encourage to provide the inputs for measurement step size for EERIP measurement; Issue 4-4: RF channels for the conformance testing - Proposal 1: B, T [Nokia] - Proposal 2: M. [Huawei] - Proposal 3: T [Ericsson] Agreement: Companies are encourage to provide the inputs for RF channels for conformance testing Issue 4-6: Test procedures - Proposal 1: RAN4 need to capture the high-level test procedure above in BS RF conformance test specification in the new sub-clause for OTA spatial emission. - Proposal 2: RAN4 need to create entry in TR 37.941 for OTA spatial emission requirement and corresponding conformance test aspects, such as measurement procedure, calibration procedure and measurement uncertainty evaluation. Agreement: Companies are encourage to provide the inputs for test procedures for EEIRP measurement. Issue 4-7: Measurement uncertainty and CI - Proposal 1: EEIRP summation Error is 0.75dB [Huawei] - Proposal 2: follow the legacy approach adopted for TRP accuracy requirements as baseline [ZTE, Huawei] - Proposal 3 : MU and CIs are different measures and one cannot exchange one for the other but MU needs to be taken into account when estimating the CI [Spark] - Proposal 4: If the numbers of samples (N) is large say >20 the students t distribution and Normal distribution CIs are similar, and any can be chosen. [Spark] - Proposal 6: RAN5 to consider defining the estimation/measurement accuracy error as the difference between the expected EIRP over the oversampling factors (Oazim, Oelev) and (Lazim, Lelev) relative to the largest oversampling factor allowed (where testing is done). Agreement: Companies are encourage to provide the inputs for MU and CI values based on the assumption of test beams and measurement grid size. Issue 4-9: Others - Proposal 1: unless there are clear benefits identified to align the coordinate system between EEIRP mask and legacy 3GPP coordinate system, it’s not necessary to have such kind of converting. [ZTE]

In this contribution, we would like to provide our viewpoints on OTA spatial emission conformance testing, based on the WRC-23 outcome.

# 2. Discussion

## 2.1 EEIRP calculation in discrete spatial sampling grid

EEIRP calculation in discrete form in the conformance test specification is provided [8], and the following text proposal shall be noted and accepted for editorial correction.

Proposal 1: The following text proposal shall be adopted for EEIRP calculation in discrete spatial sampling grid in the conformance test specification:

Agreement: - Proposal 1: EEIRP calculation in discrete form in the conformance test specification. For i-th elevation angular range (θHLi ≤ θ < θHHi), EEIRP shall be calculated as EIRPθn,φm=k=1KwkEIRPkθn,φm,αk,βk , where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam. EEIRPi=θHHi-θHLisinθHHi-sinθHLiMN∙n=1nLiNnHim=1MEIRPθn,φmθmcosθn Where - EIRPθn,φm=k=1KwkEIRPkθn,φm,αk,βk, where wk is the beam weighting value of k-th test beam and EIRPkθn,φm,αk,βk is the EIRP pattern for the given direction of the vertical angle θn and the horizontal angle φm per k-th test beam with direction of (αk,βk). - M is the number of the equally divided intervals over the horizontal (azimuth) range of −180° to +180°; - N is the number of the equally divided intervals over the specified vertical (elevation) range of θHLiθL to θHHiθH. - nL is the lowest elevation sampling angle within the θL , θH bounding range - nH is the highest elevation sampling angle within the θL , θH bounding range - θn is the vertical angle measured within the n-th interval out of the vertical range of θHLiθL to θHHiθH, where the interval is from θHLi+n-1NθHHi-θHLi to θHLi+nNθHHi-θHLi, - φm is the horizontal angle measured within the m-th interval out of the horizontal range of −180° to +180°, where the interval is from -π+2π∙m-1M to -π+2π∙mM. - i means different bin index.

## 2.2 Declared CAR (Coverage Angular Range) and test vector in conformance Test

In the last RAN4 meeting, the definition of CAR declarations is given in the approved WF:

Agreement: Table 2.2.2-1: [CAR] declarations Parameter; Description. min; Minimum angle along  axis. max; Maximum angle along  axis. min; Minimum angle along  axis. max; Maximum angle along  axis. - NOTE 1: further discuss the definition of min max min max - NOTE 2: reusing the existing declaration is not precluded.

As described in the proponent company’s contribution [R4-2409118], the concept of CAR is defined as “a set of directions describing the intended angular operation range for which the expected EIRP requirement is met”.

Based on the above-mentioned agreement, the CAR concept could be visualized as a rectangular area in the q/j plane, in which the test beams will be located. Obviously, the number of test beams and the location within the q/j plane will impact the measured EEIRP profile.

Proposal 1: For conformance testing, Coverage Angular Range (CAR) can be introduced as “a set of directions describing the intended angular operation range for which the expected EIRP requirement is met” as [R4-2409118], with the restriction of:

(1) CAR declaration shall be totally up to BS vendor and associated with a certain mechanical tilt;

(2) All directions in the declared CAR shall be below horizon.

Next question is how to determine the weight for each beamforming direction within the test vector. Based on Annex to Resolution 220 (WRC-23), the beamforming directions (αn, βn) shall have a uniform statistical angular distribution within the steering range of the IMT base station. And for the given beamforming directions, the weight for each beamforming direction, i.e. the fraction of the steering range represented by the n-th beamforming direction, is also introduced by WRC-23. Therefore we propose that the weight for

Proposal 2: For conformance testing, the weight of each beamforming direction within the test vector shall be provided by gNB vendors based on their practical field experience. E.g., after N directions within test vector is determined, gNB vendors could provide the corresponding weights accordingly.

## 2.3 Mechanical tilt angle in conformance Test

Based on the agreement when specifying the core requirement, “the requirement shall apply to all supported mechanical tilts.” On the other hand, the following agreement is provided [8]:

Agreement: To declare the supported mechanical tilt for CAR/beam steering direction sets for BS conformance testing

Considering the much-increased test burden if all supported mechanical tilts shall be tested for BS conformance testing, we prefer to select one typical or worst-case mechanical tilt supported. For EEIRP requirement, the worst performance is more likely to be achieved at the lowest mechanical down-tilt angle, therefore we would like to propose that this angle is chosen in the conformance test.

Proposal 3: For conformance testing for spatial mask requirement, gNB shall be tested only with the lowest mechanical down-tilt angle (and accordingly the declared CAR associated to this mechanical down-tilt angle).

# 3. Conclusion

In this contribution, we provided our analysis on the expected EIRP mask for upper 6GHz (i.e., the OTA spatial emission requirement), and discuss the related BS conformance testing based on the WRC-23 outcome. Specifically, the following observations and proposals are provided:

Proposal 1: For conformance testing, Coverage Angular Range (CAR) can be introduced as “a set of directions describing the intended angular operation range for which the expected EIRP requirement is met” as [R4-2409118], with the restriction of:

(1) CAR declaration shall be totally up to BS vendor and associated with a certain mechanical tilt;

(2) All directions in the declared CAR shall be below horizon.

Proposal 2: For conformance testing, the weight of each beamforming direction within the test vector shall be provided by gNB vendors based on their practical field experience. E.g., after N directions within test vector is determined, gNB vendors could provide the corresponding weights accordingly.

Proposal 3: For conformance testing for spatial mask requirement, gNB shall be tested only with the lowest mechanical down-tilt angle (and accordingly the declared CAR associated to this mechanical down-tilt angle).

# 4. Reference

[1] RP-240829, “NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, Huawei, RAN#103, March 2024.

[2] Final Acts, World Radiocommunication Conference 2023 (WRC-23), 20th Nov.-15th Dec. 2023.

[3] R4-2405656, “Discussion on expected EIRP mask for upper 6GHz”, Samsung, RAN4#110bis, April 2024.

[4] R4-2406088, “Way Forward for [110bis][311] NR_BS_RF”, ZTE, RAN4#110bis, April 2024.

[5] R4-2409766, “Discussion on expected EIRP mask for upper 6GHz”, Samsung, RAN4#111, May 2024.

[6] R4-2409955, “Way Forward for [111][311] NR_BS_RF”, ZTE, RAN4#111, May 2024.

[7] R4-2411643, “Discussion on OTA spatial emission conformance testing”, Samsung, RAN4#112, Aug. 2024.

[8] R4-2413509, “Way Forward for [112][305] NR_BS_RF”, ZTE, RAN4#112, Aug. 2024.