3GPP TSG-RAN WG4 Meeting #112-bis                                       R4-2416503

# Hefei, China, 14th – 18th of October 2024

Title: 	On the VSAT and Satellite Parameters for Ku-band Coexistence

Agenda Item:	6.9.2

Source: 	THALES

Document for:	Discussion

# For information

As described in the approved Way Forward for [112][314] NR_NTN_Ku_Band_General (R4-2413519, Eutelsat Group), the following agreements have been made at the previous RAN4#112 meeting:

#### Issue 1-1: Work plan

#### The work plan in R4-2411506 is approved

#### Issue 1-2: Band definitions

Define as a first Ku band for Region 1 (CEPT countries): 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S).

Additional band definitions are expected once the applicable regulations have been identified and studied.

#### Issue 2-2: Coexistence scenarios

Adopt the interference scenarios in [Intelsat] R4-2411507 for the coexistence studies

#### Issue 2-4: Coexistence frequencies

#### Use 11GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band.

#### Issue 2-8: Coexistence TN channel model

For TN channel model, reuse existing version of TR 38.901.

#### Issue 2-9: Coexistence TN channel model

For NTN channel model, reuse parameters from TR 38.811 and collect the following missing information:

- Shadow fading and clutter loss tables for the reference frequencies

- Tropospheric scintillation.

#### Issue 2-10: TN Parameters

As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from:

- The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz

- The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

# Proposals

For the time being, the following NTN Ku-band has been agreed in WF R4-2413519: 10.7 - 12.75 GHz (DL); 14.0 - 14.5 GHz (UL). Moreover, it seems that SAN/VSAT UE parameters for Ku-band discussed at previous meeting were incomplete (see e.g. R4-2411507, Intelsat; R4-2320973, THALES).

Based on current Ku-band UL/DL assumption, with the use of 11 GHz for DL coexistence evaluation and 14 GHz for UL coexistence evaluation, in this contribution we propose the Ku-band VSAT and Satellite parameters to be considered for the coexistence simulation/link budget evaluation as follows:

Proposal 1: RAN4 to use following Satellite parameters for Ku-band work:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 9.1 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.177 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 7.14 m | 0.714 m | 0.714 m |
| G/T |  | 31.4 dB K-1 | 11.4 dB K-1 | 11.4 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Proposal 2: RAN4 to use following VSAT UE parameters for Ku-band work:

- Option 1/2

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 36.6 dBi |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |

- Option 2/2 (for EIRPmax=76.2dBm equivalent to Ka-band) – with increase of the transmission power up to 10W instead of 2W.

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 10 W (40 dBm) |
| Tx antenna gain | 36.2 dBi |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |

# Conclusion

Proposal 1: RAN4 to use following Satellite parameters for Ku-band work:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 9.1 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.177 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 7.14 m | 0.714 m | 0.714 m |
| G/T |  | 31.4 dB K-1 | 11.4 dB K-1 | 11.4 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Proposal 2: RAN4 to use following VSAT UE parameters for Ku-band work:

- Option 1

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 36.6 dBi |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |

- Option 2 (for EIRPmax=76.2dBm equivalent to Ka-band) – with increase of the transmission power up to 10W instead of 2W.

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 10 W (40 dBm) |
| Tx antenna gain | 36.2 dBi |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |

# Annex

Please recall the Ka-band values (they are also in the TR 38.821 from the study phase):

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 6.1.1.1-3: UE characteristics for system level simulations

| Characteristics | VSAT (Note 2) | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear: +/-45°X-pol | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | TBD dBi per element |
| Antenna temperature | 150 K | 290 K | TBD K |
| Noise figure | 1.2 dB | 7 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | TBD dBi per element |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |