3GPP TSG-RAN WG4 Meeting RAN4#112bis      				R4-2416505 Hefei, China

October 14 - 18, 2024

Agenda Item:	4.2.2.6

Source:	Lenovo

Title:	On the Need for Additional Scenarios for CPP

Document for:  Approval

- Introduction

The effect of carrier frequency offset on carrier phase measurements was discussed extensively in [1-8].  The fundamental issue is that the double differencing method that is the basis of the carrier phase positioning method assumes that the phase offset between the TRP and the UE depends only on the initial phase offsets of the TRP and the UE and the distance between them.  In actuality, the carrier phase offset between the UE and the TRP is constantly changing due to the carrier frequency offset that always exists in practice. The consequence of this time variation of the carrier phase is that both the carrier phase measurement and the carrier phase difference measurements are not well defined for measurement window sizes larger than one.

Observation 1:	Due to the carrier frequency offset between the TRP and the UE, carrier phase and carrier phase difference are not well-defined for measurement window sizes larger than one.

It follows that if the carrier phase is not well defined, then the carrier phase measurement error is also not well defined.

To address this issue, it was proposed that a reference time be used to clarify the definition of carrier phase so that in the presence of a carrier frequency offset, the carrier phase measurements taken anywhere within the measurement window could be referred to this reference time.  The use of such a reference time would not only clarify the definition, but it would allow carrier phase measurements to be combined over larger time windows, thus improving the signal-to-noise ratio of the measurements.

To address the issue of carrier frequency offset, the following agreement was made in RAN4#102:

Agreement:

- RAN4 to clarify that the RSCPD accuracy requirement and the relative RSCP accuracy requirement apply given that the carrier phase measurements are performed on PRS resources within a slot/a set of symbols, which can be controlled at least by the window configuration.

- Companies can bring proposals on other scenarios in WI performance maintenance phase.

In the remainder of this contribution, we consider some of the implications of this agreement, and also comment on the need to consider some additional scenarios.

- On the Need for Additional Scenarios

The intention of the agreement in the last meeting is clearly to minimize the ambiguity of the carrier phase definition in the presence of a carrier frequency offset by making the measurement window very small.  However, there is no guidance as to how small the window should be other than the PRS resources should be contained within a slot. As noted in the introduction, carrier phase and carrier phase difference are not well defined for any window size larger than one symbol.

In previous contributions, it has been shown that the maximum symbol-to-symbol rotation of the carrier phase of the TRP relative to the UE can be as large as

Δθmax=5.1 fc 2-μ degrees/symbol

where fc  is the carrier frequency in GHz and μ = 0, 1, 2, and 3 for subcarrier spacings of 15, 30, 60, and 120 kHz, respectively.  As an example, for the specific case in which fc equals 4 GHz and the subcarrier spacing is 30 kHz (in simulation scenarios [9]), the worst-case symbol-to-symbol variation of the carrier phase of the TRP relative to the UE is 10.2 degrees/symbol.

Observation 2:	The maximum symbol-to-symbol rotation of the carrier phase of the TRP relative to the UE can be as large as

Δθmax=5.1 fc 2-μ degrees/symbol

where fc  is the carrier frequency in GHz and μ = 0, 1, 2, and 3 for subcarrier spacings of 15, 30, 60, and 120 kHz, respectively.

As a result, even for measurement windows much less than a slot, the phase rotation of the TRP carrier relative to that of the UE can be quite large.  With the current agreement, there seems to be an expectation that the carrier frequency offset issue is completely resolved and that the lack of definition of carrier phase in the presence of carrier frequency offset will not impact measurements for measurement windows less than a slot.

A second issue to consider with respect to the agreement is that it will not be possible to test the behavior of the UE with PRS repetition since no requirements will be defined.  There is significant benefit with PRS repetition as the signal-to-interference ratio of the measurement scales linearly with the size of the measurement window. It can be noted that PRS repetition was included in the simulation scenarios for this reason.  Almost one-half of the simulation scenarios in [9] include a repetition factor of 4.

Observation 3:	With the current agreement, the performance of the UE with PRS repetition will be unspecified.

The benefit of PRS repetition on measurement accuracy can be seen in Figures 1 and 2.  Here, the cumulative distribution function of the carrier phase error is shown with and without PRS repetition for one of the simulation scenarios defined in [9] and shown in Tables 1 and 2 in the Appendix.  It can be observed that the 90th percentile phase error is reduced from 10 degrees to 5 degrees with PRS repetition equal to 4.

Figures 1 and 2:  Cumulative distribution of the absolute phase measurement error, no CFO. 			Es/Iot for three cells (-6, -13, -13 dB)

The third issue to consider with respect to the agreement is that there will always be a carrier frequency offset between the TRP and the UE in the field.  With the current agreement, the behavior of the UE with a carrier frequency offset will be both undefined and unspecified.

Observation 4:	With the current agreement, the measurement behavior of the UE when there is a frequency offset between the TRP and the UE (and which always exists in the field) is both undefined and unspecified.

With a carrier frequency offset, the carrier phase is constantly changing and so it is not possible to define the carrier phase without also defining a reference time for the carrier phase with respect to the measurement window. Similarly, without a definition of carrier phase, it is not possible to define the carrier phase measurement error.

The simulation results in Figure 3 show the cumulative distribution of the phase error for another of the scenarios in [9] (absolute phase error is not used because the error is not symmetric about 0 degrees).  In this scenario, Kcomb is equal to 4 and there is no repetition so that the measurement window has size 4. The carrier frequency is 4 GHz and the carrier frequency offset is -0.05 ppm which corresponds to an offset of -200 Hz.  The subcarrier spacing is 30 kHz and the carrier bandwidth is 10 MHz. The resulting symbol-to-symbol phase rotation of the PRS relative to the UE is approximately -2.6 degrees.

In these simulations, the UE averages its carrier phase measurements without knowledge of the carrier frequency offset. The reference time used in the definition of the carrier phase error was the start of the slot containing PRS, where the PRS symbols are offset from the start of the slot by 2 symbols.  The measured error is the difference between the UE measurement average and the carrier phase at the defined reference time. In the example in Figure 3. the carrier phase measurement error is significantly impacted by the carrier frequency offset even though the measurement window is only 4 symbols. It should be noted that the error reported in Figure 3 depends on the defined reference time.

Figures 3:  Cumulative distribution of the phase measurement error with CFO = -0.05 ppm (200 Hz @ 4 GHz).  Es/Iot for three cells 0, -6, -6 dB)

Based on the observations above, we propose that RSCPD accuracy requirements and relative RSCP accuracy requirements be defined for scenarios with PRS repetition and carrier frequency offsets.

Proposal: 	RSCPD accuracy requirements and relative RSCP accuracy requirements be defined for scenarios with PRS repetition and with carrier frequency offsets.

- Summary

In this contribution, we have considered some of the consequences and limitations resulting from the agreement that “the RSCPD accuracy requirement and the relative RSCP accuracy requirement apply given that the carrier phase measurements are performed on PRS resources within a slot/a set of symbols,” and have the following observations:

Observation 1:	Due to the carrier frequency offset between the TRP and the UE, carrier phase and carrier phase difference are not well-defined for measurement window sizes larger than one.

Observation 2:	The maximum symbol-to-symbol rotation of the carrier phase of the TRP relative to the UE can be as large as

Δθmax=5.1 fc 2-μ degrees/symbol

where fc  is the carrier frequency in GHz and μ = 0, 1, 2, and 3 for subcarrier spacings of 15, 30, 60, and 120 kHz, respectively.

Observation 3:	With the current agreement, the performance of the UE with PRS repetition will be unspecified.

Observation 4:	With the current agreement, the measurement behavior of the UE when there is a frequency offset between the TRP and the UE (and which always exists in the field) is both undefined and unspecified.

Based on these observations, we have the following proposal:

Proposal: 	RSCPD accuracy requirements and relative RSCP accuracy requirements be defined for scenarios with PRS repetition and with carrier frequency offsets.

# References

- R1-2204807, “Design Aspects of Carrier Phase Measurements for NR Positioning”, Intel, #109-e.

- R4-2311991, “Effect of Carrier Frequency Offset on Carrier Phase Positioning,” Lenovo, Motorola Mobility, RAN4#108.

- R4-2316796,” Carrier Frequency Offset Correction for CPP,” Lenovo, Motorola Mobility, RAN4#108bis.

- R4-2318899, “Correction of Carrier Phase Measurement Errors Due to Carrier Frequency Offsets,” Lenovo, Motorola Mobility, RAN4#109.

- R4-2402603, “Positioning Error Due to Carrier Frequency Offset,” Lenovo, RAN4#110.

- R4-2405873, “Correction of CPP Errors Due to Carrier Frequency Offsets,” Lenovo, RAN4#110bis.

- R4-2409575, “Definition of Carrier Phase and Carrier Phase Difference with Carrier Frequency Offsets,” Lenovo RAN4#111.

- R4-2413461, “On the definition of Carrier Phase and the Impact of CFO on Averaging Carrier Phase Measurements,” Lenovo, RAN4#112.

- R4-2321459,” Updated simulation assumptions for CPP measurement,” CATT, RAN4#109.

- 3GPP TS 38.104 NR; Base Station (BS) radio transmission and reception

- 3GPP TS 38.101-1 User Equipment radio transmission and reception; Part 1: Range1 Standalone

Appendix: Simulation Scenarios from [9]

Table 1: General parameters (subset of Table 1 from [8])

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 30 | 24 | 1 | 128 |
|  |  | 24 | 4 | 128 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| Number of samples | 1, 4 |  |  |  |
| PRS periodicity | 40ms, 200ms |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |
| NOTE 1: Repetition 4 is achieved by PRS in 4 consecutive slots with PRS-ResourceTimeGap=1. NOTE 2: Companies are encouraged to check suitability of the parameters and the values. |  |  |  |  |

Table 2: PRS transmission configuration parameters (subset of Table 2 from [8])

| Parameter | Value |
| --- | --- |
|  | FR1 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the reference cell |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> |
| Duplex modes | FDD |
| Carrier frequency / SCS / duplex mode | - 4 GHz - 30 kHz - FDD |
| Propagation conditions | AWGN, |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-6, -13, -13)dB for NSample = 1; (-3, -6, -6)dB for NSample = 1. (-3, -13, -13)dB for NSample = 1; (0, -6, -6)dB for NSample = 1. |
| Number of UE receive antennas | 2 |
| UE measurement bandwidth | Full carrier bandwidth |
| Number of transmit PRS antennas | 1 |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |