3GPP TSG-RAN WG4 Meeting #112bis	R4-2416506

Hefei, China, 14th October – 18th October 2024

Agenda item:	6.16.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on BS demodulation for MMSE-IRC.

Document for:	Discussion

- Introduction

In the last meeting RAN4 #111, it was discussed that RAN4 impact of adding TDD band. We should specify PRACH format 1 for BS demodulation part to add TDD band for HAPS. In this contribution, we state the view of PRACH specification content.

- Discussion

Test scenarios

According to RAN4#112 WF [1], the status is noted as below,

- Proposals: - Proposal 1: Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104 - Proposal 2: Consider the scenario with different TDD patterns between BS and discuss how to model the corresponding interference profile

LTE test requirements for MMSE-IRC covers both of HomNet and HetNet scenario. Both scenarios are deployed NR network. Thus, we proposed cover HomNet and HetNet scenarios. For different TDD pattern, it is out of scope based on latest WID, but we are fine to extend the TDD pattern for deployment scenarios.

Proposal 1: BS NR MMSE-IRC covers both of HomNet and HetNet scenarios.

Proposal 2: Different TDD patterns are not required by WID, but it’s fine to extend the scope if needed.

Interference profile

According to RAN4#112 WF [1], the status is noted as below,

- Candidate options on interference modelling: – Option 1: Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH – Other options not precluded - On interference power level: – Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. – It is not precluded for companies to bring system level simulation on other profiles. – FFS how to account for HPUE in existing LTE profiles.

This topic is not SI but WI. There is no big difference LTE and BS, thus LTE interference profile can be reused.

Proposal 3: BS interference profile for LTE MMSE-IRC can be reused for that of NR.

Waveform

WF notes the option as below,

• Candidate options for CP-OFDM: – Option 1: o For 15kHz SCS, cover 5MHz, 10MHz, 20MHz o For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz – Option 2: 10MHz/15kHz and 40MHz/30kHz for initial simulation purpose – Option 3: Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz – Option 4: Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz • Candidate options for DFT-s-OFDM: – Option 1: o For 15kHz SCS, 5MHz o For 30kHz SCS, 10MHz

MMSE-IRC can be applied to almost all BS in commerce. We should cover not only CP-OFDM but also DFT-s-OFDM.

Proposal 4: We support CP-OFDM and DFT-s-OFDM for MMSE-IRC.

Antenna configuration

We discussed the option 1, but it was not concluded as below,

• Candidate options: – Option 1: Consider 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna – Other options not precluded

LTE BS MMSE-IRC considers only 1 interferer for 2/4/8 Rx MMSE-IRC, but the increase of the number of receiver antennas MMSE-IRC can improve capability of the interference rejection. We should discuss the effect by increasing number of Tx antennas based on simulations.

Proposal 5: Support 1 interferer for 2/4/8 Rx antennas. RAN4 clarify the effect by increasing number of Tx antennas based on simulations.

- Conclusion

This contribution states general aspects of  NR BS MMSE-IRC. The summary is follows.

Proposal 1: BS NR MMSE-IRC covers both of HomNet and HetNet scenarios.

Proposal 2: Different TDD patterns are not required by WID, but it’s fine to extend the scope if needed.

Proposal 3: BS interference profile for LTE MMSE-IRC can be reused for that of NR.

Proposal 4: We support CP-OFDM and DFT-s-OFDM for MMSE-IRC.

Proposal 5: Support 1 interferer for 2/4/8 Rx antennas. RAN4 clarify the effect by increasing number of Tx antennas based on simulations.

References

- R4-2413570, WF on NR demodulation performance: Phase 5, China Telecom, RAN4#112, August 2024