3GPP TSG-RAN WG4 Meeting #112-bis	R4-2416510

Hefei, China, October 14 – October 18, 2024

Agenda Item:	6.23.4.2

Source:	MediaTek Inc.

Title:	Discussion on the RRM core requirements for LP-WUS

Document for:	Discussion

# Introduction

The RRM core requirements for Rel-19 LP-WUS/WUR was further discussed in the last RAN4 meeting, and the achieved agreements are captured in the WF [1]. In this paper, we continue the discussion on the open issues in the RRM core part and provide our view. The WID [2] is also provided as below for reference.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

# Discussion

## Cases/states of RRM relaxation for LP-WUS capable UE

In the last meeting, RAN4 discussed the possible RRM offloading/relaxation cases of LP-WUS capable UE that can be considered in Rel-19 LP-WUS. In RAN4#111 meeting, RAN4 agreed on Case#1 (RRM offloading case) first and then in RAN4#112 meeting, RAN4 agreed on Case#3 (RRM relaxation case) as captured below.

RAN4#111 agreement: Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON. RAN4#112 agreement Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

Regarding whether to discuss other cases for LP-WUS, in our view the current agreed cases in RAN4, Case#1 and Case#3, are sufficient to consider both scenarios of MR RRM offloading and MR RRM relaxation of serving and neighbour cell measurements, which align with the WID objective in [2]. Therefore, RAN4 should focus on defining the core requirements for the current agreed cases, and if some issues are detected later to consider other cases, it can be revisited.

Proposal 1: No need to discuss further cases for LP-WUS, and RAN4 to start discussing the delay requirements instead for the agreed cases. If some issues are detected later to consider other cases, it can be revisited.

## Serving cell measurements by LP-WUS capable UE

Following the progress in RAN4, LP-WUS capable UE may operate under three scenarios (see Figure 1):

- RRM offloading (Case#1): when MR is deactivated and LR is activated.

- RRM relaxation (Case#3): when both MR and LR are activated.

- Legacy UE: when MR is activated and LR is deactivated.

Figure 1: Different cases of LP-WUR UE

In Case#1, MR is deactivated and LR is activated. LR should perform wake-up signal monitoring and serving cell measurements in RRC IDLE/INACTIVE mode. All other tasks performed by MR (paging, serving and neighbour cell RRM measurements) are deactivated for power saving. Since, in this case, serving cell measurement is exclusively performed by LR, UE needs to rely on the LR’s RRM measurements to determine whether the UE remains camped on a suitable cell. LR shall measure the RSRP and/or RSRQ level of the serving cell (based on LP-SS or SSB of serving cell, depending on LR implementation) to evaluate the cell selection criterion S. LR shall evaluate the cell selection criterion S of the serving cell within X number of measurement cycles. If LR evaluation for serving cell does not fulfil cell selection criterion S within X measurement cycles, the UE (MR) shall initiate the measurements of all neighbour cells indicated by the serving cell by triggering the MR to wake up (i.e., exit from LP-WUS monitoring mode and activate MR).

Observation 1: In Case#1 (RRM offloading), serving cell measurement is exclusively performed by LR, therefore, UE needs to rely on the LR’s RRM measurements to determine whether the UE remains camped on a suitable cell.

Proposal 2: RAN4 to define serving cell requirements for Case#1 (RRM offloading) based on LR evaluation for serving cell using RSRP and/or RSRQ measurements obtained from LP-SS or PSS/SSS (depending on LR implementation).

Proposal 3: In Case#1 (RRM offloading), LR shall evaluate the cell selection criterion S of the serving cell within X number of measurement cycles. X is FFS.

Proposal 4: In Case#1 (RRM offloading), exit condition can be defined when serving cell evaluation by LR does not meet the criterion S.

Proposal 5: In Case#1 (RRM offloading), if cell selection criterion S is not fulfilled by LR within X measurement cycles, the UE (MR) shall initiate the measurements of all neighbour cells indicated by the serving cell by triggering the MR to wake up (i.e., exit from Case#1 and activate MR).

In Case#3, both MR and LR are activated. LR performs wake-up signal monitoring and serving cell measurements in RRC IDLE/INACTIVE mode. MR performs relaxed serving and neighbour cell measurements in IDLE/INACTIVE mode. Since, in this case, serving cell measurements are performed by both MR and LR, different solutions can be applied to determine whether the UE remains camped on a suitable cell as captured below from the WF [1].

- Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios.

In our view, combining the measurements across the radios (MR+LR) seems complicated. LR measurements can be based on LP-SS or PSS/SSS, depending on LR implementation, while MR measurement is based on SSB in Idle/Inactive mode. Therefore, it is not straightforward to consider all the combination of combining different reference signals from two different receivers with different accuracy. It is also worth mentioning that RAN4 never combined two different measurements from deferent RS in the legacy requirements.

Proposal 6: For Case#3 (RRM relaxation), RAN4 not to consider combining the measurements across the two radios, i.e., MR and WUR.

Therefore, to perform serving cell evaluation in Case#3 (RRM relaxation), we prefer a simple procedure by relying only on LR measurements for serving cell, without combing MR serving cell measurements. This will also help achieving more power saving in Case#3 when MR is activated. In addition, it will simplify the requirements for both cases, Case#1 and Case#3, where serving cell evaluation will be based on LR measurements only for both.

Proposal 7: For Case#3 (RRM relaxation), RAN4 to use LR serving cell measurements only to define serving cell evaluation requirements, same as Case#1, (i.e., without combining MR serving cell measurements).

## Entry/Exit conditions

Transitioning between different cases (Case#1, Case#3 and legacy UE) as shown in Figure 1, is based on the entry/exit conditions for each case. These conditions can be met by either MR or LR, depending on the current state of the UE. Analysing all the transitioning cases, we observe that transitioning:

- From legacy UE to Case#1: entry condition is based on MR.

- From legacy UE to Case#3: entry condition is based on MR.

- From Case#1 to legacy UE: exit condition is based on LR.

- From Case#1 to Case#3: entry condition is based on LR.

- From Case#3 to legacy UE: exit condition is based on MR and/or LR.

- From Case#3 to Case#1: entry condition is based on MR and/or LR.

As it can be noticed from above, the evaluation of entry/exit criteria is either based on MR or LR or both MR+LR. To simplify the evaluation requirements in RAN4, we observe that it is possible to define the entry conditions based on MR thresholds only (e.g., when entering RRM fully offloading case from legacy UE) and exit conditions can be based on LR thresholds only (e.g., when exiting RRM fully offloading to legacy UE).

Proposal 8: RAN4 to discuss the delay requirements to evaluate the entry criteria by using MR measurement results only and to evaluate exit criteria by using LR measurement results only.

Proposal 9: Same entry/exit conditions are used for both LP-WUS monitoring and LP-WUR serving cell measurements.

In RAN2, the entry/exit conditions were discussed in the last meeting and some agreements were made as captured below:

RAN2 agreement: - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

The discussion in RAN2 on entry/exit conditions were focused on the configurations of the thresholds for MR and LR in general (i.e., without considering the different cases which were agreed later in RAN4). In RAN4, two cases were agreed, Case#1 for RRM offloading and Case#3 for RRM relaxation. RAN4 therefore needs to discuss whether and how to differentiate these entry/exit conditions for the two cases (Case#1 and Case#3). One option will be to define multiple thresholds (e.g., two thresholds) to differentiate the entry/exit condition to Case#1 or Case#3, which may need to inform RAN2. Another option will be to define single threshold for both cases, Case#1 and Case#3, however, UE enters, for example, Case#3 (RRM relaxation) first and after some confidence time period T UE enters Case#1 (RRM offloading) to allow UE confidently offloading RRM to LR, with this option may not need to inform RAN2.

Proposal 10: RAN4 to discuss whether and how to differentiate the entry/exit conditions for Case#1 and Case#3. The following options can be considered:

- Option 1: Multiple thresholds: separate entry/exit thresholds are defined for different cases, Case#1 and Case#3.

- Option 2: Single threshold: single entry/exit threshold is defined for both cases, Case#1 and Case#3, in which UE enters Case#3 (RRM relaxation) first, then enters Case#1 (RRM offloading) after some confidence period T.

## MR RRM relaxation

The MR RRM relaxation can be achieved by using a scaling factor for the RRM measurements performed in IDLE/INACTIVE mode. It is noted that different scaling factors can offer different power saving gain for the UE. RAN4 can discuss the following options for RRM relaxation:

- Option 1: MR RRM Relaxation from 8 to 16 times

- Option 2: MR RRM Relaxation >=16 times

There is a trade-off between power saving gain and the delay. The higher the scaling factor, the greater the power savings that can be achieved, and the longer the delay will be (also vice versa). In the last meeting, some companies supported option 1 and others supported option 2. RAN4, however, can consider multiple stages of RRM relaxation with different scaling factors (i.e., not to limit the relaxation on a single value), which can depend on some conditions (e.g., channel condition, UE mobility, UE location). For example, MR RRM can be relaxed with a larger value when some conditions are met (e.g., UE closer to cell centre) otherwise a smaller value can be used. Note that this relaxation will only apply for Case#3 (when both MR and LR are on).

Proposal 11: RAN4 to discuss the following options for RRM relaxation:

- Option 1: MR RRM Relaxation from 8 to 16 times

- Option 2: MR RRM Relaxation >=16 times

Proposal 12: RAN4 to discuss multiple stages of RRM relaxation, with different scaling factors (e.g., 8 in 1st stage and 16 in 2nd stage) depending on some conditions (e.g., channel condition, UE mobility).

On the other hand, when UE operates in Case#3 (MR and LR are on), the relaxation factor applied for MR serving cell measurement can also be applied for MR neighbour cell measurements.

Proposal 13: Same relaxation factor applies to serving and neighbour cell measurements.

## Others

Some other issues were also discussed in RAN4 as captured in the WF [1] below:

Issue 1-2-1: Accuracy requirements Recommendations: How and where to capture the accuracy requirement can be discussed later. Issue 1-2-4: LP-WUR operating carrier frequency Issue 1-2-6: Nubmer of Rx for LP-WUR requirements Recommendations: Encourage companies to check whether P1 of this issue is agreeable or not.

For Issue 1-2-1, we support that not to capture dedicated accuracy requirement in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 14: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

For Issue 1-2-4, this issue is also related to the received LS from RAN1 (R1-2407558), and in our understanding the defined core requirements in RAN4 shall apply assuming LR and MR are operating on the same carrier frequency as baseline.

Proposal 15: RAN4 assumed LR and MR are operating on the same carrier frequency as baseline.

For Issue 1-2-6, we also support P1 as discussed in the last meeting, that is to assume a single Rx for LR when defining the RRM measurement requirements in RAN4.

Proposal 16: Single Rx is assumed for LR based RRM measurement.

# Summary

In this contribution, the following observations are provided:

Observation 1: In Case#1 (RRM offloading), serving cell measurement is exclusively performed by LR, therefore, UE needs to rely on the LR’s RRM measurements to determine whether the UE remains camped on a suitable cell.

Also, the following proposals are concluded:

Proposal 1: No need to discuss further cases for LP-WUS, and RAN4 to start discussing the delay requirements instead for the agreed cases. If some issues are detected later to consider other cases, it can be revisited.

Proposal 2: RAN4 to define serving cell requirements for Case#1 (RRM offloading) based on LR evaluation for serving cell using RSRP and/or RSRQ measurements obtained from LP-SS or PSS/SSS (depending on LR implementation).

Proposal 3: In Case#1 (RRM offloading), LR shall evaluate the cell selection criterion S of the serving cell within X number of measurement cycles. X is FFS.

Proposal 4: In Case#1 (RRM offloading), exit condition can be defined when serving cell evaluation by LR does not meet the criterion S.

Proposal 5: In Case#1 (RRM offloading), if cell selection criterion S is not fulfilled by LR within X measurement cycles, the UE (MR) shall initiate the measurements of all neighbour cells indicated by the serving cell by triggering the MR to wake up (i.e., exit from Case#1 and activate MR).

Proposal 6: For Case#3 (RRM relaxation), RAN4 not to consider combining the measurements across the two radios, i.e., MR and WUR.

Proposal 7: For Case#3 (RRM relaxation), RAN4 to use LR serving cell measurements only to define serving cell evaluation requirements, same as Case#1, (i.e., without combining MR serving cell measurements).

Proposal 8: RAN4 to discuss the delay requirements to evaluate the entry criteria by using MR measurement results only and to evaluate exit criteria by using LR measurement results only.

Proposal 9: Same entry/exit conditions are used for both LP-WUS monitoring and LP-WUR serving cell measurements.

Proposal 10: RAN4 to discuss whether and how to differentiate the entry/exit conditions for Case#1 and Case#3. The following options can be considered:

- Option 1: Multiple thresholds: separate entry/exit thresholds are defined for different cases, Case#1 and Case#3.

- Option 2: Single threshold: single entry/exit threshold is defined for both cases, Case#1 and Case#3, in which UE enters Case#3 (RRM relaxation) first, then enters Case#1 (RRM offloading) after some confidence period T.

Proposal 11: RAN4 to discuss the following options for RRM relaxation:

- Option 1: MR RRM Relaxation from 8 to 16 times

- Option 2: MR RRM Relaxation >=16 times

Proposal 12: RAN4 to discuss multiple stages of RRM relaxation, with different scaling factors (e.g., 8 in 1st stage and 16 in 2nd stage) depending on some conditions (e.g., channel condition, UE mobility).

Proposal 13: Same relaxation factor applies to serving and neighbour cell measurements.

Proposal 14: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 15: RAN4 assumed LR and MR are operating on the same carrier frequency as baseline.

Proposal 16: Single Rx is assumed for LR based RRM measurement.

# References

- R4-2413899, WF for RRM core requirements for LP-WUSWUR, vivo.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO.

- 3GPP TR 38.869, “Study on low-power wake-up signal and receiver for NR (Release 18)”, V18.0.0.