3GPP TSG-RAN WG4 Meeting #112-bis	R4-2416512

Hefei, China, October 14 – October 18, 2024

Agenda Item:	6.15.3

Source:	MediaTek Inc.

Title:	Discussion on the RRM requirements for fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

The objective of R19 NR RRM Phase-5 work item was agreed in the WID as captured below [1]. In the last meeting, RAN4 started the work on the second objective of the WID “Fast SCell activation for UE supporting Rel-18 EMR”. Therefore, in this paper, we continue to discuss the RRM core requirements for achieving fast SCell activation for UE supporting R18 EMR.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

For the UE supporting Rel-18 EMR, the UE can be configured to perform RRM measurements in idle/inactive mode and then check the validity of these measurements before reporting them in the connected mode. The validity of Rel-18 EMR can be checked based on whether the measurement is performed within [X] seconds before msg1 transmission for RRC resume/setup request.

In order to utilize Rel-18 EMR reporting for fast SCell activation, RAN4 needs to discuss the required conditions that ensure the applicability of using R18 EMR towards enhancing SCell activation delay.

Proposal 1: RAN4 to discuss the conditions required to ensure the applicability of using R18 EMR reporting to achieve fast SCell activation.

In the last meeting, RAN4 discussed the applicability of fast SCell activation delay requirements as captured in the WF [1].

#### Issue 1-1-2: Applicability of fast SCell activation delay requirements Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured - and the UE has reported valid results on the SCell to be activated before SCell activation command.

Regarding the FFS part, if UE support R18 EMR but is not configured with the validity check, this can result in UE reporting some old measurement results from the IDLE mode to the NW when connection is resumed/setup. Therefore, these results cannot reflect the current status of the SCell to be activated and should not be applicable to fast SCell activation. In addition, according to the WID, it is clear that fast SCell activation is applicable when UE send valid EMR reporting, which in our view, should meet the validity check of R18 EMR and the measurement accuracy as well.

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

Proposal 2: Fast SCell activation delay requirements should not be defined if UE support R18 EMR but not configured with validity check since EMR reporting cannot be checked for its validity anymore.

One of the main issues that RAN4 need to address is that whether the reported Rel-18 EMR in connected mode is fresh enough to be considered for SCell activation delay enhancement. One scenario can be, for example, the measurements performed in idle/inactive mode are valid (based on validity check of R18 EMR) but happened at the beginning of X window used for validity check, where X value can be very large according to R18 Specs. Then if these measurement results are reported in connected mode after some time, RAN4 need to study whether the report can still be used for SCell activation enhancement. In our view, RAN4 need to introduce a new time window Y to represent the validity of R18 EMR for SCell activation delay reduction.

Figure 1: Fast SCell activation for UE supporting Rel-18 EMR

If the above condition is met, then the unknown SCell can be activated by applying the activation delay used for the known SCell scenario, i.e., significant delay reduction can be achieved with respect to the unknown SCell activation delay. Otherwise, if the above condition is not met, legacy SCell activation shall apply. This can be achieved by updating the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command as below.

Proposal 3: RAN4 to update the known condition for SCell activation to include the following conditions:

- Rel-18 EMR of the SCell-to-be-activated is sent within [Y] time window before the reception of the SCell activation command.

- The SSB measured of the SCell-to-be-activated remains detectable during [Y]

- FFS [Y]

Proposal 4: If the updated known conditions for SCell activation are not met, legacy SCell activation requirements shall apply.

# Summary

In this contribution, the following proposals are concluded:

Proposal 1: RAN4 to discuss the conditions required to ensure the applicability of using R18 EMR reporting to achieve fast SCell activation.

Proposal 2: Fast SCell activation delay requirements should not be defined if UE support R18 EMR but not configured with validity check since EMR reporting cannot be checked for its validity anymore.

Proposal 3: RAN4 to update the known condition for SCell activation to include the following conditions:

- Rel-18 EMR of the SCell-to-be-activated is sent within [Y] time window before the reception of the SCell activation command.

- The SSB measured of the SCell-to-be-activated remains detectable during [Y]

- FFS [Y]

Proposal 4: If the updated known conditions for SCell activation are not met, legacy SCell activation requirements shall apply.

# References

- R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT.