3GPP TSG-RAN WG4 Meeting #112-bis                                       R4-2416520

# Hefei, China, 14th – 18th of October 2024

Title: 	Discussion on the RAN2 LS Reply on Common TA in a Regenerative Payload Scenario

Agenda Item:	6.26.4

Source: 	THALES

Document for:	Discussion

# For information

In the LS R1-2407590 (R2-2407623) on common TA in a regenerative payload scenario [1], RAN2 kindly requests RAN4 to provide feedback on the question below:

# 1. Overall Description: In RAN2#127 meeting, the setting of common TA and Kmac for regenerative payload with full gNB on board was discussed, and a question related to the value setting was raised. Related to this, RAN2 kindly asks RAN1 and RAN4: Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0. # 2. Actions: # To RAN1 and RAN4: # RAN2 kindly requests RAN1 and RAN4 to provide feedback on above question.

In this contribution we provide our view on the topic of common TA in a regenerative payload scenario.

# Configuration of Common TA in a regenerative payload scenario

We would like to recall that Common TA as agreed/defined in RAN1#104-bis-e: NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

The actual value of Common TA depends on the relative position of the reference point of uplink synchronization with regards to the satellite. During Rel-17 normative work it was agreed to support also a value equal to zero of Common TA. The intention is to support the deployment scenario where the reference point for uplink sync is located onboard the satellite. Such deployment applies specifically to regenerative payload. The value ranges of Common TA and related parameters (1st and 2nd order parameters) where specified at RAN1#107-e:

RAN1#107-e Agreement Confirm the Working assumption on granularity and bits allocation for Common TA parameters: Value range, granularity and bits allocation of Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are as follows: Parameter name; Value range; Granularity; Bits allocation. 0 ...66485757 (i.e: 0… 270.73 ms); 26 bits. TACommonDrift; - 261935… + 261935 (i.e: --53.33 … +-53.33 ); 19 bits. TACommonDriftVariation; 0…29470 (0…0.60 ); 15 bits. - Value ranges are given in unit of corresponding granularity.

In our understanding the rationale behind RAN2’s discussion on support of negative Common TA is mainly related to TA margin issue. This is explained in [2, R2-2406972] and recopied hereafter:

[R2-2406972]: In NTN system, UE needs to pre-compensation TA based on NTN assistance information and its own GNSS, which would result in that UE may both overestimate and underestimate its TA. And with the existing common TA related parameters mentioned above, both positive and negative values are supported for NTA,adjcommon under transparent architecture whose reference point is between satellite and gNB on the ground. However, for regenerative payload with gNB on board, the reference point is most likely to be at the satellite (as in figure 1) in actual implementation, at this moment NTA,adjcommon calculation mainly depends on TACommon (i.e. ta-Common-r17 parameter) which only support positive values and 0. Then considering TA margin, it may be needed to introduce negative values for ta-Common-r17 parameter to help adjust TA better. Therefore, for regenerative payload, we could just introduce negative values for ta-Common-r17 in TS 38.331 and up to network configuration, then additional clarification is not needed in stage 2 specification. Proposal 1: For regenerative payload, introduce negative values for ta-Common-r17 in TS 38.331 and up to network configuration, then additional clarification is not needed for both common TA and kmac in stage 2 specification.

According to [2] for regenerative payload, considering TA margin, it may be needed to introduce negative values for ta-Common-r17 parameter to help adjust TA better.

With the above in mind, we would like to recall that during Release-17 work item on NTN, there was an extensive discussion on the need of TA margin that may be indicated by the network in order to account for the TA estimation uncertainty. To avoid that the UE over pre-compensates its TA during RACH procedure the following options were discussed:

3GPP TSG RAN WG1 Meeting #106-e: Agreement: In NTN, to avoid that the UE over pre-compensates its TA during RACH procedure, down-select one option from below: - Option 1: PRACH transmission is delayed by minCP2,GP2 - Option 2: TA margin can be considered, and it is explicitly indicated to the UE - Option 3: TA margin can be considered, and it is included within the Common TA - Option 4: UE handles it via implementation

RAN1#106-bis-e made the following conclusion:

3GPP TSG RAN WG1 Meeting #106-bis-e: Conclusion: Do not define a TA margin.

Based on the above we have the following observation and proposals:

Observation: a value equal to zero for Common TA is enough to support regenerative payload.

Proposal 1: Support of negative Common TA is not needed.

Proposal 2: Consider the following in the reply LS to RAN2, from RAN4’s perspective:

- Support of negative Common TA is not needed;

- A value equal to zero for Common TA is enough to support regenerative payload.

As a matter of fact, RAN4 should take into account that Option 3 on TA margin support from RAN1#106-e was not agreed, and it was concluded not to define a TA margin as per RAN1#106-bis-e.

# Conclusion

Observation: a value equal to zero for Common TA is enough to support regenerative payload.

Proposal 1: Support of negative Common TA is not needed.

Proposal 2: Consider the following in the reply LS to RAN2, from RAN4’s perspective:

- Support of negative Common TA is not needed;

- A value equal to zero for Common TA is enough to support regenerative payload.

# References

- R1-2407590 (R2-2407623) LS on common TA in a regenerative payload scenario, October, 2024

- R2-2406972 CMCC Considerations on regenerative payload, 3GPP TSG RAN WG2 Meeting #127, August 2024

- R1-2408397 Draft reply LS on common TA in a regenerative payload scenario, October 2024