3GPP TSG-RAN WG4 Meeting #112bis	R4-2416560

# Hefei, China, 14 – 18 October 2024

Title:	LS on NR NTN and GNSS concurrency

Response to:

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG4

To:	RAN WG1, RAN WG2

Cc:	-

Contact person:	Toni Lähteensuo

tlaehtee@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 has studied the emissions and blocking effects from NR NTN transmissions to GNSS frequencies and observed that successful GNSS reception cannot always be guaranteed, the problem being worst in nearby frequency band n254 and n255. Especially, it’s very important for NR NTN UE to acquire the accurate location information during both initial access and connected mode to do the pre-compensation when UE is moving. This includes also high speed scenarios. To avoid any field performance issues both for NR NTN RedCap as well as NR NTN non-RedCap UEs, a reliable in-device co-existence solution needs to be available in real deployments.

RAN4 therefore respectfully asks RAN2 and RAN1 consider the solution(s) to ensure that NR NTN UEs can rely on an in-device co-existence solution for GNSS co-existence. As an example and not limiting the solution space, UE could need e.g. UL gaps or TDM pattern to limit Tx duty cycle for GNSS reception in the NR UL transmissions for a few seconds and then continue normal operations. The required parameters including how often UE needs these Tx gap patterns can have large differences between different UE/GNSS implementations, with some UEs being able to perform completely without such solution.

# Actions

ACTIONS:

To RAN WG2 and WG1: RAN4 respectfully asks RAN2 and RAN1 consider the potential solution(s) to ensure that NR NTN UEs can rely on an in-device co-existence solution for GNSS co-existence with sufficient gaps in NR UL transmissions considering different UE/GNSS implementations and/or scenarios, as described above.

# Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #113	Nov 18 - 22, 2024	Orlando, USA

TSG-RAN4 Meeting #114	Feb 17 - 21, 2025	Athens, Greece