3GPP TSG-RAN WG4 Meeting #112bis	R4-2416561

Hefei, China, October 14 – 18, 2024

Title: 	Way Forward for [112][310] NR_NTN_Ph3_UE_RF

Agenda Item:	6.26.5

Source: 	Qualcomm Inc.

Document for:	Approval

## Issue 1-1: PC3 output power for HD-FDD (e)RedCap

Agreement: Nominal power for PC3 is 23 dBm.

## Issue 1-2: HD-FDD refsens for 2 Rx

Agreement: Agree n256 Refsens as below.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n256 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |

## Issue 1-3: Simultaneous operation with GNSS

Agreement:

- Strive towards common solution for (e)RedCap NR NTN and non-RedCap NR NTN

- Consider asking RAN1/2 to ensure IDC framework allows appropriate configurations to ensure GNSS location acquisition and NR NTN can operate in parallel

- Encourage further evaluations in RAN4

## Issue 1-4: Specification updates

Agreement:

  - The maximum transmission bandwidth for NTN eRedCap with bandwidth reduction should be 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

  - REFSENS of NTN eRedCap with bandwidth reduction for wider channel BW in FR1-NTN bands, it should be defined as below:

    - The 5MHz REFSENS requirements are applicable for all the channel bandwidths including 5MHz, 10MHz, 15MHz, 20MHz channel bandwidth.

    - 25 contiguous RB placed in middle of channel BW both in UL and DL for both HD-FDD and FD-FDD, where both UL and DL allocations are following the 5MHz NTN RedCap REFSENS configurations and located at the center of the channel BW.

    - UL RB size follow the 5MHz UL configuration table

  - Reuse the same scaling factor of NTN (e)RedCap without bandwidth reduction for REFSENS for NTN eRedCap with bandwidth reduction with 2Rx HD-FDD, 1Rx HD-FDD and 1Rx FD-FDD.

  - The bandwidth reduction in TS 38.101-1 5.3I applies also for NTN (e)RedCap