3GPP TSG-RAN WG4 Meeting #112bis												    		R4-2416849

Hefei, China, 14th – 18th October 2024

Agenda item:			6.4.4

Source:	Intel Corporation

Title:	WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2

Document for:	Information

# Topic #1: RRM requirements

## Open issues summary for topic #1

### Sub-topic 1-1 RRM scope for SCell configured with 12PRB SSB

Issue 1-1-1a: Known SCell activation delay requirements:

- Proposals

  - Option 1a: When the measurement period of the SCell being activated is larger than 2400ms, specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth.

  - Option 1b: When UE is not configured with beam level reporting for the SCell, extend the SCell activation delay by 2 Trs.

  - Option 2: No new requirement is specified on any case for known SCell activation.

Agreement:

For known SCell activation delay requirements RAN4 specifies fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth.

Issue 1-1-1b: Unknown SCell activation delay requirements:

- Proposals

  - Option 1: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the SCell activation delay requirements in FR1 in Rel-19.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, - TFirstSSB_MAX + TSMTC_MAX + 4*Trs + 5ms, if the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

- Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + 3 * Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, and the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth, - 3ms + TFirstSSB_MAX + TSMTC_MAX + 3 * Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting, and the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

Agreement: Unknown SCell activation delay requirements:

  - Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth. FFS the CR details.

Issue 1-1-2: Measurement requirements for deactivated SCC:

- Proposals

  - Option 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

- Table 9.2.5.1-x: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Issue 1-1-3: Whether intra-band CA is in the scope:

- Proposals

  - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19.

  - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations.

  - Option 3: No need to discuss since there is no impact to RRM requirements in Rel-19.

### Sub-topic 1-2 RRM scope for PSCell configured with 12PRB SSB

Issue 1-2-1a: PSCell addition delay requirements:

- Proposals

  - Option 1: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell addition delay requirements.

### 8.9.2 PSCell Addition Delay Requirement Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR1 or FR2 no later than in slot n+Tconfig_PSCellNR slot length: where: T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell. T∆ = 3*Trs ms for either known or unknown target PSCell operating with 12 PRB SSB BW.

Agreement on PSCell addition delay requirements

  - Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth.

Issue 1-2-1b: Conditional PSCell addition delay requirements:

- Proposals

  - Option 1: RAN4 to capture the following text for Conditional PSCell activation delay for NR-DC: T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Agreement for Conditional PSCell addition delay requirement

  - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Issue 1-2-1c: Subsequent Conditional PSCell addition delay requirements:

- Proposals

  - Option 1: In subclause 8.9C.2 add: T∆ = 3*Trs ms for PSCell operating with 12 PRB SSB BW.

Agreement for Subsequent Conditional PSCell addition delay requirements:

  - T∆ = 3*Trs ms for PSCell operating with 12 PRB SSB BW.

Issue 1-2-2a: PSCell change delay requirements:

- Proposals

  - Option 1: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change delay requirements.

## 8.11 PSCell Change This clause defines requirements for the delay within which the UE shall be able to change PSCell to other cell in EN-DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC. In the case of NR-DC PSCell change in FR1, when the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target PSCell that is with 12PRB SSB bandwidth, the UE shall transmit PRACH preamble towards the target PSCell no later than the delay requirements specified in clause 8.9.2.

  - Option 2: No need to update the text.

Issue 1-2-2b: Conditional PSCell change delay requirements:

- Proposals

  - Option 1: In subclause 8.11B.2, add T∆ = 3*Trs ms for PSCell operating with 12 PRB SSB BW.

Issue 1-2-3a: Handover with PSCell:

- Proposals

  - Option 1: SSB index acquisition of less than 5MHz cell can be extended in legacy requirement.

  - Option 2: The HO requirements with PSCell-PCell and PSCell-PSCell can be reused for NR CA/DC in less than 5MHz.

Issue 1-2-3b: NR Conditional Handover including target MCG and target SCG:

- Proposals

  - Option 1: SSB index acquisition of less than 5MHz cell can be extended in legacy requirement.

  - Option 2: T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

Agreement: NR Conditional Handover including target MCG and target SCG:

T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

Issue 1-2-4: SCG activation delay requirements:

- Proposals

  - Option 1: RAN4 to define new SCG activation requirements for PSCell with 12RB PBCH, and same delay extension as for PSCell addition or PSCell change is re-used.

    - RAN4 to capture the following text for SCG activation delay: T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

Issue 1-2-5: Measurement requirements for deactivated PSCell:

- Proposals

  - Option 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell in FR1 as in the following table.

- Table 9.2.5.1-y: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

### Sub-topic 1-3 Network indication on SSB bandwidth

Issue 1-3-1a: Allowing 12PRB SSB bandwidth off sync raster:

- Proposals

  - Option 1: RAN4 allows that 12 PRB SSB can be transmitted off sync raster except for FRMCS services deployed in band n100.

Issue 1-3-1b: NW indication on PBCH BW in MO configuration:

- Background

  - RF room Agreement (online session)

    - No new signalling from network to inform UE the number of PRB transmission is required in the RF scope of this item.

  - Rapporteur comment

    - The intention of this agreement is to discuss any possible signalling design not in the RF room but in RRM room.

    - This agreement does not prevent any RRM discussions.

- Proposals

  - Option 1: Introduce network signalling in the MO to facilitate UE knowledge about the unknown target SSB bandwidth.

  - Option 2: RAN4 RRM should not introduce new network signalling on PRB transmission in alignment with RF.

Agreement:

- Unless it will be agreed in RF session that the SCell SSB block with 12 PRB Bandwidth should be only on sync raster in all the cases in the WI, otherwise it is agreed in RRM session that:

- Introduce network signalling in the MO on the target SSB bandwidth.

Issue 1-3-1c: NW indication on PBCH BW in HO command/SCell addition command/SIB4 message:

- Proposals

  - Option 1: No need to introduce new indication.

  - Option 2: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in MO and/or HO command.

Agreement: NW indication on PBCH BW in HO command/SCell addition command:

- Further check the case of Scell addition during handover during this meeting, and not introduce new network indication for the other cases.

Issue 1-3-2: LS to RAN1 and RAN2:

- Proposals

  - Option 1: RAN4 sends RAN2 LS on the agreement if any about the network indications.

  - Option 2: RAN4 to send LS to inform potential impacts to other WGs in the November meeting after the RRM requirements are finalized.

### Sub-topic 1-4 Other RRM impacts

Issue 1-4-1: EMR requirements:

- Proposals

  - Option 1: RAN4 to define EMR requirements for target cell with 12RB PBCH. Delay requirements for SSB index reading in EMR is extended by [3] samples.

  - Option 2: No changes on top of rel-18 EMR measurement reporting are needed.

Agreement:

  - RAN4 to define EMR requirements for target cell with 12RB PBCH, and the delay requirements for SSB index reading in EMR is extended by [3] more samples.

Issue 1-4-2: Requirements applicability:

- Proposals

  - Proposal 1: Even though UE capability indicates to support fast SCell activation or inter-band SSB-less SCell activation, no requirement shall be applied for fast SCell activation or inter-band SSB-less SCell activation when the target SCell BW is less than 5MHz, i.e., UE capability of fast SCell activation and inter-band SSB-less SCell activation only applies to SCell on 5MHz and larger BW.

  - Proposal 2: SCell activation requirement shall not be applied when CSI-RS is configured for L1-RSRP measurement/report for this being-activated SCell

Agreement:

SCell activation requirement specified in Rel-19 shall not be applied when CSI-RS is configured for L1-RSRP measurement/report for the target SCell.

Issue 1-4-3: Conditional handover delay requirements to PCell with 12PRB SSB:

- Proposals

  - Option 1: T∆ = 3*Trs for target PCell operating with 12 PRB SSB BW.

  - Option 2: No changes to Rel-18 requirement.