3GPP TSG-RAN WG4 Meeting #112bis														R4-2416862

Hefei, Anhui, China, 14 – 18, 2024

Agenda item:			6.24.3

Source:	Apple

Title:	WF on RRM requirements for NR_Mob_Ph4_Part1

Document for:	Approval

# Event triggered L1 measurement reporting

#### Issue 1-1: RRM scope of Event triggered L1 measurement reporting

- Agreement:

  - RAN4 to define event-triggered L1 reporting requirement for SSB based and CSI-RS based measurements.

    - To start RAN4 discussion for SSB based L1 measurement firstly, and check how to apply/extend the agreements for SSB based L1 measurement to accommodate CSI-RS based L1 measurement.

  - From RAN4 RRM requirement perspective, for FR2-1, RRM requirements for multi-Rx based LTM L1 measurement is not considered in the WI scope.

  - RRM scope of Event triggered L1 measurement reporting

    - In addition to intra-frequency measurement and inter-frequency measurement without gap, support gap-based inter-frequency measurement for SSB-based L1 measurement. Further discuss for CSI-RS based L1 measurement.

#### Issue 1-2: supported frequency range in RRM requirements

- Agreement:

  - Both FR1 and FR2-1 are in scope of RRM requirements for Event triggered L1 measurement reporting.

#### Issue 1-3: Requirements for reporting criteria per measurement category

- Candidate solutions:

  - Option 1: (MTK)

    - Define a separate capability for supported number of events triggered L1 measurement reporting.

    - Focus on SSB based L1-RSRP measurement for event triggered reporting at first.

    - For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options:

      - Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers.

      - Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling.

  - Option 2: discuss and confirm in RAN4#112bis that whether requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell. If needed, it is proposed to use the following table as baseline. (Apple)

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5 | 10 | Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5 | 1 | Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5 | 1 | Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| DL RSTD Note 2,4,5 | 1 | DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34]. |
| UE Rx-Tx Note 2,4,5 | 1 | UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34]. |
| DL PRS-RSRP Note 2,4,5 | 1 | DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| DL PRS-RSRPP Note 2,4,5 | 1 | DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| SRS-RSRP Note 2,3,4,5 | 1 | SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16. |
| CLI-RSSI Note 2,3,4,5 | 1 | CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16. |
| NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode. |  |  |

  - Option 3: RAN4 to introduce requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” (OPPO)

#### Issue 1-4: How to coordinate with R19 MIMO

- Candidate solutions:

  - Option 1: (MTK)

    - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

    - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

#### Issue 1-5: filtering assumption in event evaluation

- Candidate solutions:

  - Option 1: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report (MTK, Apple, HW, Nokia, E///, Qualcomm, CATT).

  - Option 1a: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT)

    - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

    - Otherwise, there is no impact on RAN4.

  - Option 2: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements. (CTC)

    - Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected.

    - In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline.

  - Option 3: no need to define additional SSB based L1 measurement delay requirements with filtering, since the agreement about filtering in RAN2 is about filtering with filter coefficient as defined in Clause 5.5.3.2, TS38.331 or about Time-to-Trigger (TTT), neither of them have impact on L1 measurement delay requirements. (CMCC)

#### Issue 1-6: accuracy

- Agreement:

  - for SSB based L1 RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R18 LTM L1-RSRP accuracy requirements

  - for CSI-RS based L1-RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R19 CSI-RS based L1-RSRP accuracy requirements for LTM, which will be discussed under CSI-RS based L1-RSRP measurement agenda.

#### Issue 1-7: beam level or cell level measurement result for new LTM event evaluation

- Candidate solutions:

  - Option 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation. (Xiaomi, CMCC, Apple, Samsung)

#### Issue 1-8: starting point in event triggered L1 reporting requirements

- Agreement:

  - Use the following as starting point. Wording can be discussed in CR.

    - Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting.

#### Issue 1-9: L1 event triggered reporting delay

- Candidate solutions:

  - Option 1: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point. (CTC, Samsung, ZTE)

  - Option 2: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period. (Xiaomi)

    - It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS.

  - Option 3: (CMCC)

    - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements.

      - The delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM.

    - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements, and L1 measurement reporting accuracy requirements

  - Option 4: Reporting delay is defined as one L1-RSRP measurement period on target cell. (Apple)

  - Option 4a: The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS. (Samsung)

  - Option 5: SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (Nokia)

  - Option 6: RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (Ericsson)

#### Issue 1-10: where to capture event triggered L1 report requirement

- Candidate solutions:

  - Option 1: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. (CATT, Apple, vivo, HW)

#### Issue 1-11: Impact of UL resource waiting time

- Candidate solutions:

  - Option 1: RAN4 to discuss whether to consider the impact of UL resource waiting time on L1 reporting requirement. (Xiaomi)

#### Issue 1-12: Impact on other LTM requirements

- Candidate solutions:

  - Option 1: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements such as: (Nokia)

    - Known TCI state condition for early TCI state activation and cell switch

    - Applicability of cell switch delay requirements

  - Option 1a: RAN4 to revisit following requirements to accommodate event triggered LTM reporting (E///)

    - Definition of TCI state known condition

    - TCI state activation requirements

  - Option 2: RAN4 to consider the impact of L1 event triggered reporting to: (Nokia)

    - early DL sync (Tfirst-RS = 0 conditions)

    - early ASN.1 decoding and validity check

    - LTM L1 measurement requirements

#### Issue 1-13: others

- Candidate solutions:

  - Proposal 1: For the topic of ‘event triggered L1 measurement reporting,’ RAN4 to wait for further progress to be made in other working groups. (QC)

  - Proposal 2: For event triggered L1 reporting in FR2-1, regarding the neighbour cell which is prioritized to be measured, RAN4 re-use R18 solutions, i.e. if UE supports early TCI activation, UE prioritize the neighbour cell with pre-TCI activation in L1 measurement reporting. (vivo)

# CSI-RS based L1 measurement

#### Issue 2-1: CSI-RS based L1 measurement for known/unknown cell

- Agreement:

  - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells

#### Issue 2-2: known cell definition in CSI-RS based L1 measurement

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

- Agreement:

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

    - Further check whether to consider SSB-based L3 measurement only

  - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2.

    - L1 measurement in the 2nd bullet is not needed for FR1.

    - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet.

  - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable].

    - Further discuss whether to use detectable or measurable.

    - Further discuss the side condition

#### Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap

- Candidate solutions

  - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia)

  - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi)

  - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia)

  - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT)

    - Without gap

    - With gap

  - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency  measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC)

  - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT)

  - Option 2c: CSI-RS based L1 measurements for candidate cells shall support

    - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE)

#### Issue 2-4: whether SSB based L3 measurement is a pre-requisite of CSI-RS based L1 measurement

Tentative Agreement during online session: further discuss the two alternatives

Alternative 1:

    - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

        - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions.

        - Note: Step 2 is removed for FR1.

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

        - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Alternative 2:

    - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

#### Issue 2-5: supported frequency range

- Agreements:

  - Both FR1 and FR2-1 are in scope of RRM requirements for CSI-RS based L1-RSRP measurement on LTM candidate cell(s).

#### Issue 2-6: other applicability

- Further discuss the following assumption:

  - RAN4 requirements apply all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type.

  - FFS on other parameters, such as same center frequency, BW, periodicity and etc.

#### Issue 2-7: measurement gap related

- Candidate solutions

  - Proposal 1: In Rel-19 LTM, RAN4 needs to discuss whether to reuse or introduce the new gap for CSI-RS based L1 measurement. (CATT)

  - Proposal 2: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., RAN4 to define requirements applicability for CSI-RS based L1 measurement. (CATT)

    - Motivation is to make sure “existing gap can cover the CSI-RS configuration in the same frequency layer”

  - Proposal 3: For CSI-RS based L1 with gap measurement, at least the type 1 MG needed to be supported in Rel-19 LTM. (CATT)

    - RAN4 to discuss whether to support other types of gap and wait for more RAN1 conclusions.

  - Proposal 4:For the decision on MG to use, RAN4 to wait for further RAN1 progress. (E///)

  - Proposal 5: defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement (ZTE).

#### Issue 2-8: L1 measurement on CSI-RS with repetition ON

- Candidate solutions

  - Option 1: Not to support CSI-RS resources with repetition ON in FR2 for L1 measurement on neighbor cell. (MTK)

  - Option 1a: RAN4 to not consider P3-like early beam management in CSI-RS based L1-RSRP measurement requirement definition, i.e. no Rx beam sweeping factor, assuming the CSI-RS is not configured with ‘repetition = on.’ (QC)

    - P1 (wide gNB Tx & wide UE Rx): SSB based BM (implicit/explicit SSB-ID report to gNB)

    - P2 (narrow gNB Tx & wide UE Rx): On CSI-RS resources with ‘repetition = off’, UE applies a fixed Rx beam obtained from P1 and measures/reports CRI and L1-RSRP.

    - P3(narrow gNB Tx & narrow UE Rx): On CSI-RS resources with ‘repetition = on’, UE sweeps its Rx beam with a finer beamwidth, desirably confined within the P1/P2 Rx beamwidth. No report is required.

  - Option 2: If the repetition is ON, UE can receive the configured CSI-RS resources and perform CSI-RS measurement with Rx beam sweeping directly. (CTC, CATT)

#### Issue 2-9: L1 measurement on CSI-RS with repetition OFF

- Candidate solutions

  - Option 1: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. (MTK)

  - Option 1a: RAN4 to define the L1 measurement requirements for CSI-RS with repetition OFF in FR2 for the following procedure (CATT)

    - UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE perform measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement

      - UE is capable of supporting R18 LTM.

  - Option 2: If the repetition is OFF, UE needs to perform measurement on the associated SSB firstly, but we do not capture the significant benefit of second situation. (CTC)

#### Issue 2-10: CSI-RS resource type

- Candidate solutions:

  - Option 1: RAN4 prioritize CSI-RS based L1 measurement requirements for periodic CSI-RS. (vivo)

#### Issue 2-11: CSI-RS resource density and BW

- Agreements:

  - For CSI-RS based L1-RSRP measurement on neighbor cell, define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs.

#### Issue 2-12: RTD assumption

- Further discuss based on the following assumption:

  - RTD is the received time difference among CSI-RS and SSB from different cells.

    - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP.

    - Alt 2: only define requirements for RTD<CP.

      - NW shall only enable this when RTD<CP is guaranteed.

#### Issue 2-13: category of CSI-RS based L1 RSRP measurement (already discussed during online session)

- Candidate solutions:

  - Option 1: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion (MTK, CATT, E///, vivo)

  - Option 2: Categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency (CATT, CTC, CMCC, Apple, Samsung, HW, OPPO, Nokia, ZTE)

#### Issue 2-14: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement (already discussed during online session)

- Candidate solutions:

  - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?])

    - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

      - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

      - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

        - It is applied for SCS = 60KHz

      - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

  - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs. (Xiaomi)

  - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT)

    - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell.

  - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo)

    - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules

    - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements.

  - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (HW)

    - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

#### Issue 2-15: definition of frequency layer in CSI-RS based L1-RSRP measurement

- Candidate solutions:

  - Option 1: Following CSI-RS based L3 measurement, for CSI-RS resources of the same L1 frequency layer, SCS, CP type, center frequency, BW and periodicity are the same. (MTK)

  - Option 2: Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and has overlapping periodicity. (E///)

#### Issue 2-16: number of candidate cells and beams UE is expected to measure

- Candidate solutions:

  - Option 1: Discuss the number of candidate cells and beams UE is expected to measure for intra-frequency L1-RSRP CSI-RS measurements. (Nokia)

#### Issue 2-17: CSI acquisition on candidate cell(s) before or during LTM cell switch

- Candidate solutions:

  - Option 1: Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4. (MTK, QC)

#### Issue 2-18: accuracy and side condition for CSI-RS based L1-RSRP measurement

- Candidate solutions:

  - Option 1: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point. (Apple)

  - Option 1a: CSI-RS based L1-RSRP accuracy of the serving cell can be applied for CSI-RS based L1-RSRP of the candidate neighbour cells. (HW)

  - Option 2: RAN4 to discuss the CSI-RS accuracy requirements during performance part. (E///)

#### Issue 2-19: CSI-RS based L1-SINR measurement

- Candidate solutions:

  - Option 1: RAN4 need to discuss whether to define the requirements for CSI-RS based L1-SINR measurement. (CATT)

    - RAN4 to wait for more RAN1 input.

  - Option 2: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement (Apple, Nokia)

#### Issue 2-20: other CSI-RS based L1 measurement issues

- Candidate solutions:

  - Option 1; Define early DL sync requirements for using CSI-RS once RAN1 approval, and SSB based DL sync in LTM can be used as baseline. (ZTE)

# Others

#### Issue 3-1: other issues

- Candidate solutions:

  - Proposal 1: it is proposed to define dedicated requirements, e.g.timing requirements, for UE based TA adjustment. (CMCC)

  - Proposal 2: For subsequent LTM, cell switch delay requirements for R18 LTM in clause 6.3 could be reused. (CMCC, ZTE)

  - Proposal 3: RAN4 to discuss whether to define requirements for subsequent LTM cell switching delay in R19. The requirements would be the same for both intra-CU and inter-CU. (HW)

- Agreements:

  - It is RAN4 common understanding that R18 LTM cell switch requirements also apply for R18 subsequent LTM.