3GPP TSG-RAN WG4 Meeting #112bis	R4-2416916

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.22.3

Source:	Ericsson

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part1

Document for:	Approval

# Introduction

Note: The detail open issue list is captured in the moderator summary R4-2415662.

# Topic #1: On-demand SSB(OD-SSB) requirements

### Sub-topic 1-1: General OD-SSB requirements

Issue 1-1-2: OD-SSB impact on neighbour cell measurement

Agreement:

- For the RRM measurement on neigboring cell, the legacy meaurement framework is applied.

  - Note: The discussion of SMTC configuration is a separate issue.

- This agreement can be revisited if it is not aligned with further agreement in other WGs.

Issue 1-1-3: OD-SSB type

Agreement:

- RAN4 to first take NCD-SSB as a start point for OD-SSB requirements.

### Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 1)

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1)

Agreements:

Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

- After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 2-1: fallback legacy requirement

  - Option 2-2: stop measurement

  - Option 2-3: No requirement

- Time window means the time duration starting from receiving OD-SSB activation indication

  - FFS the time window configuration or pre-defined

- FFS the fast L3 measurement requirement

- FFS the relation between time window and OD-SSB deactivation command in the time domain

When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

### Sub-topic 1-3: OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation

- Background

From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements.

  - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different frequencies.

Agreement:

- In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement.

  - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies.

- Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS.

- RAN4 will inform RAN1 if any further agreement.

Issue 1-3-2: Deactivated SCell measurement before OD-SSB transmission activation

Agreement:

- The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) before UE receiving OD-SSB transmission indication.

Issue 1-3-3: Deactivated SCell measurement after OD-SSB transmission deactivation

Agreement:

- The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) after the OD-SSB transmission deactivation.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreements:

OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

  - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE)

  - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO)

  - Option 3:  RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE)

  - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm)

  - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

Sub-topic 1-4: OD-SSB based SCell activation

Issue 1-4-1: General

Agreement:

- RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios.

Issue 1-4-3: known cell condition design – principle

Agreement:

- The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command.

- When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met.

  - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB

- Other principles are not precluded.

Issue 1-4-5: Which type of SSB is used during SCell activation

Agreement:

  - In Case 1, UE is required to always use OD-SSB for Scell activation.

  - In Case 2, when both OD-SSB and always-on SSB are transmitted,

    - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline

    - The agreement can be revisited if different with other WG