3GPP TSG-RAN4 Meeting #112bis					R4-2417057

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	China Telecom

Title:	WF on notes for downlink interruption clarification

Agenda item:			5.1

Document for:	Approval

Background

This WF captures the agreements for the note of table in the TS 38101-1 that indicates no DL interruption requirement for Tx switching across up to 4 bands.

Wayforward

Issue 1: Note for DL interruption clarification for dynamic switching

Agreement: Considering the following changes for the notes in the tables in TS 38101-1 to mandate no DL interruption applicability for Tx switching across up to 4 bands

Table 5.2A.2.2-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_nX-nY-nZ | nX, nY, nZ | No for CA_nX-nY/No for nX-nY |
| NOTE 4: Applicable when dynamic Tx switching is conducted scheduled among 2 or 3 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. The CA_nX-nY/nX-nY in the requirement of “No for CA_nX-nY/No for nX-nY” refers to the two UL Bands nX and nY that are involved in dynamic Tx switching. |  |  |

Table 5.2A.2.3-1: Inter-band CA operating bands involving FR1 (four bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 2) |
| --- | --- | --- |
| CA_nX-nY-nZ-nK | nX, nY, nZ, nK | No for CA_nX-nY/No for nX-nY |
| NOTE 2: Applicable when dynamic Tx switching is scheduled among 2, 3 or 4 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. The CA_nX-nY/nX-nY in the requirement of “No for CA_nX-nY/No for nX-nY” refers to the two UL Bands nX and nY that are involved in dynamic Tx switching. |  |  |

Way Forward: Evalute and consider whether to change ’No for CA_nX-nY’ to be ’No for nX-nY’ for DL interruption clarification in next meeting for release-19 and earlier.