3GPP TSG-WG4 Meeting #112bis	 R4-2417058

# Hefei, China, October 14 – 18, 2024

Title: 	WF on simultaneous Rx-Tx for CA_n40A-n41A

Agenda Item:	5.1

Source: 	Huawei

Document for:	Approval

Background:

In Rel-18, the cross band isolation defined for simultaneous Rx-Tx of CA_n40A-n41A was based on 4Rx RF front-end architecture as illustrated in Figure 1 Murata’s paper in RAN4#112. The MSD defined for CA_n40-n41 is large due to the close frequency separation between UL & DL and the marginal filter isolation. The current cross band isolation test point is not reflecting operator deployment scenario. It is asked to create an additional test point which provides MSD with respect to realistic deployment scenario.

Table 1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for CA_n40-n41

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.1 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | ACLR2 |

Table 2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for CA_n40-n41

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 31.1 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 34.4 | ACLR2 |

Figure 1. RF front end architecture for CA_n40-n41 using 4 antennas in n41 with simultaneous Rx-Tx operation

In RAN4#112, the WF [1] was approved with candidate ways to handle the simultaneous Rx-Tx for CA_n40A-n41A in Rel-19. One of the options listed in the WF is to analyse the UE architecture with n41 using 2Rx in simultaneous Rx-Tx, as illustrated in [2]. [2] shows that MSD is reduced by only 1.4dB with the relaxed 2Rx architecture and the same test point. Therefore, the cross band isolation value defined for CA_n40-n41 can roughly address the relaxed architecture of 2Rx.

Figure 2. UE RFFE architecture for relaxed 2Rx MSD calculation of CA_n40-n41 (n40 UL, n41 DL)

<Way forward>

- Companies are asked to verify whether the same MSD value defined for CA_n40-n41 with 4Rx can be applied to 2Rx RF front end architecture for CA_n40-n41.

- Provide further information on the need for new test point for CA_n40-n41, taking into consideration the applicable bandwidth and frequency range of n40 and n41 from operator(s). For example, wider frequency separation between n40 and n41, and wider DL bandwidth.

- Companies are asked to propose suitable test point reflecting operator demand in next meeting

- 2Rx RF front end architecture can be considered.

- For n41, consider if a new signalling is necessary to support maximum of 2-layer MIMO with 2Rx antenna ports, if the UE reports both the capability of 4-layer MIMO for band n41 and simultaneous Rx-Tx for CA_n40A-n41A.

Reference

[1] R4-2412606, WF on simultaneous Rx-Tx for CA_n40-n41

[2] R4-2415359, Further discussion on simultaneous Rx-Tx of CA_n40-n41, Murata