3GPP TSG-RAN WG4 Meeting #112bis	R4-2417082

# Hefei, China, 14 – 18 October 2024

Agenda item:			5.1

Source:	CMCC

Title:	WF on PC2 and 40MHz CBW for n28

Document for:	Approval

Introduction

This way forward captures the agreements for [112bis][112] NR_n28_PC2_40MHz. The summary in RAN4#112bis is R4-2415216.

Topic #1: UE RF requirements

Sub-topic 1-1 General issue

Issue 1-1-1: Single set or two sets requirements

Agreement:

- Single set of A-MPR and RSD requirements for PC2 n28 independent of implemented filter implementation.

Sub-topic 1-2 RF requirements

Issue 1-2-1  NS_17 A-MPR for PC2 for BW<=30MHz

Agreement:

- Agree on regions in table 1 and A-MPR values in table 2

- Table 1: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | >= 5.4 | A2 |

Table 2: A-MPR for NS_17 for PC2 for 1Tx and 2Tx

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3 | ≤ 3.5 |
|  | QPSK | ≤ 3 | ≤ 4 |
|  | 16 QAM | ≤ 3.5 | ≤ 5 |
|  | 64 QAM | ≤ 4 | ≤ 5 |
|  | 256 QAM |  | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤6 |
|  | 16 QAM | ≤ 5 | ≤6 |
|  | 64 QAM | ≤ 5 | ≤6 |
|  | 256 QAM |  |  |

Issue 1-2-2 NS_18 A-MPR for PC2 for BW<=40MHz

Agreement:

- Agree on the regions in Table 1 and 2.

- For A-MPR values in Table 3:

- Further check the A-MPR values for 2Tx

- Yellow highlighted values will be further confirmed by measurements.

- Values in [] are to be confirmed

Table 1: PC3 region definition for 25MHz, 30MHz and 40MHz channel bandwidth

| Channel Bandwidth, MHz | Frequency range of channel bandwidth, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | [703~748] | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
| 30 | [703~748] | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
| 40 | 703~743.04 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |

Table 2: PC2 region definition for 25MHz, 30MHz and 40MHz channel bandwidth

| Channel Bandwidth, MHz | Frequency range of channel bandwidth, MHz | Regions (PC2) |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | [703~748] | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | [703~748] | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 40 | 703~743.04 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+8.46 ≤(LCRB*12*SCS)/2+11.7 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

Table 3: PC2 A-MPR for NS_18 for [1Tx/2Tx]

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 8 | [3+0.5] | [8+2] | [3+2] | [0+2.0] |
|  | QPSK | ≤ 3.5 |  | ≤ 8 | [3+1] | [8+2] | [3+2] | [0+3] |
|  | 16 QAM | ≤ 4 |  | ≤ 9 | [3+1] | [8+3] | [3+2.5] | [1.0+3] |
|  | 64 QAM | ≤ 4.5 |  | ≤ 10 | [3+1] | [8+3] | [4.5+1.5] | [2.5+2] |
|  | 256 QAM | ≤ 6.5 |  | ≤ 11 | [3+1] | [8+3] | [5.5+0.5] | [4.5] |
| CP-OFDM | QPSK | ≤ 5.5 |  | ≤ 9.5 | [4.5+1] | [9.5+2.5] | [5+2] | [1.5+3] |
|  | 16 QAM | ≤ 5.5 |  | ≤ 10 | [4.5+1] | [9.5+2.5] | [5+2] | [2.0+3] |
|  | 64 QAM | ≤ 6 |  | ≤ 11.5 | [4.5+1] | [9.5+3] | [5.5+1.5] | [3.5+1.5] |
|  | 256 QAM | ≤ 9 |  | ≤ 11.5 | [4.5+1] | [9.5+3] | [7.5+0.5] | [6.5] |

Issue 1-2-3 n28 REFSENS for PC3

Agreement:

- Take as average value -66.3dBm as PC3 REFSENS

Issue 1-2-4 PC2 RSD for 1Tx and 2Tx for 40MHz

Agreement:

- PC2 RSD for 1Tx is 3 dB, for 2Tx is 8.2 dB

Issue 1-2-5 ∆MPR for 40MHz

Agreement:

- 0.5 dB for 40 MHz channel bandwidth both for PC3 and PC2.

Issue 1-2-6 channel location

Agreement:

- Update the NOTE 7 as follows:

  - NOTE 7:	For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] 703-733MHz or 718-748MHz [for the 20, 25 and 30MHz bandwidth].

- Introduce the new note as follows:

  - For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] [703-743.04MHz] or 718-748MHz [for the [20, 25, and] 30MHz bandwidth. And for the 40MHz bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz.]

- FFS the content in []

Issue 1-2-7 channel raster

Agreement:

- Alt1: Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

- Alt2: Cover all the channel raster points of n28 based on the enhanced channel raster to UE RF specification TS 38.101-1 for UE CBW 40MHz.

Issue 1-2-8 Release independence

Agreement:

- Option 1: Rel-16

- Option 2: Rel-15