3GPP TSG-RAN WG4 Meeting # 112bis														R4-2417088

# Hefei, China,14 – 18 Oct., 2024

Agenda item:			6.21.4

Source:	Moderator (Huawei)

Title:	WF on impacts of A-IoT on RF requirements

Document for:	Approval

Introduction

This document captures the agreements about the impacts of Ambient IoT on RF requirements for R19 SI（RP-240826）.The topic summaries are R4-2408946 (RAN4#111), R4-2412834(RAN4#112) and R4-2412834 (RAN4#112bis). The WF are R4-2410597(RAN4#111)，R4-2414305 (RAN4#112).

Topic #1: A-IoT System Parameters

- Issue 1-1: System parameter

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| System parameter for A-IoT |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT divice | Intermediate UE |
| Operating band |  | YES |  |  |
| Channel bandwidth | Transmission bandwidth | YES the value needs be discussed in WI phase depending on other WG conclusion. |  |  |
|  | channel bandwidth | YES，as RAN4#112 agreed |  |  |
| Channel arrangement | Channel spacing | TBD for standalone NA for in-band/Guard-band |  |  |
|  | Channel raster | YES | TBD | TBD |
|  | Synchronization raster | NO for device 1/2a，as RAN4#112 agreed |  |  |

Note:

- YES means this requirement is potentially necessary, relevant factors and considerations will be explained in the TR.

- NO means this requirement may not be necessary, and if necessary, reasons can be provided in the TR.

- TBD means this requirement will be discussed during the RAN4#113 meeting.

TBD means this requirement will be discussed during the RAN4#113 meeting. Several options can be presented in the TR.Topic #2: A-IoT BS

- Issue 2-2: A-IoT BS class and BS type

Agreement:

- Define BS type 1-C for Ambient-IoT indoor scenarios

    - FFS on CW signaling transmission capability for BS

- Issue 2-4: TX

Agreement:

- Whether requirements for A-IoT device RX RF potentially needed or not is summarized in the following table:

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES, as RAN4#112 agreed |
| Output power dynamic | RE power control dynamic range | TBD |
|  | Total power dynamic range | TBD |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | TBD |
|  | Transmitter transient period | TBD |
| Transmit signal quality | Frequency error | YES, as RAN4#112 agreed |
|  | Some requirements like EVM (Modulation quality) | YES, as RAN4#112 agreed |
|  | TAE | NA, as RAN4#112 agreed |
| Unwanted emissions | Occupied bandwidth | YES, as RAN4#112 agreed |
|  | ACLR | YES, as RAN4#112 agreed |
|  | IBE | NO |
|  | Operating band unwanted emissions | YES, as RAN4#112 agreed |
|  | Transmitter spurious emissions | YES, as RAN4#112 agreed |
| Transmitter intermodulation |  | TBD |

- Issue 2-5: RX

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | YES, as RAN4#112 agreed |
| Dynamic range |  | Discussed in the WI phase |
| Adjacent Channel Selectivity |  | YES, as RAN4#112 agreed |
| In-band blocking | general blocking | YES |
|  | narrowband blocking | YES |
| Out-of-band blocking |  | YES |
| Receiver spurious emission |  | YES, as RAN4#112 agreed |
| Receiver intermodulation | General intermodulation | Wait for RAN1 conclusion. |
|  | Narrowband intermodulation | YES |
| In-channel selectivity |  | TBD |

- Issue 2-6: CW for D1T1

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| RF Requirement for CW for D1T1 | Requirements potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD |
| Frequency error | YES |
| Phase noise | NO |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD |

Topic #3: AIoT device

- Issue 3-3: TX(D2R)

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | Device 1 | Device 2a | Device 2b |
| Transmit output power | Maximum output power /output power | NO for maximum output power YES for backscattering power or power backscattering loss | NO for maximum output power YES for backscattering power or power backscattering loss | YES |
| Output power dynamic | Transmit OFF power | No | NO | YES, as RAN4#112 agreed |
|  | Transmit ON/OFF time mask | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed |
|  | Minimum output power | N/A | N/A | YES |
|  | Power control | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | YES, as RAN4#112 agreed |
| Transmit signal quality | Frequency error | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
|  | Modulation quality requirements | TBD | TBD | TBD |
|  | In band emissions | NO, as RAN4#112 agreed | TBD | TBD. |
|  | Carrier leakage | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
| Output RF spectrum emissions | Occupied bandwidth | YES | YES | YES |
|  | SEM | TBD | TBD | TBD |
|  | ACLR | TBD | TBD | TBD |
|  | Spurious emissions | TBD | TBD | TBD |
| Unwanted emissions |  | TBD | TBD | TBD |
| Transmit intermodulation |  | NO | NO | TBD |

- Issue 3-4: RX(R2D)

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT device- RX Requirement | Potentially needed or not |  |  |
| --- | --- | --- | --- |
|  | Device 1 | Device 2a | Device 2b |
| Reference sensitivity | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed |
| Maximum input power | TBD | TBD | TBD |
| ACS | NO | NO | Depend on co-existence study |
| ACSC | NO | NO | NO |
| In-band blocking | NO | NO | TBD |
| Out-of-band blocking | NO | NO | TBD |
| Receiver intermodulation | NO | NO | TBD |
| Rx spurious emission | NO | NO | TBD |
| Spurious response | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed |
| Interference rejection | TBD | TBD | TBD |

^Issue 3-5: testability

- Take OTA test as the baseline for at least Devices 1 and 2a

  - FFS on the side conditions in the OTA test setup including CW signal and RF energy source, etc.

Topic #4: Intermediate node （UE）

- Issue 4-2: TX

Agreement:

- Whether all the requirements defined need to be tested for intermediate UE can be further discussed in WI phase.

- Whether requirements potentially needed or not is summarized in the following table:

| RF Requirement for A-IoT intermediate UE- TX part |  |  | Potentially needed or not |
| --- | --- | --- | --- |
| Maximum output power |  |  | YES, as RAN4#112 agreed |
| Output power dynamics | Minimum output power |  | NO |
|  | Transmit OFF power |  | TBD |
|  | Transmit ON/OFF time mask |  | TBD |
|  | Power control |  | NO |
| Transmitted signal quality | Frequency error |  | YES, as RAN4#112 agreed |
|  | Transmit modulation quality | EVM | YES |
|  |  | Carrier leakage | TBD |
|  |  | In-band emissions | TBD |
| Output RF spectrum emissions | Occupied bandwidth |  | YES, as RAN4#112 agreed |
|  | ACLR |  | Depends on the co-existence, as RAN4#112 agreed |
|  | Spurious emissions |  | YES, as RAN4#112 agreed |
|  | Transmit intermodulation |  | TBD |

- Issue 4-3: RX

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT intermediate UE- RX RF Requirement | Potentially needed or not |
| --- | --- |
| Reference sensitivity | YES, as RAN4#112 agreed |
| Maximum input power | YES, as RAN4#112 agreed |
| ACS | YES, Depends co-existence study |
| In-band blocking | YES, Depends co-existence study |
| Out-of-band blocking | YES |
| Narrow band blocking | TBD |
| Spurious response | TBD |
| Receiver intermodulation | YES, as RAN4#112 agreed |
| RX spurious emissions | YES |

- Issue 4-4: CW for D2T2

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| RF Requirement for CW node | Potentially needed or not |
| --- | --- |
| Output power | YES, as RAN4#112 agreed |
| Output power dynamic range | TBD |
| Transmit signal quality | TBD |
| RF spectrum emission | YES, as RAN4#112 agreed |
| Spurious emissions | TBD |
| Phase noise | NO |

Topic #6: draft TP

Agreement:

- TP  work split among companies are as follows:

| No. | sections in TR38769 (Based on the TR 38.769 V1.0.0) | Volunteer company |
| --- | --- | --- |
| 1 | 6.6 Coexistence of Ambient IoT and NR/LTE | Huawei, Qualcomm |
| 2 | 6.7.1 System parameters | Ericsson |
| 3 | 6.7.2.1 Ambient IoT BS -TX | CATT |
| 4 | 6.7.2.2 Ambient IoT BS -RX | ZTE, Ericsson |
| 5 | 6.7.3.1 Intermediate node (UE) -TX | VIVO |
| 6 | 6.7.3.2 Intermediate node (UE) -RX | LGE, OPPO |
| 7 | 6.7.4.1 Device 1 | Spreadtrum |
| 8 | 6.7.4.2 Device 2a | Xiaomi |
| 9 | 6.7.4.3 Device 2b | OPPO |
| 10 | 6.7.4.4 Testability | Qualcomm |
| 11 | 6.7.5 Feasibility study | Huawei |
| 12 | 6.8 Conclusions and recommendations | CMCC |