3GPP TSG-RAN WG4 Meeting #112bis				                 							R4-2417089

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.3.4

Source:	OPPO

Title:	WF on sidelink intra-band CA part 1

Document for:	Approval

Introduction

This WF captures most the agreements for NR SL intra-band non-contiguous CA.

WF agreements

Simulation configuration

Below WF is agreed:

- CBW configuration:

  - Gap = max{CBW1, CBW2} is the typical case.

- Cases alignment:

  - Below case in table 1 when the LO and image fall inside the gap region, the ACLR is hard to meet.

    - Larger LO and IQ image suppression might be needed.

Table 1 Problematic Configurations of NCCA

| case | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | CBW1 | CBW2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 10 | 41 | 10 | 0 | 15 | 10 | 10 |
| 23 | 10 | 41 | 20 | 2 | 15 | 10 | 20 |
| 39 | 10 | 13 | 10 | 0 | 30 | 10 | 20 |
| 43 | 12 | 11 | 25 | 0 | 30 | 10 | 20 |

PC3/PC2 PSSCH non-contiguou CA MPR

WF: Companies are encouraged to provide further simulation results considering the in-gap ACLR with the agreement of issue 1.1.1

PSFCH MPR

WF: Encourage comapnies to provide simulation results.

S-SSB MPR

WF: Encourage comapnies to provide simulation results.

LO and IQ suppression

Background:

- Current LO and IQ assumption: 34dB IQ image; 28dB LO suppression.

- NR intra-band NCCA assumption as below table 1:

Table 1 NR intra-band NCCA assumption for -13dBm/MHz

| In-gap emission limit | LO suppression requirement | IQ image suppression requirement |
| --- | --- | --- |
| SEM -13 dBm/MHz | 37 dB | 32 dB |

WF:

Together with issue 1.1.1, Use larger LO suppression IQ image as optional assumption to further check the simulation result.

Configured output power

WF: It is proposed companies to provide consolidate proposals next meeting.

TP to TR

WF: TP can be postponed to further check the NCCA results.

A-MPR

WF: Encourage companies to further study the A-MPR regulation requirements