3GPP TSG-RAN WG4 Meeting #112bis	R4-2417108

Hefei, Anhui, China, 14th – 18th October 2024

Agenda Item:	10.3.6

Source:	Qualcomm Incorporated

Title:	WF on 15GHz UE parameters

Document for:	Approval

# 1	Introduction

This WF captures the agreement for IMT UE matters for [112bis][117] FS_NR_IMT - UE types offline discussion. The moderator summary can be found in R4-xxxxx.

# 2	Agreements

UE-type discussion

- LS

  - Only one UE antenna model to be captured.

  - 1Tx omnidirectional antenna UE, understood as the worst case (highest ACIR values) assumptions for sharing studies and not on deployment, coverage, implementation, or requirement aspects.

- TR

  - Additional UE antenna models could be captured.

UE output power

- 23dBm, mention other power classes possible in TR

Noise figure

- For LS to ITU, noise figure is [13] dB

- The common understanding is that the current co-existence simulation work (based on 11 dB UE NF) is not changed due to above noise figure value.

Power dynamic range

- Proposals

  - Option 1: 56dB for 100MHz assuming 23dBm

  - Option 2: 56dB for 200MHz assuming 23dBm

Emissions mask

- Option 1: UE SEM given in Section 7.1.2, TR 38.921.

- Option 2: Other options.

ACLR

- Decide based on co-existence study

Spurious emission

- FFS if RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1

Sensitivity

- Capture it as “To be specified” in the LS reply

Blocking and spurious response

- FFS if RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

ACS

- Decide based on co-existence study