3GPP TSG-RAN WG4 Meeting #112-bis	R4-2417111

Hefei, China, 14th -18th Oct, 2024

Title: 	WF on power domain enhancements

Agenda Item:	6.1.3

Source: 	Huawei, HiSilicon

Document for:	Approval

Topic #1: Power domain enhancements for single carrier

Issue 1-1-1: Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios)

- WF

  - Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation.

    - It does not preclude separate solutions for each applicable scenario

    - No relaxation of ACLR/SEM/SE values

Issue 1-2-1: Clarification of multi-CC scenario

- WF

  - The approach of converting outer RB allocation to inner RB allocation for MPR reduction should consider multi-carrier spectrum scenario from NW perspective while the UE is configured for single CC operation.

Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation

- WF

  - The approach should consider the following aspects

    - unified solution could be applicable for both scenarios 1 and scenario 2

    - unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction

    - Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW

    - Solution could include both symmetric and asymmetric CBW extension

    - Whether the extended UE CBW could exceed the BS CBW for different scenarios

    - Signalling overhead is considered for the proposed approach(s)

Issue 1-2-3: Where to use IBE in the larger BS CBW

- WF

  - IBE should be used between edges of UE CBW and extended UE CBW

    - FFS whether IBE could be used in the guard band of UE CBW

    - FFS the full RB allocation scenario

    - FFS the impact to testability

Issue 1-2-4: Boundary to apply ACLR and SEM

- Proposals

  - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

  - Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth.

- WF

  - FFS in next meeting

    - ITU regulation of 250% necessary bandwidth should be considered in further analysis

Issue 1-2-5: Boundary to apply SE

- Proposals

  - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW.

  - Proposal 2: SE is applied at BS channel bandwidth.

- WF

  - FFS in next meeting

    - ITU regulation of 250% necessary bandwidth should be considered in further analysis

Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary

- Proposals

  - Proposal 1: The OOBE requirement and applicable boundary should be based on UE CBW.

  - Proposal 2: The OOBE requirement and applicable boundary is based on BS CBW.

- WF

  - FFS in next meeting

Issue 1-2-7: Ratio of extended CBW to UE CBW and to larger BS channel BW

- WF

  - To discuss the following aspects for the extended UE CBW:

    - Whether the extended CBW with new boundary of FOOB should comply with the recommendation by ITU-R on necessary bandwidth in terms of SE for the original UE BW

    - Whether default or fixed extension, e.g. 1/2 UE CBW could be stipulated in the spec or the extension ratio could be a UE capability to fulfil the conversion of outer to inner

    - Whether the extended UE CBW could belong to the regular defined channel bandwidth

    - Whether some premise should be established for the extension size, e.g.

      - minimum extended CBW compared to original UE CBW

      - minimum BS CBW compared to UE CBW

      - frequency separation of the RB allocation to the BS edge

    - Other identified issues are not precluded

Issue 1-2-8: Asymmetrical extended CBW approach

- WF

  - Asymmetrical extension is considered, FFS the solution.

Issue 1-2-10: Signaling aspects

- WF

  - To further discuss the signalling aspects in conjunction with the solution to be adopted for MPR reduction.

    - Companies are encouraged to provide thinking on signalling impact together with the proposed solution

Topic #2: MPR applicability for FR1 intra-band UL CA

Sub-topic 2-1: Intra-band contiguous UL CA

Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell

- WF

  - For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied

    - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

    - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are indicated

    - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are absent.

    - The Rel-18 power boosting feature can be supported depending on UE capability

      - Feature group description shall clarify the interaction between Rel-18 power boosting capabilities and Rel-19 MPR applicability enhancement capability.

  - FFS PC1.5 MPR enhancement as intra-band CA MPR requirements for PC1.5 are not available yet

Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE

- Proposals

  - Proposal 1: activated CC CBW or aggregated CBW adopted for integral region and boundary of spurious emissions/ACLR/SEM depending on dualPA-Architecture IE indication.

  - Proposal 2: Emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated.

- WF

  - FFS in next meeting

Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell

- Proposals

  - Proposal 1: CA requirements of ACLR/SEM/SE should be applied for intra-band contiguous CA with single activated cell.

  - Proposal 2:

    - For UE supporting contiguous ULCA with the dualPA IE:

      - The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

    - For UE supporting contiguous ULCA without signalling the dualPA IE or signalling PC1.5 or TxD or UL MIMO support:

      - The applicable SEM, ACLR and spurious emissions are the configured contiguous ULCA

  - Proposal 1: After the initial configuration of contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the aggregated CA BW SEM as stated in sub-clause 6.5A.2.2.1 in TS38.101-1. Further discuss SEM mask for single CC fall back only if dualPA-architecture is supported

  - Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

- WF

  - FFS in next meeting

Issue 2-1-4: Whether UL interruption is allowed for intra-band contiguous CA with single activated cell

- Proposals

  - Proposal 1: No interruptions are allowed for enabling single carrier MPR for the case when only one CC is activated in intra-band contiguous UL CA.

- Agreement in AH

  - Proposal 1

Sub-topic 2-2: Intra-band non-contiguous CA

Issue 2-2-1: Applicable MPR for FR1 intra-band non-contiguous UL CA

- Proposals

Option 1: No spec impact or minimum spec impact with some clarification but no changes of requirements

  - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

  - Proposal 2: adding 1CC activation as a supplementary scenario in the spec but no changes of the requirements

  - Proposal 3: single CC MPR applied with single CC emission requirements:

Option 2: Single CC MPR/SEM applied on indication of dualPA IE

  - Proposal 4: To apply non-CA (single CC) MPR based on indication of dualPA IE

  - Proposal 5: After the initial configuration of non-contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the composite SEM of the 2CCs as stated in sub-clause 6.5A.2.2.2 in TS38.101-1. Further discuss to support the single CC SEM only if dualPA-architecture is supported.

Option 3: Possible spec impact on LO retuning

  - Proposal 6: Discuss impact on requirements if LO needs to be re-configured to support single CC MPR. This discussion would be required for intra-band contiguous and non-contiguous carrier aggregation

- WF (to be discussed in main session)

  - For PC2 and PC3, no MPR requirements to be changed.

  - FFS PC1.5 when NC CA requirements are available

  - FFS clarification for the applicable emission requirements whether they are single CC based or composite based

  - Further check whether LO retuning is out of WI scope, if not included, drop the corresponding discussion in Rel-19

Topic #3: MPR applicability for FR2

Issue 3-1-2:  Applicable MPR for FR2 UL CA with DL intra band CA

Agreement in main session:

  - In general, with UE indication of new capability [FR2 MPR enhancements-R19] for MPR improvement, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW).

    - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused.

    - A note should be added to the MPR table, e.g.

  - For FR2-1, for 200MHz BW granularity, RAN4 to change the MPR calculation for DFT-s-BPSK or DFT-s-QPSK from “MPRC_CA = MAX(MPR1, MPR2)” to “MPRC_CA = MPR2”, for UE with UE indication of new capability for MPR improvement.

Issue 3-2-1:  Optional UE capability

Agreement in main session:

  - An optional per UE capability for FR2 is introduced

    - The capability is implementation agnostic

    - Supporting FR2 MPR enhancement is applicable only from Rel-19

Issue 3-2-2:  CC activation-based MPR improvement

- WF

  - Further discuss activated based MPR improvement.

    - FFS using common UE capability between configuration based and activation based, or another UE capability for activated-based improvements.

    - FFS MPR can be determined based on activated UL CCs BW instead of Cumulative aggregated channel bandwidth (CABW). (i.e., activation based also applies to UL CA case)

    - FFS limitation the enhancement to cases where the activated UL CCs form a contiguous block.

Reference: Description of scenarios from Topic #1

Scenarios 1:

- 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator)

- 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators)

Scenario 2: Narrower UE channel BW within wider BS bandwidth