3GPP TSG-RAN WG4 Meeting # 112bis					R4-2417193

Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			A-IoT general overview

Agenda item:		6.21.1

Document for:		Approval

# Introduction

In this paper, we present our thoughts on general issues for A-IoT SI.

# Discussion

### 6.7.1	System parameters

#### 6.7.1.1	Operating band

According to the SID (RP-240826), Ambient IoT uses FR1 licensed spectrum in FDD. NR band n8 can be used as an example band.

[Table 6.7.1.1-1 summarized the A-IoT system operation cases according to coexisting scenario.] It can be observed that The spectrum usage for A-IoT operation can be flexible, and it depends on several factors ,such as whether the reader is A-IoT BS  or intermediate UE, and whether CW transmission is deployed inside or outside topology. For the case where different spectrum is used for D2R and R2D, the FDD band definition can be kept. As CW transmission may interfere the R2D reception, the R2D and D2R should be TDM and therefore, for FDD band definition, it should be half duplex mode.

[Table 6.7.1.1-1: Factors impacting the A-IoT BS and UE as reader operation]

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | Comment |
| --- | --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum | [Coexisting scenario D1T1-A2] |
|  | DL spectrum | UL spectrum | UL spectrum | [Coexisting scenario D1T1-B] |
|  | UL spectrum | DL spectrum | DL spectrum | [] |
| UE as reader | UL spectrum | UL spectrum | UL spectrum | [Coexisting scenario D2T2-A2] |
|  | UL spectrum | DL spectrum | DL spectrum | [Coexisting scenario D2T2-B] |

#### 6.7.1.2	Channel bandwidth

The R2D waveform is agreed to be OFDM-based waveform, therefore the channel bandwidth definition can follow the channel bandwidth definition in NR.