3GPP TSG-RAN WG4 Meeting #112bis	 R4-2417207

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.17.6

Source: 	Qualcomm Incorporated

Title: 	WF on Data Sharing framework

Document for:	Approval

# Introduction

This document summarizes the agreements regarding the data sharing framework to be used by RAN4 for the work on AI/ML NR air.

# Discussion

Agreement:

All agreements for data sharing are only for the CSI compression use case feasibility study. These agreements can be revisited in an eventual WI phase.

## AI/ML model format

Agreement for AI/ML formats:

Use at least ONNX format for sharing.

Companies can also share PyTorch model

The models in different formats from one company shall be the same

Use version v1.16 or later

ONNX version and opset version number to be included in the file

## Dataset format

Agreement:

Use NumPy for dataset sharing

Do not use pickled data(compression mechanism)

Use npy – single array in each file

## Training Data file format

Agreement:

Dataset/model input file format:

    - N (samples) X 2 (IQ) X nSB (number of subbands) X nPorts (number of CSI-RS ports) X nLayers (number of layers)

        - In other words, dataset should content following info:

        - 1st dimension: Number of samples

        - 2nd dimension: Real and imaginary

        - 3rd dimension: Number of sub-bands

        - 4th dimension: Number of antenna ports

        - 5th dimension: Number of layers – present only if >1 layer

        - Note: Each element of the dataset will be a float32 real number

The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447

Single file for fixed scalar quantization(only dataset), TBD whether additional file is needed for variable/trainable quantization

Dataset files can be split into multiple files to enable easier upload.

- Use the 2 digits for split files starting from 00, increment for each additional file

Split files and then archive each file

## File naming scheme

Agreement:

- Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder”

- Subfolder created for each meeting

File naming scheme( ML model file and dataset file)

- a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary)

- meeting number

- differentiate model and dataset by identifier (2 characters ml and ds)

- additional identifier with 2 characters (TBD)

  - ec for encoder

  - dc for decoder

  - ei for encoder input

  - do for decoder output

  - lt for latent (decoder input or encoder output)

  - others to be added as needed

- dataset can be split in multiple files – 2 digits

- files to be compressed to zips and uploaded

Example:

Folders to be created under https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/

Folder for models: /Data_sharing/ NR_AIML_air/CSI_compression/Models/R4_XXX

Folder for datasets: /Data_sharing/ NR_AIML_air/CSI_compression/Datasets/R4_XXX

Example for model file:

vivoR4_113mlec.onnx

Example for dataset file:

vivoR4_113dsei00.npy

## Files to be shared

Agreement:

Files to be shared, to be decided based on option under study

    - Dataset containing encoder input per subband (as described in Section 5-3)

    - Encoder and/or decoder model (as described in Section 5-1)

# References

[1] R4-2415231, “Topic summary for [112-Bis][127] NR_AIML_air”, Moderator(Qualcomm Incorporated), RAN4#112-bis