# 3GPP TSG-RAN WG4 Meeting #114	R4-2500005

Athens, Greece, 17th – 21st Feb, 2025

3GPP TSG RAN WG1 #119			R1-2410870

Orlando, US, November 18th – 22nd, 2024

Title:					Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement

Response to:	R1-2409353 (R4-2417117)

Release:			Rel-19

Work Item:	    NR MIMO Phase 5

Source:			RAN1

To:					RAN4

Cc:

Contact person: Afshin Haghighat, Li Guo

E-mail Address: afshin.haghighat@interdigital.com, v-guoli@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Overall Description

RAN1 thanks RAN4 for the LS related to UE RF issues for Rel-19 MIMO Enhancement. Per request, RAN1 provides responses to the questions.

(NR MIMO 9.2.4) Enhancement for asymmetric DL sTRP/UL mTRP scenarios Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID. (NR MIMO 9.2.3) 9.2.3 Support for 3-antenna-port codebook-based transmissions RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks as below for example but not with 1Layer 3-port codebooks. 13100100, 13100001, 13 001001. Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers. Question 3: RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks.

Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

Regarding Questions 2 and 3, RAN1 would like to inform RAN4 that UL full power Mode 0 delivers full power for any precoder in the 3-port codebook, regardless of the number of layers in the precoder.

Reply for Question 2: Yes, a 3Tx UE supporting UL full power transmission mode 0, delivers the same maximum output power with both 1- and 2-layer precoders.

Reply for Question 3: No, a 3Tx UE supporting UL full power transmission mode 0, shall support full power with both 1- and 2-layer 3-port codebook.

# Actions

RAN1 respectfully asks RAN4 to take the above clarifications and replies into consideration in their future work.

# Dates of Next TSG RAN 1 Meetings

TSG RAN WG1 Meeting #120				Feb. 17 – Feb. 21, 2025, Athens, Greece

TSG RAN WG1 Meeting #120-bis			April 7 –  April 11, 2025, China