# 3GPP TSG-RAN WG4 Meeting #114	R4-2500006

Athens, Greece, 17th – 21st Feb, 2025

3GPP TSG RAN WG1 Meeting RAN1#119 		R1-2410889

Orlando, US, November 18th – 22nd, 2024

Title:	LS on impact on transmit signal quality/MPR requirement

Response to:

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN1

To:	RAN4

Cc:

Contact person:	Xinghua Song

songxinghua@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

On UL resource muting, RAN1 has made the following agreements

Agreement If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Note that RAN1 is still discussing the boosting in symbols containing PT-RS. RAN1 would like to ask RAN4 to check impact on transmit signal quality/MPR, if any.

# 2	Actions

To RAN4

ACTION: RAN1 respectfully asks RAN4 to take the above information into account in their future work.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#120	17th – 21st February 2025	 	Athens, Greece

RAN1#120bis	7th – 11th April 2025	 		China