# 3GPP TSG-RAN WG4 Meeting #114	R4-2500012

Athens, Greece, 17th – 21st Feb, 2025

3GPP TSG-RAN WG2 Meeting #128	R2-2410978

Orlando, US, 18 – 22 Nov, 2024

Title:	LS on soft satellite switch with re-sync

Response to:

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN2

To:	RAN4, RAN1

Cc:

Contact Person:

#### Name:	Lili Zheng

E-mail Address:	zhenglili4@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

For soft satellite switching with re-synchronization, to ensure non-colliding SSB with same PCI at the UE side, RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. To be specific, the target satellite transmits SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite. At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite.

RAN2 wonders whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

# 2. Actions:

To RAN4, RAN1

ACTION: 	RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

# 3. Date of Next RAN2 Meetings:

3GPP TSG RAN WG2#129	17 – 21 Feb, 2025		Athens, Greece

3GPP TSG RAN WG2#129bis	7 – 11 Apr, 2025		TBD, China