3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503108

Wuhan, China, 07th – 11th April. 2025

3GPP TSG RAN WG1 Meeting RAN1#120 		R1-2501620

Athens, Greece, February 17th – 21st, 2025

Title:	LS on L1 based UE-to-UE CLI measurement and reporting

Response to:

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN1

To:	RAN4

Cc:

Contact person:	Xinghua Song

songxinghua@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

The following working assumption was made in RAN1 on L1-based CLI reporting since RAN1 specifications must capture the format and bitwidths of the CLI reports.

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.

RAN1 assumes that the reporting range and granularity for L3 SRS-RSRP measurement in Table 10.1.22.1.2-1, TS38.133 can be reused for L1 SRS-RSRP. In addition, the reporting range and granularity for L3 CLI-RSSI measurement in Table 10.1.22.2.2-1 of TS38.133 can be reused for L1 CLI-RSSI.

For a CLI report, RAN1 agreed that the absolute value of the most interfered (largest) measured L1-SRS-RSRP or L1-CLI-RSSI is reported. RAN1 also agreed to use differential reporting for L1 SRS-RSRP and L1 CLI-RSSI when the number of reported measurement resources is larger than one, in the same manner as for existing L1-RSRP/SINR reporting.

RAN1 would like to ask RAN4 whether the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI are reasonable. RAN1 also would like to check with RAN4 whether there is any issue with the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI.

# 2	Actions

To RAN4

ACTION: RAN1 respectfully asks RAN4 to provide feedback to following two questions

- Is the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI in the RAN1 working assumption reasonable from RAN4 perspective?

- Is the range and resolution for differential reporting of L1 SRS-RSRP and CLI-RSSI reporting reasonable from RAN4 perspective?

# 3	Dates of next TSG RAN WG1 meetings

RAN1#120bis	7th – 11th April 2025	 		Wuhan, China

RAN1#121	19th – 23rd May 2025	 		Malta