3GPP TSG-RAN WG4 #114bis	R4-2503126

Wuhan, China, April 7-11, 2025

Agenda item:	7.25.2.3		On-demand SIB1

Source: 	Qualcomm Incorporated

Title: 	On RRM requirements for on-demand SIB1

Document for:	Discussion

# Introduction

In RAN4 #114, a few companies submitted contributions covering OD-SIB1 and identified potential impacts on RRM requirements [1]. In this paper we provide our view on RRM impact of OD-SIB1. The WID for OD-SIB1 is as follows [3].

Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

Discussion

In RAN4 #114, the following way forward was agreed for OD-SIB1 [2]. Companies are expected to provide their view on RRM impact of OD-SIB1 in RAN4 #114bis.

Issue 2-1-1: RRM requirements impacts for OD-SIB1

Agreement:

RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

Several scenarios were under discussion in RAN1 differing in where the UE sends the UL WUS signal to, where the UE gets the UL WUS configuration from and where the UE receives the OD-SIB1 from. At the end, RAN1 agreed to define requirements only for case 2 (option 1+B+X) with

- Option 1: UE transmits WUS to NES cell

- Option B: UE obtains UL WUS configuration from cell A

- Option X: UE receives OD-SIB1 from NES cell

This scenario is outlined in Figure 2-1. The UE receives UL WUS config from cell A, transmits UL WUS to the NES cell and receives the on-demand SIB1 from the NES cell as well.

Figure 2-1: OD-SIB1 Design Case 2

OD-SIB1 only applies to UEs in idle/inactive mode. Hence, potential impact for RRM requirements is only in these modes but not for RRC_CONNECTED.

Observation 1: Potential impact for RRM requirements is only in idle/inactive mode but not for RRC_CONNECTED.

In RAN2 #128, it was agreed that the legacy cell reselection criterion is reused as trigger condition of OD-SIB1 acquisition. Other conditions like new RSRP thresholds will not be specified. The cell selection criterion for the NES cell is based on measurements of the SSB of the NES cell. In RAN1 #118bis, it was concluded that no further optimization in RAN1 on power control and ramp-up procedure for UL WUS in Rel-19 will be done. The assumption in Rel-19 is that the NES cell transmits SSB at least on the sync-raster. Then the baseline operation is that, after the UE has received the UL WUS configuration from cell A and fulfills the cell reselection criterion, the UE transmits WUS to the NES cell and receives SIB1 from the NES cell. After cell reselection is completed, the UE camps on the NES cell.

In RAN1 #120, also the scenario was discussed that the NES cell is already transmitting OD-SIB1, and the UE may not need to send UL WUS to request OD-SIB1. No conclusion has yet been reached on this scenario in RAN1. Hence, we propose that RAN4 should not discuss this scenario yet and wait for further RAN1 progress.

Observation 2: The legacy cell reselection criterion is used as a trigger condition for OD-SIB1 acquisition. Hence, cell reselection related measurement procedures are not impacted by OD-SIB1.

Observation 3: The basic flow for OD-SIB1 is that the UE sends a UL WUS to the NES cell and receives OD-SIB1 from the NES cell. After cell reselection the UE camps in the NES cell.

Observation 4: It is under discussion in RAN1 whether the UE should also monitor type-0 PDCCH for OD-SIB1 in case that OD-SIB1 is already transmitted by the NES cell. In this case, the UE may not need to transmit UL WUS.

Proposal 1: RAN4 should not yet discuss the scenario that the UE does not need to transmit UL WUS to receive OD-SIB1 and wait for further progress in RAN1.

Sending UL WUS, receiving the RAR and monitoring / decoding OD-SIB1 are steps before the UE camps on the NES cell. This may have impact of the maximum interruption in paging reception. In TS 38.133, the maximum allowed interruption time is defined as TSI-NR + 2*Ttarget_cell_SMTC_period. Ttarget_cell_SMTC_period is the periodicity of the configured SMTC of the target cell and TSI-NR is the time required for receiving all relevant system information. In some tests, TSI-NR is assumed to be 1280 ms.

Since the core requirement for the maximum interruption time is written in a very generic manner, the core requirement is not impacted by OD-SIB1. It should be further investigated whether TSI-NR = 1280 ms can cover UL-WUS transmission and OD-SIB1 reception as well. If this is not the case, in the performance requirements a larger TSI-NR could be defined for OD-SIB1.

Observation 5: Since the core requirement for the maximum interruption time is very generic, the core requirement for paging interruption is not impacted by OD-SIB1.

Proposal 2: RAN4 should further investigate whether TSI-NR = 1280 ms can cover UL-WUS transmission and OD-SIB1 reception as well or whether in the performance tests a larger TSI-NR should be defined.

Another impact in RRM due to OD-SIB1 is the reference timing for UL WUS. US WUS is not sent to cell A but to the NES cell. Since the NES cell transmits SSB, it makes sense to use the SSB of the NES cell as downlink timing reference.

Proposal 3: Since the UL WUS is sent to the NES cell, RAN4 should discuss whether the SSB of the NES can be used as the downlink reference signal to define the reference timing.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114. Our observations and proposals are summarized as follows:

Observation 1: Potential impact for RRM requirements is only in idle/inactive mode but not for RRC_CONNECTED.

Observation 2: The legacy cell reselection criterion is used as a trigger condition for OD-SIB1 acquisition. Hence, cell reselection related measurement procedures are not impacted by OD-SIB1.

Observation 3: The basic flow for OD-SIB1 is that the UE sends a UL WUS to the NES cell and receives OD-SIB1 from the NES cell. After cell reselection the UE camps in the NES cell.

Observation 4: It is under discussion in RAN1 whether the UE should also monitor type-0 PDCCH for OD-SIB1 in case that OD-SIB1 is already transmitted by the NES cell. In this case, the UE may not need to transmit UL WUS.

Proposal 1: RAN4 should not yet discuss the scenario that the UE does not need to transmit UL WUS to receive OD-SIB1 and wait for further progress in RAN1.

Observation 5: Since the core requirement for the maximum interruption time is very generic, the core requirement for paging interruption is not impacted by OD-SIB1.

Proposal 2: RAN4 should further investigate whether TSI-NR = 1280 ms can cover UL-WUS transmission and OD-SIB1 reception as well or whether in the performance tests a larger TSI-NR should be defined.

Proposal 3: Since the UL WUS is sent to the NES cell, RAN4 should discuss whether the SSB of the NES can be used as the downlink reference signal to define the reference timing.

# References

- R4-2500538, Topic summary for [114][224] Netw_Energy_NR_enh_Part2, RAN4 #114, February 2025

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, RAN4 #114, February 2025

- RP-250343, Revised WID on enhancements of network energy savings for NR, RAN #107, March 2025