3GPP TSG-RAN WG4 Meeting #114bis	R4-2503157

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on performance requirements for 6 layer MIMO

Agenda item:	7.1.3

Document for:	Discussion

Introduction

The ongoing work item on UE RF enhancements in Rel-19 [2] includes the following objective (highlighting added for emphasis):

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

During RAN4#114, the ongoing discussion led to an agreement that 6 Layer MIMO was feasible for FWA Devices, but not for handheld.

In this document we present views to help define performance requirements for 6Rx devices

Discussion

General considerations for MIMO layer evaluation for 6Rx UE

4 and 6 layer requirements for 6Rx UE

As the agreement was made during RAN4#114 that only FWA devices could support 6-layer and 4-layer can be supported by handheld it is proposed that requirements are defined for both 4-layers and 6-layers.

Requirements shall be defined for 4-layer devices

Requirements shall be defined for 6-layer devices

Channel model

Regarding 6 layer devices supporting FWA, it is proposed that a relevant channel that mimics the FWA deployment (low doppler and velocity) should be used.

For 6-layer requirements, use TDLA 30-10 ULA Low

Regarding 4-layer devices being for handheld UE only, it is proposed that a relevant channel that mimics the a mobile handheld UE (medium doppler, velocity and correlation) should be used.

For 4-layer requirements, use TDLB 100-400 ULA Medium B

PDSCH and/or PMI and CQI reporting

Due to time constraints in release-19, it is proposed the focus on 6Rx requirements should be on non-CA PDSCH, and if time allows other channels can be looked at.

RAN4 shall only define requirements for 6Rx PDSCH initially.

Nokia notes that CQI requirements will be important, and would support CQI as a second priority.

CQI Requirements will likely be needed following PDSCH, if it makes good progress.

MCS Options

It is likely that 6Rx devices will require a high SNR operating region; so potentially a lower MCS should be used (compared to the used in for example 8Rx), a good starting point for assessment could be MCS 13 (table 2).

RAN4 to use MCS 13 (table 2) as a starting point for requirements derviation.

Conclusion

The paper discusses the topic of 6 Layer MIMO evaluation in RAN4 for performance requirements derivation, and presents some of the Nokia views on this topic for discussion during RAN4#114.

The following Observations and Proposals were made:

Proposal 1: Requirements shall be defined for 4-layer devices

Proposal 2: Requirements shall be defined for 6-layer devices

Proposal 3: For 6-layer requirements, use TDLA 30-10 ULA Low

Proposal 4: For 4-layer requirements, use TDLB 100-400 ULA Medium B

Proposal 5: RAN4 shall only define requirements for 6Rx PDSCH initially.

Observation 1: CQI Requirements will likely be needed following PDSCH, if it makes good progress.

Proposal 6: RAN4 to use MCS 13 (table 2) as a starting point for requirements derviation.

- References

- R4-2417095, “WF on requirements for 6Rx”, AT&T, RAN4#112-bis, Hefei, October  2024

- RP-241656,  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024