3GPP TSG-RAN WG4 Meeting #114bis	R4-2503158

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on inclusion in specifications of 6 Layer MIMO feasibility

Agenda item:	7.1.1.3.2

Document for:	Discussion

Introduction

The ongoing work item on UE RF enhancements in Rel-19 [2] includes the following objective (highlighting added for emphasis):

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

During RAN4#113, the ongoing discussion led to an agreement that 6 Layer MIMO was feasible for FWA Devices, but not for handheld; within that discussion it was captured that RAN4 should include this distinction in the specifications.

In this document we present a Nokia view on this to provide clarity on how this should be included in the specifications.

Discussion

Signaling for support of 6-layers

During RAN4#114, there seems to have been some discussion from some parties that RAN2 will need to define UE signalling as to whether a device is FWA or a handheld UE, based upon the agreement of 6-layer only being feasible for FWA devices.

This is not needed, as the device declares to the network the maximum number of layers which it can support through the ‘maxMIMOlayers’ capability definition. Which enables the UE to declare it’s support of how many layers it supports to the network.

UEs declare the maximum number of layers it can support to the network as is.

There is no need to include any further signaling based upon the 6Rx work item.

Inclusion in the specifications

In order to include the agreement that 6-layer is only feasible for FWA devices, it is agreed that this can be included in the specifications, however Nokia believes this should be a discussion once the requirements can be agreed. Furthermore, this can be included in TS 38.101-4.

RAN4 to include information to the effect that 6-layer is feasible for FWA devices in TS 38.101-4.

The exact wording of the information and location to be discussed once requirements are stable.

Conclusion

The paper discusses the topic of 6 Layer MIMO evaluation in RAN4, and presents some of the Nokia views on this topic for discussion during RAN4#114.

The following Observations and Proposals were made:

Observation 1: UEs declare the maximum number of layers it can support to the network as is.

Proposal 1: There is no need to include any further signaling based upon the 6Rx work item.

Proposal 2: RAN4 to include information to the effect that 6-layer is feasible for FWA devices in TS 38.101-4.

Proposal 3: The exact wording of the information and location to be discussed once requirements are stable.

- References

- R4-2417095, “WF on requirements for 6Rx”, AT&T, RAN4#112-bis, Hefei, October  2024

- RP-241656,  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024