3GPP TSG-RAN WG4 Meeting #114-Bis	R4-2503171

Wuhan, CN, Apr. 2025

Source	:	Qualcomm Incorporated

Title	:	On test method for UE RF requirements for the LPWUR

Agenda item	:	7.26.2.4

Document for	:	Approval

# 1.	Introduction

In this contribution we consider the test method to measure LPWUS detection reliability.

# 2. 	Discussion

## 2.1	LPWUS test concept

### 2.1.1	Consistency with RRM LPWUR/MR coex principles

In a companion contribution [3], we made the following observation regarding LPWUR operation (itself based on a WF from the RRM session [2]):

Observation 1: The UE can autonomously choose between LR and MR while meeting the network level goal of reliable detection of PDCCH, and whether LR was used to trigger PDCCH monitoring is not known outside the UE.

### 2.1.2	Test method concept

If the TE or network cannot be guaranteed whether the UE is using LPWUS to trigger PDCCH monitoring, LPWUS monitoring is fundamentally flowed as recorded as an observation in [3]

Observation 2: MDR of LPWUS is not directly testable, because the network or TE cannot enforce or verify that a feature-compliant UE engages its LPWUR for the entire duration of the test.

Consequently, an alternative method is needed to test LPWUS reliability.

In [3], we point out that a less intrusive and more meaningful way to test the LPWUS is to get confirmation of a successful PDCCH detection, perhaps as an ACK to a PDSCH grant for connected mode, and RACH MSG1/MSGA for idle mode UEs. Ultimately, this method is reliable provided RAN4 stick to a functional test with no reliance on special behavior beyond what is specified in RAN1.

Proposal 1: RAN4 stick to a functional test for the LPWUS requirement with no reliance on special behavior beyond what is specified in RAN1.

Furthermore, there is no way to maintain a UE in an ‘LPWUR-enabled’ state, at least per RAN1 design. The network has neither control nor ability to monitor for this UE condition. Consequently, it is possible to verify any RF parameters of the LPWUR only with a test mode that forces it to stay in the ‘on’ condition, and by test mode design no LPWUS monitoring occasions can be skipped. A test mode for a basic core requirement like Refsens is a significant negative from a RAN4 perspective because of not only deviation from mission-mode behavior but also significant additional burden on the UE.

Proposal 2: No test mode is specified in RAN4 for Rel-19 LPWUS and RAN4 defer to RAN5 to define the test method for the LPWUS feature.

Test mode specification is RAN5 business and is intimately tied to their chosen test method. The RAN4 proposal has no bearing on whether RAN5 choose to specify a test mode or not. RAN4 are not well placed to make determination about the necessity of a test mode simply because RAN4 does not design the test case.

## 2.2	Idle vs connected

RAN1 has already agreed to creating distinct capabilities for support of the feature in idle mode and connected mode. In a companion contribution [3], we have therefore proposed that:

Proposal 9: Two sets (each set further broken out into OOK-based and OFDM-based) of LPWUS requirements are specified, one set applies to UEs supporting LPWUS in idle/inactive mode and another for UEs supporting LPWUS in connected mode.

In effect, RAN4 must first set separate requirements for the two capabilities and only then decide if any applicability rule can eliminate low-value additional testing for those UEs that support both.

Proposal 3: RAN4 to discuss test skipping rule if a UE supports both, IDLE and CONNECTED mode Rel-19 LPWUS.

Functional mission mode testing naturally lends itself to connected mode and idle mode testing and no special consideration is necessary from a RAN4 perspective. RAN5 as always are free to make test time optimizations.

# 3.	Conclusions

Observation 1: The UE can autonomously choose between LR and MR while meeting the network level goal of reliable detection of PDCCH, and whether LR was used to trigger PDCCH monitoring is not known outside the UE.

Observation 2: MDR of LPWUS is not directly testable, because the network or TE cannot enforce or verify that a feature-compliant UE engages its LPWUR for the entire duration of the test.

Proposal 1: RAN4 stick to a functional test for the LPWUS requirement with no reliance on special behavior beyond what is specified in RAN1.

Proposal 2: No test mode is specified in RAN4 for Rel-19 LPWUS and RAN4 defer to RAN5 to define the test method for the LPWUS feature.

Proposal 3: RAN4 to discuss test skipping rule if a UE supports both, IDLE and CONNECTED mode Rel-19 LPWUS.

# 4.	References

[1] 	R4-2503028, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#114

[2]	R4-2502600, ‘WF on RRM requirements for NR_LPWUS’, vivo RAN4#114

[3] 	R4-2503170, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#114-Bis