3GPP TSG-RAN WG4 Meeting #114-Bis	R4-2503172

Wuhan, CN, Apr. 2025

Source	:	Qualcomm, Sony, Nokia

Title	:	On the UE RF requirement for the FR2 asymmetric connection scenario

Agenda item	:	7.22.2

Document for	:	Approval

# 1.	Introduction

RAN4 has been discussing an asymmetric connection scenario for FR2 with an aim to improving the UL budget. This scheme assumes ‘UL TRPs’ are deployed in higher density than legacy TRPs (UL+DL), and that the UE can expect to receive SSBs from these UL TRPs when ‘pathloss offset’ is not configured. In this contribution we develop the proposal while evaluating any new RAN1 impact.

# 2. 	FR2 enhancement discussion

## 2.1	The WID directive

As part of the Rel-19 MIMO WI, the RAN4 objective is to specify the necessary core requirements to support enhanced features from other working groups, which includes asymmetric DL sTRP/UL mTRP deployment scenarios.

Observation 1: RAN4 needs to specify the UE RF requirements for asymmetric DL sTRP/UL mTRP deployment scenarios based on the Rel-19 NR_MIMO-Ph5 WI.

## 2.2	The UE RF feature proposal

UL + DL TRP, legacy

UL TRP (*)

DL

SSB + PDSCH

UL

(*) – SSB available in DL

SSB only DL

UL TRP (*) configured to UE for UL

UL TRP (*)

UL TRP (*)

For convenience, the enhancement scenario is reproduced here from [2,3]. The basic premise is the existence of UL TRPs (with SSB capability), and that these can be deployed at higher density compared to the legacy TRP (UL +DL). This higher density increases the probability of LOS as well as reduced link distances, providing much need relief to the UL budget. To exploit this scheme the system would configure the UE to communicate with an UL TRP for UL alone. In more precise terms, the UL and DL are associated with different TCI states, corresponding to the UL TRP and the TRP associated with initial access respectively.

The proposal in short is for the UE to fulfil the legacy EIRP requirement when it is configured with different TCI states for UL and DL, and specifically when the QCL type D RS across both states are different [7]. In that contribution, we also highlighted the capability dependencies, and the arguments are not repeated here.

Proposal 1: RAN4 to define the following UE RF feature for FR2 NR_MIMO-Ph5: the UE must fulfil the legacy requirement on EIRP statistics when it is configured with different TCI states for UL and DL, and specifically when the QCL type D RSs across both TCI states are different.

The RAN1 reply to RAN4 on feasibility and validity of the premise of this proposal (UL TRP with SSB capability) is in Annex 1. To summarize:

Observation 2: The proposed feature (prop1 above) is consistent with RAN1 work and the WID.

## 2.3	The UE RF requirement

An overview of the legacy beam correspondence requirements is below:

- While tracking 1 beam management RS: Single UL, single DL, UL and DL in same direction (Rel. 15)

- While tracking 2 different beam management RS:

  - In Rel-16 inter-band CA, the two RSs exist in different bands with potentially different directions

  - In Rel-18 multiRx, the UE must form concurrent DL beams in two directions in the same band

The proposed feature differs from all 3 of the legacy features listed above by requiring the UE to track two different beam management reference signals in the same band, with the intent of using one for UL and another for DL. (i.e. The beams towards the TRPs are TDM’ed rather than concurrent). Consequently, a new beam correspondence requirement must be defined where the UL and DL TCI states, specifically the QCL type D RSs are different.

Proposal 2: The UE RF requirement is a new beam correspondence requirement with different UL and DL TCI states and specifically when the QCL type D RSs across both TCI states are different.

An example spec. implementation is below [6]:

Another aspect for the requirement is the relative spatial locations of the ‘legacy TRP’ and the ‘UL TRP’. For a test condition that represents field conditions, the TRPs must ideally be spatially separated. While such a test capability exists from the test set developed for the legacy Rel-18 multiRx feature, it is complex and expensive. In contrast, for Rel-16 DL inter-band CA, there was no spatial separation between the different beam management reference signals, although the reference signals are self-evidently different.

In this Rel-19 feature, since the UE must track the RS associated with each TCI state independently, the UE itself should have no preference on spatial separation. Consequently, zero separation is equivalent to any other choice, from the perspective of verifying UE functionality. This is the same logic used for the Rel-16 inter-band CA scheme. This scheme is beneficial because it re-uses the legacy single AoA FR2 test setup, a strong practical consideration.

Proposal 3: The legacy FR2 single AoA test set up is used for the UE RF requirement for the FR2 asymmetric connection feature.

## 2.4	Procedural arguments against the proposal

There have been some non-technical arguments opposing introduction of this feature like ‘Triggering new work in other WG’. These are addressed in Annex 2. In summary, the stipulation to ‘trigger no additional work’ in other WGs is not practical, because it implies that no input whatsoever can be given to another WG (i.e. no LSs seeking clarification can be triggered by one WG even for joint development). Furthermore, analysis for this proposal in Annex2 shows that RAN4 is unlikely to trigger new work in RAN1:

Observation 3: No further work is required in RAN1 to specify detailed UL parameters pertaining to power control, RACH, timing, TCI framework or spatial direction.

# 3.	Conclusions

Observation 1: RAN4 needs to specify the UE RF requirements for asymmetric DL sTRP/UL mTRP deployment scenarios based on the Rel-19 NR_MIMO-Ph5 WI.

Proposal 1:  RAN4 to define the following UE RF feature for FR2 NR_MIMO-Ph5: the UE must fulfil the legacy requirement on EIRP statistics when it is configured with different TCI states for UL and DL, and specifically when the QCL type D RSs across both TCI states are different.

Observation 2: The proposed feature (prop1 above) is consistent with RAN1 work and the WID.

Proposal 2: The UE RF requirement is a new beam correspondence requirement with different UL and DL TCI states and specifically when the QCL type D RSs across both TCI states are different.

Proposal 3: The legacy FR2 single AoA test set up is used for the UE RF requirement for the FR2 asymmetric connection feature.

Observation 3: No further work is required in RAN1 to specify detailed UL parameters pertaining to power control, RACH, timing, TCI framework or spatial direction.

# 4.	References

- RP-242394, ‘WID revision: NR MIMO Phase 5’, Samsung, RAN#105

- R4-2415724, ‘FR1 3Tx power scaling and FR2 asymmetric connection scenario’, Qualcomm et al., RAN4#112-Bis

- R4-2417632, ‘dCR on RF requirements for a UE with different TCI states for UL and DL’, Qualcomm, RAN4#113

- R4-2417117, ‘LS on UE RF issues for Rel-19 MIMO enhancement’, RAN4 (Huawei), RAN4#112-Bis

- R4-2500005, ‘Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement’, RAN1 (Interdigital et al.), RAN4#112-Bis

- R4-2503173, ‘dCR on RF requirements for a UE with different TCI states for UL and DL’, Qualcomm, Sony, Nokia, RAN4#114-Bis

- R4-2500110, ‘On the UE RF requirement for the FR2 asymmetric connection scenario’, Qualcomm, RAN4#114

- R4-2502667, ‘LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP’, RAN4 (CMCC), RAN4#114

- Chairman notes from RAN1#120

# 5.	Annex 1 – WID scope

For Rel-19, RAN4 has been investigating a proposal [2,3] for the FR2 asymmetric connection feature where UL and DL are associated with different TCI states, each linked to different DL RSs from the respective TRPs. Implicit in this proposal is that the UE can expect to receive SSBs from UL TRPs, but this aspect was not clear in RAN4. Consequently, RAN4 enquired if the SSB assumption (DL RS) of the proposal was in the scope of RAN1 work [4]:

The reply from RAN1 confirmed that the RAN4 proposal based on reception of SSBs from the UL TRP was indeed within scope of the Rel-19 work in RAN1 [5]

Unfortunately, the message is ambiguous for some companies, so we deconstruct the RAN1 reply logically. Noting that in an IF-THEN-ELSE construction, ‘else’ is equivalent to ‘if condition not true’, the reply can be broken down into:

If (condition true), no SSB from UL TRP

If (condition not true), UE can expect SSB from UL TRP

Here, ‘condition’ is configuration of PL offset.

The RAN1 LS reply effectively says ‘if PL offset is not configured, the UE may expect to receive SSB from UL TRP’.

# 6.	Annex 2 – RAN1 Impact

## 6.1	Motivation for this section

The opposing view to this proposal is based on the premise that no new work should be requested of RAN1 from RAN4. This is an unusual stipulation for a WI with joint development in RAN1 and RAN4. Perhaps there are special circumstances where this could be necessary, but examples of those have not been laid out for the FR2 proposal under discission by proponents of this viewpoint. In fact, RAN4 have sent 2 LSs on this subject to RAN1 seeking clarifications, and these always trigger technical work in RAN1. It is self-evident that the stipulation implies sending no LSs to RAN1 that request clarification.

Observation 6a: The stipulation to ‘trigger no additional work’ in other WGs is not practical, because it implies that no input whatsoever can be given to another WG (i.e. no LSs seeking clarification can be triggered by one WG even for joint development).

Companies opposing the proposed FR2 UE RF proposal to list precise technical details that prevent introduction of feature for consideration of their viewpoint.

A treatment more specific to this proposal is below:

The novelty of this feature is in the UL, therefore all aspects pertaining to UL must be re-evaluated. These include:

- RACH procedure

- Timing

- UL power control and

- Direction (UE Tx beam)

## 6.2	RACH to multiple TRPs

RAN1 continues to develop the RACH procedure necessary to support the asymmetric TRP operation. The summary from RAN1#120 is captured below:

In asymmetric DL sTRP/UL mTRP deployment scenario, the PDCCH-order PRACH to UL TRP is CFRA

Observation 6b: RAN1 design already supports RACH’ing to a second TRP using PDCCH-order contention-free RACH, and no new work is required in RAN1.

## 6.3	Timing aspects: Is 2TA operation enabled?

Two TAs can be configured to the UE currently using legacy mTRP mDCI framework, but mTRP is a very heavy feature. For this reason, RAN1 has also enabled 2TA operation in the UE for non mDCI mTRP scenarios like sTRP and mTRP sDCI  as agreed for FG 59 in RAN1 [R1-2501397], reproduced below.

59. NR_MIMO_Ph5; 59-4-4b; Support two TAs enhancement for inter-cell beam management operation; Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management; 23-1-1a; …..; Per FS.

Observation 6c: RAN1 has enabled 2TA operation in the sTRP framework under NR_MIMO_Ph5 and no new RAN1 work is necessary

## 6.4	UL power control

### 6.4.1	Open loop power control

The key to open loop power control is to have an unambiguous reference signal for pathloss calculation. This detail was resolved in RAN4#120, excerpted below:

(from RAN1#120) Agreement For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH: - The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH; - The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH: - In this case, the PRACH configuration associated with addition PCI is used. - This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured.

Observation 6d: RAN1 has unambiguously defined the PL RS for the case when PL offset is not configured

### 6.4.2	Closed loop power control

Closed loop power control is implemented by TPC. The TPC command for CLPC is DCI-resident and DCI can be sent from the DL TRP to change the Tx power towards the UL TRP. This can be done either with unified TCI or even with legacy TCI. RAN1 continues to discuss and enhance certain details, but these are not immediately relevant for the UE RF feature in RAN4.

Observation 6e: Legacy framework is sufficient for CLPC even when the UL TRP and the DL TRP do not share the same direction.

## 6.5	Direction of UL

The proposed feature is based on UL TRPs transmitting SSBs to help UEs locate them. In RAN1 terms, this means that the SSB from UL TRP can be configured as the QCL type D reference signal for the desired UL channels.

Observation 6f: The SSBs available to the UE from the UL TRP when PLoffset is not configured can serve as the QCL Type-D RS and no further RAN1 work is required for spatial determination of UL

## 6.6	TCI state framework

We have previously identified support of the Rel-17 unified TCI state framework as a prerequisite for this feature. The Rel-17 unified TCI state framework accommodates multiple ways to support the proposed UE RF feature:

- ‘Separate’ (for FR2), with one UL TCI state + one DL TCI state supported

- ‘Joint’, with one TCI state for each TRP. Both are active and network scheduling decides which QCL type D RSs are associated with UL and DL resp.

- ‘Separate’, with two DL TCI states + one UL TCI state, where the second DL TCI state may have same root QCL source (SSB) as the UL TCI state

Of the three, option #1 is the simplest, and is used for this feature, but there is no need to preclude any other case in RAN4.

Observation 6g: The Rel-17 unified TCI state framework accommodates multiple ways to support the proposed UE RF feature and there is no need to preclude any from the RAN4 requirement.

## 6.7	Summary of RAN1 impact from proposed feature

Section 2.4.2 to 2.4.5 confirm that RAN1 framework already exists to support power control, spatial direction and independent timing control. RACH to multiple TRPs has also been resolved in RAN1. In other words, RAN4 does not have to wait for any further developments in RAN1 to specify the proposed feature.

No further work is required in RAN1 to specify detailed UL parameters pertaining to power control, RACH, timing, TCI framework or spatial direction.