3GPP RAN WG4 Meeting #114bis																R4-2503178

Wuhan, China, 7th – 11th April 2025

Agenda item:	6.15.1

Source:	Apple

Title:	Further considerations on the emission requirements for the UE operating in 1626.5-1660.5MHz and 1668-1675MHz UL frequency ranges

WI/SI:	NR_NTN_combinedLband-Core

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already at that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. For instance, after the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2].

After the RAN#104 meeting another spectrum WI was agreed to introduce new bands operating in the L-band frequency range [3]. As discussed during the RAN4#114, there exist ETSI requirements that govern UE operation in the frequency ranges corresponding to the considered L-bands. Based on that RAN4#114 meeting concluded to capture the corresponding ETSI requirements with a few open issues on which NS flags will be needed, etc. Thus, in this discussion paper we present our technical analysis on the ETSI requirements and how they can be captured in the 3GPP core specifications.

# 2	Regulatory requirements for L-bands

## 2.1	Background and ETSI L-band frequency arrangements

The satellite L-bands are regulated by a number of documents including ETSI requirements, such as ETSI EN 301441 and EN 301681. It should be noted that ETSI EN 301441 covers the case when the L-band has only UL allocation at 1.6GHz, while the DL part is in the S-band at 2.5GHz; this combination has been introduced as 3GPP NTN band n254. On the contrary to it, ETSI EN 301681 covers the case when both DL and UL are in the L-band frequency range (more precisely 1.5GHz for the DL block and 1.6GHz for the UL block).  The table below from the corresponding ETSI document presents frequency block arrangements, for which additional ETSI specific requirements exist. As can be seen from the table these frequency ranges, sub-bands 1 and 2, corresponding to the considered 3GPP NTN bands.

Table 2.1-1: ETSI EN 301681 frequency block arrangements.

## 2.2	Emission requirements for L-bands (in- and out-of-band)

Referring to sub-clause 4.2.2 in ETSI EN 301681, the following in-band emission requirements are defined for a device operating in either sub-band 1 or sub-band 2. It should be noted that there are two sets of requirements which have slightly different "shape" of the resulting emission mask. Nevertheless, these emission masks are not the same as the existing NR general mask adopted for NTN.

Figure 2.2-1: ETSI in-band emission masks for a device operating in 1626.5-1660.5MHz (left table) and 1668-1675MHz (left and right tables)

In addition to the in-band emission requirements, ETSI EN 301681 also defines out-of-band emission requirements as presented below in Figure 2.2-2 (refer to sub-clause 4.2.1 in the ETSI EN 301681 for a complete technical description). So, while the table on the left-hand side is applicable to a device capable of operating in 1626.5-1660.5MHz frequency range, the second table is followed by a device implementing 1668-1675MHz or both sub-bands.

Figure 2.2-2: ETSI out-of-band emission requirements for a device operating in 1626.5-1660.5MHz (left table) and 1668-1675MHz (right table)

## 2.4	Emission requirements and NS flags

ETSI in-band emission requirements are presented in Table 2.3-1 and can be captured as the following table in the corresponding TS 38.101-5 sub-clause on "additional spectrum emission mask". And it should be noted that these in-band emission requirements are applicable to both ETSI sub-band 1 and 2.

Table 2.4-1: Additional requirements for ETSI sub-band 1 and 2

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

As for the out-of-band emission requirements, the ETSI sub-band 1 requirements can be captured as the following table in TS 38.101-5 sub-clause on "additional spurious emissions". Note that for the range of 1660.5-1662.5MHz same levels as for the in-band emissions from Table 2.4-1 should apply. Thus, for the sake of clarity the corresponding values are already substituted in Table 2.4-2 below.

Table 2.4-2: Additional out-of-band requirements for ETSI sub-band 1 (1626.5-1660.5MHz)

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1626.5 to 1660.5 | ETSI sub-band 1 |  |  |
| 1660.5 to 1660.525 | 30 to 15 | 3 kHz | Average |
| 1660.525 to 1660.625 | 15 to - 20 | 3 kHz | Average |
| 1660.625 to 1660.925 | -20 | 3 kHz | Average |
| 1660.925 to 1662 | -20 to -35 | 3 kHz | Average |
| 1662 to 1662.5 | -25 | 30 kHz | Average |
| 1662.5 to 1665.5 | -30 | 30 kHz | Average |
| 1665.5 to 1670.5 | -30 | 100 kHz | Average |
| 1670.5 to 1680.5 | -30 | 300 kHz | Average |
| 1680.5 to 1690.5 | -30 | 1 MHz | Average |
| 1690.5 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

The summary of ETSI sub-band 2 out-of-band emission requirements is presented in the table below and can be also captured in TS 38.101-5 sub-clause on "additional spurious emissions". Note that for the frequency ranges above and below band edges, same levels as for the in-band emissions from Table 2.4-1 should apply. Thus, for the sake of clarity the corresponding values are already substituted in Table 2.4-3 below.

Table 2.4-3: Additional out-of-band requirements for ETSI sub-band 2 (1668-1675MHz)

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1666.5 | -25 | 30 kHz | Average |
| 1666.5 to 1667.575 | -20 to -35 | 3 kHz | Average |
| 1667.575 to 1667.875 | -20 | 3 kHz | Average |
| 1667.875 to 1667.975 | 15 to - 20 | 3 kHz | Average |
| 1667.975 to 1668 | 30 to 15 | 3 kHz | Average |
| 1668 to 1675 | ETSI sub-band 2 |  |  |
| 1675 to 1675.025 | 30 to 15 | 3 kHz | Average |
| 1675.025 to 1675.125 | 15 to - 20 | 3 kHz | Average |
| 1675.125 to 1675.425 | -20 | 3 kHz | Average |
| 1675.425 to 1676.5 | -20 to -35 | 3 kHz | Average |
| 1676.5 to 1677 | -25 | 30 kHz | Average |
| 1677 to 1680 | -30 | 30 kHz | Average |
| 1680 to 1685 | -30 | 100 kHz | Average |
| 1685 to 1695 | -30 | 300 kHz | Average |
| 1695 to 1705 | -30 | 1 MHz | Average |
| 1705 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

Based on the summary of the ETSI requirements presented above we can see that we need at least two NS flags to capture additional ETSI requirements in the NTN band definitions. Since sub-ranges 1 and 2 have different out-of-band emission requirements, we will have to define one NS flag per each sub-range so that a UE knows whether ETSI requirements are applied or not in the given operator network.

Proposal 1:	Introduce two NS flags to define additional ETSI specific in- and out-of-band requirements for sub-band 1 (1626.5-1660.5MHz) and sub-band 2 (1668-1675MHz) frequency ranges.

Proposal 2:	The first NS flag will capture sub-band 1 requirements (in-band requirements from Table 2.4-1 and out-of-band requirements from Table 2.4-2)

Proposal 3:	The second NS flag will capture sub-band 2 requirements (in-band requirements from Table 2.4-1 and out-of-band requirements from Table 2.4-3)

## 2.5	Power back-off analysis for ETSI L-band sub-ranges

### 2.5.1	ETSI sub-range 1 (1626.5-1660.5MHz)

In this section we provide our preliminary power back-off results for a UE operating in the ETSI sub-band 1. We consider the following three major test points:

-	a 5MHz channel in the middle of the band (Fc=1645MHz)

-	a 5MHz channel at the lower edge of the band (Fc=1629MHz)

-	a 5MHz channel at the upper edge of the band (Fc=1658MHz)

As can be seen from the Figure 2.5.1-1 below, when the NR channel is in the middle of the band, no additional AMPR is anticipated: the out-of-band emission requirements are not relevant because of the size of the band and the in-band ETSI emission mask does not create any additional restrictions either. Of course, more simulations are needed to check that the same conclusion applies to larger channels, such as 20MHz.

Figure 2.5.1-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1645MHz).

As for the channels either at the lower or the upper edge of the band, extra A-MPR is needed for larger allocations with DFT-s-OFDM and the right-most channel edge allocations as presented in Figure 2.5.1-2 below. And the same observations can be done for the 5MHz channel at the upper edge of the band: left-most channel allocations will need extra A-MPR to comply with the out-of-band emission requirements.

Figure 2.5.1-2: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1629MHz).

Figure 2.5.1-3: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1658MHz).

### 2.5.2	ETSI sub-range 2 (1668-1675MHz)

By the time of writing this paper we have not run explicit power back-off simulations for this ETSI sub-band, but the following general observations can be already made:

-	since ETSI sub-band 2 out-of-band emission requirements are identical to the ETSI sub-band 1 upper edge out-of-band requirements, we anticipate same level of A-MPR needed for this sub-band;

-	the total sub-band 2 size is 7MHz, so only 5MHz channel is applicable to this band;

-	if the 5MHz channel resides right in the middle of the sub-band, i.e. having 1MHz "guard-bands" to the left and to the right, then power back-off is anticipated to be within existing MPR limits.

# 3	Conclusions

In this discussion paper we have presented our further technical analysis on the existing ETSI regulations for a device operating UL in the 1.6GHz frequency range. Since there are additional ETSI in- band out-of-band emission requirements for the 1626.5-1660.5MHz and 1668-1675MHz sub-ranges, we suggest introducing two NS flags that will be associated with the corresponding ETSI requirements.

Proposal 1a:	Introduce two NS flags to define additional ETSI specific in- and out-of-band requirements for sub-band 1 (1626.5-1660.5MHz) and sub-band 2 (1668-1675MHz) frequency ranges.

Proposal 1b:	The first NS flag will capture sub-band 1 requirements (in-band requirements from Table 2.4-1 and out-of-band requirements from Table 2.4-2)

Proposal 1c:	The second NS flag will capture sub-band 2 requirements (in-band requirements from Table 2.4-1 and out-of-band requirements from Table 2.4-3)

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

RP-241689, "Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges ", Inmarsat, Viasat