3GPP TSG-RAN WG4 Meeting #114bis	R4-2503229

Wuhan, Hubei, China, 7th – 11th April, 2025

Agenda Item:	7.19.1

Source:	CAICT

Title:	Discussion on general aspects of AI/ML for NR air interface

Document for:	Discussion

1	Introduction

In this contribution, we discuss following issues from last meeting:

- Issue 1-2-3: Post deployment options

- Issue 1-3-2: Delay requirements

2	Discussion

Issue 1-2-3: Post-deployment options

In last meeting, a Way Forward [1] is drafted for next step discussion, in which “further potential clarification” and “significant issues” parts were raised for each option. Accordingly, we provide our initial thinkings on the discussion points of option 2 (Post deployment management based on LCM):

| Further potential clarification | - Whether the metric used for performance testing and the metric used for monitoring can be aligned. - Whether monitoring used for post-deployment testing should be NW sided only. |
| --- | --- |
| Significant issues | - Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) - Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs. |

- Whether the metric used for performance testing and the metric used for monitoring can be aligned: This may depend on different use cases. For beam management, the testing metrics could be same as monitoring metrics, both taking RSRP prediction accuracy and/or beam prediction accuracy. But for CSI prediction, the testing metrics would be the ratio γ while the monitoring metrics could be some intermediate metrics, for example SGCS or NMSE. The discussion of monitoring metric is within RAN1 scope.

- Whether monitoring used for post-deployment testing should be NW sided only: Our understanding is, it does not need to be limited to NW-side. For the case of CSI prediction, monitoring on UE side is more straightforward since UE owns the measured CSI information.

- Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.): In our understanding, performance monitoring is based on measurements. During measurement, UE would keep recording some statistics information on metrics, such as, channel quality, noise level, delay spread and doppler spread, along with performance monitoring. These statistics information can be used to identify the root cause of performance degradation once being discovered in performance monitoring. The details could be up to inner implementation and optimization.

- Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs: RAN4 needs to define requirements for LCM monitoring, at least including delay requirements of monitoring reporting and model switching/fallback.

Regarding to option 1 (Pre-activation functionality/model update testing), the concerns are two-fold. Firstly, conducting conformance tests/validation for all potential models or updates would induce significant testing workload, of which the feasibility and necessity are both questionable. If we differentiate so-called major and minor update, the criteria would require more discussion. Clear rules should be specified to help effectively planning and managing tests. Secondly, if pre-activation tests are conducted in lab environment, it is nearly the same as conformance test, unless field data could be introduced to verify the performance. If it means verifying the model through real operation in the field, the procedure is similar as performance monitoring and LCM.

Proposal 1: Prioritize option 2 (performance monitoring and LCM) for post-deployment handling.

Proposal 2: For LCM monitoring, suggest RAN4 to consider specifying requirements on delay requirements of monitoring reporting and model switching/fallback.

Issue 1-3-2: Delay requirements

In this section, we discuss the delay requirements of transition from AI to non-AI. If both AI and non-AI are configuration-based, we understand it resembles legacy activation and deactivation. Thus, our discussion focuses on fallback transition which is trigger by performance monitoring. The motivation to specify delay requirement of fallback transition is to guarantee the effectiveness and performance under the case that severe performance degradation or dramatic condition change is detected and no suitable/available model is in hand. Consequently, UE needs to timely fallback to non-AI mode.

Some relevant agreements in RAN2 are excerpted as below.

Agreement (RAN2 #125bis): •For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). •“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19 Agreement (RAN2 #126): •For BM use case, As a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality. FFS reactive Agreement (RAN2 #127bis): •UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality) Agreement (RAN2 #128): •When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Based on RAN2 progress, the case of “UE-autonomous, UE’s decision is not reported to network” is precluded. Remaining possible cases are:

- NW-decision and NW-initiated: Considering the decision process is inner part of gNB implementation, thus delay requirement of this case could be considered as the duration from gNB transmitting fallback command to UE being ready to starting transmission/reception with non-AI configuration.

- NW-decision and UE-initiated: the difference from 1st case is, the LCM request is initiated by UE and NW responses to the request based on own inner decision. Exact form of the LCM request could be further discussed. The delay requirement of this case may depend on the form the LCM request. Taking monitoring metrics on UE-side as an exemplary LCM request, the delay requirement could be considered as the duration from UE reporting to UE being ready to starting transmission/reception with non-AI configuration, which implicitly covers the NW-decision periods.

- UE-decision, reporting and NW-initiated: monitoring result is reported by UE for this case. It has not decided whether the reporting is event-triggered or not. If so, an additional component of delay uncertainty from when the UE receives event configuration until a condition exists at the measurement reference point which will trigger the reporting, shall be incorporated in the delay requirement. If not, it would be analogous to 2nd case.

For beam management, RAN1 agreed to support Type-1 performance monitoring and FFS Type-2. According to definition in TR 38.843, Type-1 includes aforementioned case 1 and 3.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

For position case 1, RAN1 agreed to support option A and FFS option B. Option A may include aforementioned case 2 and 3 since the content of monitoring outcome has not been decided. Position case 3a/3b are both NW-side model which is out the scope of the discussion. Position case 2a/2b have been treated as 2nd priority without sufficient progress on monitoring discussion. Overall, all 3 possible cases currently are still on table for further discussion.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: • Option A. The target UE side performs monitoring metric calculation. o The target UE may signal the monitoring outcome to the LMF. o FFS: content of monitoring outcome • FFS: Option B

Observation 1: For beam management, RAN1 agreed to support Type-1 performance monitoring, which includes case 1 (NW-decision and NW-initiated) and 3 (UE-decision, reporting and NW-initiated).

Observation 2: For positioning case 1, RAN1 agreed to support option A of performance monitoring, which includes case 2 (NW-decision and UE-initiated) and 3.

Proposal 3: Following cases could be considered when discussing delay requirement of AI fallback to non-AI, including:

- NW-decision and NW-initiated

- NW-decision and UE-initiated

- UE-decision, reporting and NW-initiated

Conclusion

In this contribution, the issue of post-deployment and delay requirement of AI to non-AI transition are discussed with following observations and proposals:

Proposal 1: Prioritize option 2 (performance monitoring and LCM) for post-deployment handling.

Proposal 2: For LCM monitoring, suggest RAN4 to consider specifying requirements on delay requirements of monitoring reporting and model switching/fallback.

Observation 1: For beam management, RAN1 agreed to support Type-1 performance monitoring, which includes case 1 (NW-decision and NW-initiated) and 3 (UE-decision, reporting and NW-initiated).

Observation 2: For positioning case 1, RAN1 agreed to support option A of performance monitoring, which includes case 2 (NW-decision and UE-initiated) and 3.

Proposal 3: Following cases could be considered when discussing delay requirement of AI fallback to non-AI, including:

- NW-decision and NW-initiated

- NW-decision and UE-initiated

- UE-decision, reporting and NW-initiated

Reference

[1] R4-2502953, WF on AI/ML air interface study, Moderator (Ericsson)