3GPP TSG-RAN WG4 Meeting #114bis	R4-2503230

Wuhan, Hubei, China, 7th – 11th April, 2025

Agenda Item:	7.19.3

Source:	CAICT

Title:	Discussion on RRM core requirements for beam management

Document for:	Discussion

1	Introduction

In this contribution, we provide some discussions on absolute RSRP accuracy and relative RSRP accuracy.

2	Discussion

2.1 Absolute RSRP accuracy

In RAN4 #112bis meeting, RAN4 had following agreement on definition of absolute RSRP accuracy.

Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In current tests of L1-RSRP measurement accuracy in specification, the power level of beams and channel conditions are specified, such that the benchmark of measured value is deterministic. With the deterministic benchmark value, RAN4 is able to define a range of reporting that could meet the accuracy requirements. Based on our understanding, the part “ground-truth of L1-RSRP of beam index i” in the definition of absolute RSRP accuracy serves the same as benchmark value. Therefore, the measurement under certain high SINR level for ground-truth shall meet a tight accuracy requirement compared to legacy.

Proposal 1: Suggest to discuss the feasibility of defining a tight requirement for measurement used to generate ground truth and how to verify the measurement performance.

2.2 Relative RSRP accuracy

Issue 3-1: Relative RSRP Accuracy - Proposals - Option 1: The definition of relative RSRP accuracy shall be: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j - Option 2: The definition of relative RSRP accuracy shall be: predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams. - Option 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams. - Option 4: others - Recommended WF - Option 1

It is essential firstly to identify the motivation of defining relative RSRP accuracy. In our understanding, for models that are able to predict beam index and RSRP, absolute RSRP accuracy is adequate to verify the mode performance, considering RSRP prediction accuracy and beam prediction accuracy are positively correlated. A more accurate prediction of RSRP value is more likely to induce an accurate prediction of beam index.

While for the models that only predict beam indices, the critical point is whether beam prediction accuracy, proposed under “issue: Metrics/KPIs for beam prediction”, is adequate to verify the performance or not. We understand this may relate to the design of beam reporting from UE and subsequent NW’s reaction:

- UE reports Top-K beams with strongest beam index (if specified),

  - NW directly chooses the strongest beam based on UE reporting: the holistic performance deeply relies on the accuracy of beam prediction. For example, in figure 1, UE measures 3 beams and reports Top-2 (K=2) beams. The solid bar indicates the ground truth value and dash bar indicates the ranking marks inside the model of each predicted beam. Base on prediction results, UE would report beam #1 and #2 to NW and NW shall choose beam #1 for DL transmission. It can be seen that though the prediction satisfies Top-K/1 and Top-1/K metrics, the actual RSRP gap is large between the two beams and thus a degraded performance would be expected. Consequently, it is better to have relative RSRP accuracy requirement in addition to beam prediction accuracy. The definition of relative RSRP accuracy can go with option 1 in the proposal.

  - NW randomly chooses one out of the K beams: the randomness diminishes the dependency on UE reporting, but NW still possibly falls into an inferior beam, and the probability grows with a larger reported value K. From this perspective, it would also be beneficial to have relative RSRP accuracy requirement to guarantee a set of sufficient high-quality candidate beams.

  - NW initiates another round of beam measurement for reporting: NW’s decision on DL transmission beam depends on another round of beam measurement reporting, thus beam prediction accuracy together with measurement RSRP accuracy are adequate to verify the performance.

- UE reports Top-K beams without sorting

  - NW randomly chooses one out of the K beams: similar to case 1.b.

  - NW initiates another round of beam measurement for reporting: similar to case 1.c.

Figure 1 Illustrative example of beam prediction

Overall, relative RSRP accuracy shows the benefit for models that only output predicted beam indices, and shall be defined as option 1.

The definition in option 2 and 3 requires clarification before further decision: Option 3 follows the definition of relative RSRP accuracy in legacy specification with replacing measurement by prediction, as copied below. The vague part of option 2 is “owns largest RSRP”, whether it is largest measurement value or largest predicted value. If taking predicted value, option 2 has no much difference with option 3. If taking measurement value, how the metric promotes the performance is unclear as shown in the example.

Observation 1: Relative RSRP accuracy shows the benefit for models of beam prediction that only output beam indices.

Observation 2: Definitions in option 2 and 3 require more clarification before further decision.

Proposal 2: Suggest to consider option 1 as the definition of relative RSRP accuracy requirement,

- Option 1: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j

Conclusion

In this contribution, RRM core requirements for beam management are discussed with following proposals:

Proposal 1: Suggest to discuss the feasibility of defining a tight requirement for measurement used to generate ground truth and how to verify the measurement performance.

Observation 1: Relative RSRP accuracy shows the benefit for models of beam prediction that only output beam indices.

Observation 2: Definitions in option 2 and 3 require more clarification before further decision.

Proposal 2: Suggest to consider option 1 as the definition of relative RSRP accuracy requirement,

- Option 1: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j