# 3GPP TSG-RAN WG4 Meeting # 114b                                                         R4-2503238

# Wuhan, China, 7 – 11 April 2025

# Title:	On 1UL HPUE MSD simplifications

# Agenda Item:	7.1.1.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

The discussion paper presents our views on 1UL MSD simplifications for Rel-19 as a follow up of [1]. We also address concerns raised at RAN4 meeting #114 on our HPUE to power class 3 (PC3) look-up tables (LUT).

# Discussion

At meeting #113, it was agreed in way forward [2] that RAN4 HPUE MSD requirements will be simplified using our previous proposals [3,4,5,6] of delta power class MSD “PCx” (x=1.5 or 2) as reproduced below.

Based on Figure 1 [6], we remind that only 3 variables are needed to calculate the 1UL HPUE PCx and corresponding LUTs/equations.

- PC3 Main to Diversity branch interference power imbalance,

- Interference power level increase from 1Tx PC3 to 1Tx PC2,

- Reverse-IMD interference power level increase for 2Tx PC3+PC3 or PC2+PC2.

Figure 1: Example of Band n77 UL2/DL1 MSD due to Band n25 1UL PC2 1Tx and PC2 2Tx operation, assuming 2.9dB 1Tx PC2 interference increase from 1Tx PC3, 0.5dB rev-IMD and 9dB Main/Div. imbalance.

In this paper, we recap and re-submit the justifications from [1] for adopting 6dB PC3 Main/Diversity imbalance and 3dB 1Tx PC2 interference increase from 1Tx PC3. For reverse-IMD, we do not see the need to further discuss since there was consensus to account for 0.5dB impact [2].

## Addressing concerns on assumptions to calculate “PCx” and 1UL HPUE LUTs.

### Justification on 3dB interference increase from 1Tx PC3 to 1Tx PC2

For ACLR “1/2 regions” and for all harmonics:

Assume that the RAN4 PC2 and PC3 PA MPR1 calibration points were identical, say to meet -30dBc ACLR for both power classes, then we may assume that the 1Tx PC2 PA emission levels would be 3dB higher than the 1Tx PC3 emissions within both ACLR “regions.”

For cross-band isolation MSD regions “> ACLR2:”

We may assume two cases:

- 1Tx PC2 is achieved with 3dB higher PA gain and say near identical noise figure than 1Tx PC3.

- 1Tx PC2 is achieved without 3dB PA gain increase. In this case the RF transceiver delivers at least 3dB higher input power to the PA.

In both cases, it is reasonable to assume that the “flat-band” noise of the region “> ACLR2” also increases by 3dB.

In practice,

For cross-band isolation:

- On one hand, RAN4 calibrates the 1Tx PC2 PA MPR1 linearity with -31dBc ACLR, i.e., the PC2 linearity is slightly better than the 1Tx PC3 MPR1 calibration point of -30dBc. Hence, the difference between 1Tx PC3 and 1Tx PC2 PA emissions may be slightly less than 3dB.

- On the other hand, RAN4 MSD analyses are usually evaluated without MPR, i.e. at MPR0. This means the PA is driven 1dB further into compression. This may lead to cases where the interference level difference may slightly exceed 3dB, and cases where it may slightly be lower than 3dB depending on how the compression affects the combining of multiple IMD products within each ACLR “1/2 region.”

For harmonic MSD and harmonic mixing MSD:

Our 1Tx PC2 measurements of PA harmonics at MPR0 in [4] have shown that the PC2 harmonics levels are approximately 3dB higher than the PC3 levels. Like cross-band cases, this is only an approximation since the actual measured levels are sometimes slightly greater than 3dB, sometimes slightly lower than 3dB as shown in Figure 2 [4]. The 3dB harmonic level increase is an assumption that can be used for both harmonic MSD, and for harmonic mixing MSD since the source of interference in both cases is the same PA harmonic interference. For Rx harmonic mixing, we assume that all other Rx parameters remain identical when the PA power class is changed from PC3 to PC2 or PC2+PC2, for example the Rx mixer harmonic gain does not change with PC.

Figure 2:  Measured fundamental (H1), H2, H3, H4, H5 harmonic interference power difference between MPR0 1Tx PC3 and MPR0 1Tx PC2

Finally, we would like to recap that these practical considerations are valid for MSD evaluations based on measurements. Most MSD analyses are based on “excel calculations” where the increase of the PA interference levels is either modelled, or sometimes simulated given a certain PA model. This variety of approaches result in different assumptions used for legacy HPUE MSD analyses. We have also observed MSD Text Proposals (TP) where the 3dB increase was not applied to the interference level but to the PC3 MSD. This may have also contributed to discrepancies between band combinations.

In summary, 3dB interference increase from 1Tx PC3 to 1Tx PC2 may be considered as a reasonably and sufficiently accurate approximation to serve the goal of building MSD simplification look-up tables.

Observation 1: 1UL HPUE MSD simplification may be implemented by assuming that the 1Tx PC2 harmonic and the 1Tx PC2 ACLR interference is 3dB greater than the 1Tx PC3 levels.

### Justification on 6dB interference increase due to 2Tx reverse-IMD

In WF [2], [9]dB Main/Diversity imbalance was recommended as a way to ensure the PC1.5 maximum value (plateau) “PC1.5max” is greater than “PC22Tx”. In [1] we proposed instead to adopt [6dB] imbalance to serve two purposes:

- Ensure “PC1.5max” is greater than “PC22Tx” according to WF [2],

- Prevent “PC1.5max” from reaching excessive near ~11dB value. With 6dB imbalance, “PC1.5max” reaches 9dB.

The main concern is that 11dB “PC1.5max” may penalize PC1.5 performance when 9dB imbalance is assumed as shown in Figure 3.

Figure 3:  PC2, PC1.5 curves for PC3 1Tx Main/Div. imbalance of 6dB (left) and 9dB (right) and 0.5dB rev-IMD, 3dB PC2 1Tx interference increase from PC3 1Tx.

At meeting #114, several companies commented that 6dB imbalance may not be representative of all cases. This is true for some cases; however, we would like to remind that the specified MSD requirements rarely represent the assumptions of all contributing companies. This is because when different MSD requirements are proposed for a given band combination, RAN4 consensus is often reached by averaging the MSD levels from each TP.

For example, for the band n78 MSD due to the band n3 1Tx PC2 operation, three different Main/Div. total noise imbalance were evaluated:  2.4dB, 6.3dB and 16dB [8, 9, 10]. Consensus was reached by averaging the MSD requirements of each TP. The MSD averaging process results in an “virtual” Main/Div. imbalance of ~9dB, which was not evaluated by any company.

In summary, it can be noted that it is near impossible to find a single Main/Div interference imbalance assumption that could be considered as representative of several years’ worth of RAN4 work and several tens (if not hundreds) of MSD analyses. We believe that the 6dB assumption is a reasonable compromise that should be adopted to design MSD simplification LUT/equations.

Observation 2: The adoption of a single PC3 1Tx Main/Div. interference imbalance assumption cannot represent every company’s RF-FE assumptions. RAN4 rarely adopts MSD requirements that are 100% aligned with all company’s views and assumptions. This is because RAN4 often finds consensus by averaging the MSD evaluations from all contributing companies. In some extreme cases, RAN4 even ignored “outliers” when averaging MSD requirements.

Observation 3: Assuming PC3 1Tx 6dB Main/Div. interference imbalance may not represent all RF-FE architecture assumptions, but this assumption may be considered as a reasonable compromise to ensure that PC1.5 is not penalized.

## Addressing concerns on specifying a single 1UL HPUE LUT for all MSD types.

There are there 1UL MSD types:

- MSD due to harmonic interference;

- MSD due to harmonic mixing interference; and

- MSD due to cross-band isolation.

Based on Observations 1 and 3, we proposed to adopt the single LUT set of  Table 1 for all 1UL MSD types, with 3dB, 6dB, and 9dB PC21Tx_max, PC22Tx_max and PC1.5max.

Table 1: 1UL PC21Tx , PC22Tx , PC1.5 LUT.

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| <1 | 0.6 | 0.9 | 2.0 |
| 1 | 0.8 | 1.2 | 2.7 |
| 2 | 1.2 | 2.1 | 4.1 |
| 3 | 1.6 | 2.7 | 5.1 |
| 4 | 1.8 | 3.3 | 5.9 |
| 5≤ <7 | 2.0 | 4.0 | 7.0 |
| 7≤ ≤9 | 2.5 | 5.0 | 8.0 |
| ≥10 | 3.0 | 6.0 | 9.0 |

However:

- We acknowledge that indeed, the PC1.5 LUT can not yet apply to “1UL Interference MSD” requirements since the HPUE configurations that may lead to PC1.5 harmonic MSD are not yet supported in Rel-18 and Rel-19. To address this issue, we propose to adopt the Table 2 companion LUT that maps the 1UL MSD type to the HPUE LUT. It should be noted that such companion LUT may also become useful to accommodate future “1UL” HPUE valid configurations.

Table 2: Companion LUT mapping 1UL MSD type to 1UL HPUE PCx LUT

| MSD type |  | MSDmax (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 |
| 1UL | UL harmonic | PC2 1Tx | PC2 2Tx | [PC1.5 2Tx] |
|  | Rx mixing | PC2 1Tx | PC2 2Tx | PC1.5 2Tx |
|  | Cross-band isolation | PC2 1Tx | PC2 2Tx | PC1.5 2Tx |

- Our view is that the goal of the MSD simplification is not to match the “legacy”/previously agreed MSD requirements, i.e. the 1UL HPUE LUTs do not aim at achieving the best possible fitting of previously agreed MSD requirements.

As discussed in meeting #111, we view this LUT initiative as a way to:

- Reduce RAN4 MSD analysis workload;

- Remove inconsistencies across band combinations; and

- Minimize errors by adopting a single set of assumptions.

Observation 4: All 1UL MSD types can be mapped to either 3dB, 6dB or 9dB MSDmax HPUE simplification LUT. A companion LUT is needed 1) to reflect the current valid 1UL HPUE configurations, 2) to anticipate future 1UL HPUE configurations. It should be noted that some of the 2UL ∆MSD can also be mapped to these 3,6,9dB LUT.

## Addressing HPUE MSD requirements when no PC3 MSD has been specified

We consider two ways to address the exceptional cases where 1UL HPUE MSD may be required even though no PC3 MSD has been specified:

Option 1:

- For the configurations where PC3 MSD is not specified, the 1UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b; and

- To minimize impact on workload, the MSD levels are clamped to the first MSD entry of each LUT. Taking the example of LUT from [1], specify the PC2 1Tx MSD is set at 0.6dB,  PC2 2Tx MSD at 0.9dB and PC1.5 MSD at 2.0dB.

Option 2:

- As an exception, and because the Option 1 MSD levels are anyway very small, do not specify these exceptions MSD requirements.

Option 2 is preferred as it further simplifies RAN4 TS and workload.

Observation 5: Two options may be considered to handle the exceptional cases of band combinations which may be affected by 1UL HPUE MSD, and yet do not have any PC3 MSD requirements:

Option 1:

- For the configurations where PC3 MSD is not specified, the 1UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b; and

- To minimize impact on workload, the MSD levels are clamped to the first MSD entry of each LUT. Taking the example of LUT from [1], specify the PC2 1Tx MSD is set at 0.6dB, PC2 2Tx MSD at 0.9dB and PC1.5 MSD at 2.0dB.

Option 2:

- As an exception, and because the option 1 MSD levels are anyway very small, do not specify these exceptions MSD requirements.

Finally, it may be possible to further reduce the conformance test burden by

- Verifying only the UE performance of the highest supported power class; and

- Skipping the MSD verification of the second “optional” MSD test points.

Observation 6: To reduce the conformance test burden of HPUE band combinations, consider the following two additional simplifications:

- RAN5 to verify only the MSD of the highest supported power class; and

- RAN5 to skip the MSD verification of the 2nd “optional” MSD test points.

# Conclusion

This paper captures our views on simplifying the 1UL MSD requirements for Rel-19 HPUE NRCA inter-band combinations. A “unified” proposal that covers both 1UL and 2UL MSD simplification can be found in [11].

Observation 1: 1UL HPUE MSD simplification may be implemented by assuming that the 1Tx PC2 harmonic and the 1Tx PC2 ACLR interference is 3dB greater than the 1Tx PC3 levels.

Observation 2: The adoption of a single PC3 1Tx Main/Div. interference imbalance assumption cannot represent every company’s RF-FE assumptions. RAN4 rarely adopts MSD requirements that are 100% aligned with all company’s views and assumptions. This is because RAN4 often finds consensus by averaging the MSD evaluations from all contributing companies. In some extreme cases, RAN4 even ignored “outliers” when averaging MSD requirements.

Observation 3: Assuming PC3 1Tx 6dB Main/Div. interference imbalance may not represent all RF-FE architecture assumptions, but this assumption may be considered as a reasonable compromise to ensure that PC1.5 is not penalized.

Observation 4: All 1UL MSD types can be mapped to either 3dB, 6dB or 9dB MSDmax HPUE simplification LUT. A companion LUT is needed 1) to reflect the current valid 1UL HPUE configurations, 2) to anticipate future 1UL HPUE configurations. It should be noted that some of the 2UL ∆MSD can also be mapped to these 3,6,9dB LUT.

Observation 5: Two options may be considered to handle the exceptional cases of band combinations which may be affected by 1UL HPUE MSD, and yet do not have any PC3 MSD requirements:

Option 1:

- For the configurations where PC3 MSD is not specified, the 1UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b; and

- To minimize impact on workload, the MSD levels are clamped to the first MSD entry of each LUT. Taking the example of LUT from [1], specify the PC2 1Tx MSD is set at 0.6dB, PC2 2Tx MSD at 0.9dB and PC1.5 MSD at 2.0dB.

Option 2:

- As an exception, and because the Option 1 MSD levels are anyway very small, do not specify these exceptions MSD requirements.

Observation 6: To reduce the conformance test burden of HPUE band combinations, consider the following two additional simplifications:

- RAN5 to verify only the MSD of the highest supported power class; and

- RAN5 to skip the MSD verification of the second “optional” MSD test points.

# References

- R4-2501955 1UL MSD simplifications rules for Rel-19, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc, Nokia.

- R4-2420380 WF on MSD rule, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc, Murata Corporation Ltd., Nokia, Qualcomm France.

- R4-2407156 Guidelines for FDD PC2 Harmonic MSD Analysis, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, Skyworks Solutions, Inc.

- R4-2413068 Update on Harmonic MSD simplification, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, The Netherlands, Skyworks Solutions, Inc.

- R4-2419699 1UL HPUE NR-CA MSD Simplifications for Rel-19, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

- R4-2419712 1UL HPUE NR-CA MSD Observations, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

- R4-2219723 Preliminary REFSENS results for 1Tx and 2Tx PC2 FDD bands, 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, Skyworks Solutions, Inc.

- R4-2215660, MSD analysis for CA_n3-n78 with PC2 n3 UL, 3GPP TSG-RAN WG4 Meeting #104bis-e, Electronic Meeting, Apple.

- R4-2215895, On CA band combination of DL CA_n3-n78 with UL PC2 n3, 3GPP TSG-RAN WG4 Meeting #104bis-e, Electronic Meeting, ZTE Corporation, China Unicom.

- R4- 2302731, MSD for CA_n3-n78 with PC2 n3 UL, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, Huawei, HiSilicon.

- R4-2503239, Unified HPUE 1UL/2UL MSD simplification proposals for Rel-19, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, China, Skyworks Solutions, Inc.