# 3GPP TSG-RAN WG4 Meeting # 114b                                                         R4-2503239

# Wuhan, China, 7 – 11 April 2025

# Title:	Unified HPUE 1UL/2UL MSD simplification proposals for Rel-19

# Agenda Item:	7.1.1.1.1

# Source:	Skyworks Solutions, Inc., Samsung

# Document for:	Approval

# Introduction

The discussion paper presents a combined set of look-up tables (LUT) to simplify MSD requirements all inter-band CA MSD types, both for 1UL and 2UL HPUE configurations based on [1,2].

# Discussion

Proposal 1: For new Rel-19 inter-band CA combinations, and for all valid HPUE 1UL and 2UL configurations, specify the MSD requirements for 1UL MSD due to harmonic, harmonic mixing, cross-band isolation and for 2UL IMD2, IMD3, IMD4, IMD5 as PCx MSD = PC31Tx MSD + MSD,

where x = 2 (1Tx/2Tx) or 1.5 (2Tx), and where MSD is specified with eight “MSDmax” look-up tables (LUT),

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ ≤0.3 | 0.3 | 0.4 | 1.0 | 2.8 | 4.3 | 6.3 | 11.0 | 16.5 |
| 0.4≤ ≤0.6 | 0.5 | 0.8 | 1.8 | 4.2 | 6.1 | 8.4 | 13.6 | 19.3 |
| 0.7≤ ≤0.9 | 0.7 | 1.1 | 2.5 | 5.1 | 7.2 | 9.7 | 15.1 | 21.0 |
| 1.0≤ ≤1.9 | 1.2 | 2.1 | 4.1 | 7.1 | 9.6 | 12.3 | 18.1 | 24.0 |
| 2.0≤ ≤2.9 | 1.6 | 2.7 | 5.1 | 8.2 | 10.9 | 13.7 | 19.6 | 25.5 |
| 3.0≤ ≤3.9 | 1.8 | 3.3 | 5.9 | 8.9 | 11.7 | 14.6 | 20.5 | 26.5 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 | 9.5 | 12.4 | 15.3 | 21.3 | 27.3 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 | 10.4 | 13.4 | 16.3 | 22.3 | 28.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 | 11.0 | 14.0 | 16.9 | 22.9 | 28.9 |
| ≥10 | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |

and where the mapping of 1UL/2UL MSD type to the “MSDmax” LUT is specified in the companion table:

| MSD type |  | MSDmax (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 1UL | UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |  |  |  |  |  |
|  | Rx mixing | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |
|  | Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |
| 2UL | IMD2 | PC21 | PC2 PC1.52Tx1 |  | PC1.52Tx |  |  |  |  |
|  | IMD3 | PC21 |  | PC2 PC1.52Tx1 |  |  | PC1.52Tx |  |  |
|  | IMD4 | PC21 |  |  | PC2 PC1.52Tx1 |  |  | PC1.52Tx |  |
|  | IMD5 | PC21 |  |  |  | PC2 PC1.52Tx1 |  |  | PC1.52Tx |
| Note 1: This MSD requirement applies to 2DL band CA combinations where the victim’s band achieves PC2 with 1Tx. |  |  |  |  |  |  |  |  |  |

Proposal 2: Handling of the Rel-19 NRCA band combinations which have no PC3 MSD but may have small PC2 or PC1.5 MSD:

- TPs for TR or draft CRs for PC3 which do not propose an MSD for PC3 shall be flagged IF a company/delegate believes a HPUE MSD is needed, otherwise HPUE MSD is not specified.

Option 1:

- The 1UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b and the  2UL in the legacy MSD tables 7.3A.5-1a, 7.3A.5-1b, 7.3A.5-2a ,7.3A.5-2b with the first entry of the applicable MSDmax LUT. For example, based on the 1st entry of the 3dB MSDmax LUT, the PC2 1Tx MSD is specified at 0.3dB for all 1UL MSD types, and at 0.3dB for IMD2,3,4,5 of 2DL band combinations where the victim’s band achieved PC2 with 1Tx.

Option 2:

- As an exception, and because the Option 1 MSD levels are small, the 1UL HPUE MSD requirements are not specified.

- The 2UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.5-1a, 7.3A.5-1b, 7.3A.5-2a ,7.3A.5-2b with the first entry of the applicable MSDmax LUT and specify only HPUE MSD >6dB. For example, the PC1.5 IMD5 MSD is specified at 16.5dB based on 1st entry of 30dB MSDmax LUT. However, the PC1.5 IMD2 MSD is not specified.

Proposal 3: To reduce the conformance test burden of HPUE band combinations, consider the following two additional simplifications:

- RAN5 to verify only the MSD of the highest supported power class; and

- RAN5 to skip the MSD verification of the second “optional” MSD test points.

# Conclusion

Based on observations and proposals from [1,2], this paper proposes a single “unified” set of HPUE simplification look-up tables (LUT) for all MSD types and for both 1UL and 2UL HPUE configurations.

Proposal 1: For new Rel-19 inter-band CA combinations, and for all valid HPUE 1UL and 2UL configurations, specify the MSD requirements for 1UL MSD due to harmonic, harmonic mixing, cross-band isolation and for 2UL IMD2, IMD3, IMD4, IMD5 as PCx MSD = PC31Tx MSD + MSD,

where x = 2 (1Tx/2Tx) or 1.5 (2Tx), and where MSD is specified with eight “MSDmax” look-up tables (LUT),

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ ≤0.3 | 0.3 | 0.4 | 1.0 | 2.8 | 4.3 | 6.3 | 11.0 | 16.5 |
| 0.4≤ ≤0.6 | 0.5 | 0.8 | 1.8 | 4.2 | 6.1 | 8.4 | 13.6 | 19.3 |
| 0.7≤ ≤0.9 | 0.7 | 1.1 | 2.5 | 5.1 | 7.2 | 9.7 | 15.1 | 21.0 |
| 1.0≤ ≤1.9 | 1.2 | 2.1 | 4.1 | 7.1 | 9.6 | 12.3 | 18.1 | 24.0 |
| 2.0≤ ≤2.9 | 1.6 | 2.7 | 5.1 | 8.2 | 10.9 | 13.7 | 19.6 | 25.5 |
| 3.0≤ ≤3.9 | 1.8 | 3.3 | 5.9 | 8.9 | 11.7 | 14.6 | 20.5 | 26.5 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 | 9.5 | 12.4 | 15.3 | 21.3 | 27.3 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 | 10.4 | 13.4 | 16.3 | 22.3 | 28.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 | 11.0 | 14.0 | 16.9 | 22.9 | 28.9 |
| ≥10 | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |

and where the mapping of 1UL/2UL MSD type to the “MSDmax” LUT is specified in the companion table:

| MSD type |  | MSDmax (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 1UL | UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |  |  |  |  |  |
|  | Rx mixing | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |
|  | Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |
| 2UL | IMD2 | PC21 | PC2 PC1.52Tx1 |  | PC1.52Tx |  |  |  |  |
|  | IMD3 | PC21 |  | PC2 PC1.52Tx1 |  |  | PC1.52Tx |  |  |
|  | IMD4 | PC21 |  |  | PC2 PC1.52Tx1 |  |  | PC1.52Tx |  |
|  | IMD5 | PC21 |  |  |  | PC2 PC1.52Tx1 |  |  | PC1.52Tx |
| Note 1: This MSD requirement applies to 2DL band CA combinations where the victim’s band achieves PC2 with 1Tx. |  |  |  |  |  |  |  |  |  |

Proposal 2: Handling of the Rel-19 NRCA band combinations which have no PC3 MSD but may have small PC2 or PC1.5 MSD:

- TPs for TR or draft CRs for PC3 which do not propose an MSD for PC3 shall be flagged IF a company/delegate believes a HPUE MSD is needed, otherwise HPUE MSD is not specified.

Option 1:

- The 1UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b and the  2UL in the legacy MSD tables 7.3A.5-1a, 7.3A.5-1b, 7.3A.5-2a ,7.3A.5-2b with the first entry of the applicable MSDmax LUT. For example, based on the 1st entry of the 3dB MSDmax LUT, the PC2 1Tx MSD is specified at 0.3dB for all 1UL MSD types, and at 0.3dB for IMD2,3,4,5 of 2DL band combinations where the victim’s band achieved PC2 with 1Tx.

Option 2:

- As an exception, and because the Option 1 MSD levels are small, the 1UL HPUE MSD requirements are not specified.

- The 2UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.5-1a, 7.3A.5-1b, 7.3A.5-2a ,7.3A.5-2b with the first entry of the applicable MSDmax LUT and specify only HPUE MSD >6dB. For example, the PC1.5 IMD5 MSD is specified at 16.5dB based on 1st entry of 30dB MSDmax LUT. However, the PC1.5 IMD2 MSD is not specified.

Proposal 3: To reduce the conformance test burden of HPUE band combinations, consider the following two additional simplifications:

- RAN5 to verify only the MSD of the highest supported power class; and

- RAN5 to skip the MSD verification of the second “optional” MSD test points.

# References

- R4-2503238 On 1UL HPUE MSD simplifications, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, China, Skyworks Solutions, inc.

- R4-2503169 Simplification of HPUE 2UL DeltaMSD, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, China, Skyworks Solutions, inc.