3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503242

Wuhan, China, April 7th – 11th, 2025

# Title:	Clarifications on valid 1UL E-UTRA configuration for EN-DC intra-band contiguous MSD

# Agenda Item:	6.2.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution discusses the ambiguity of RAN4 one uplink (1UL) MSD EN-DC MSD test points where the E-UTRA UL carrier configuration parameters are specified as “N/A”. An EN-DC link cannot be established without a E-UTRA UL connectivity; hence we believe that some clarifications are needed.

- Discussion

  - Background on the introduction of using “N/A” for UL configurations in MSD tables

To our understanding, RAN4 started to systematically and consistently introduce ‘N/A” to avoid triple uplink configurations for two-band uplink (UL) to three downlink (DL) band MSD requirements due to dual UL IMD interference. This was agreed with CRs [1,2] for EN-DC/NR-CA. The first row of such clarification is reproduced below for the example of DC_1-3_n28, where the legacy requirements meant that MSD is verified with three uplink carriers (B1, B3 and band n28), which is not a valid UL configuration [1].

# Since then, this approach has been consistently implemented for all subsequent 2UL IMD MSD test points. It has also propagated through the EN-DC intra-band contiguous REFSENS/MSD requirements Table 7.3B.2.1-1. As a reminder, these requirements are verified with 1UL carrier in the NR SCG to verify the impact of transmitter chain, non-linearities on the MSD of the LTE MCG MSD. Since MSD is evaluated with only 1UL carrier, it was agreed that the E-UTRA UL parameters be specified as “N/A”. The example of DC_(n)3AA (1st entry of the table) is reproduced below in Table 1 and simplified RF landscape in Figure 1.

Table 1: DC_(n)3AA example of “N/A” specification for E-UTRA UL carrier (Table 7.3B.2.1-1).

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |

# Figure 1: Sketch of UL/DL carrier configuration for DC_(n)3AA LTE B3 MSD with 1UL carrier in band n3.

  - How to interpret RAN4 “N/A” UL configurations for conformance testing?

The LTE B3 DL MSD throughput measurements cannot be established without an LTE uplink carrier since in EN-DC the LTE carrier is the anchor carrier that transports both the control plane and the user plane payload. It is not clear how would RAN5 interpret RAN4 “N/A” specifications of the LTE UL carrier frequency and RB allocation. Since dual-UL transmissions are needed to establish the radio bearers, it is our understanding that RAN5 will interpret the “N/A” as “configure the LTE UL carrier and UL RB allocation as carrier agnostic”, i.e., configure the UL carrier to ensure the LTE MSD is verified strictly under 1UL IMD products, and not dual-UL IMD products.

But then, it is not clear what RF parameters should RAN5 configure for the LTE UL cell. To our knowledge, this RAN4 agreement was never captured in any formal document (Way Forward or PRD rules), or even communicated to RAN5. It may be necessary to send a clarification LS to RAN5.

Observation 1: EN-DC intra-band contiguous MSD requirements with 1UL carrier in the NR SCG band requires dual-UL connectivity: E-UTRA MCG + NR SCG. It is not clear how RAN5 configures the E-UTRA UL carrier to verify such 1UL MSD requirements for cases where the E-UTRA UL RF parameters “Fc” (carrier frequency) and “LCRB” (RB allocation”) are specified as “N/A”. An LS to RAN5 may be needed.

Proposal 1: For EN-DC intra-band contiguous MSD requirements with 1UL in the NR band:

- Do not change the E-UTRA UL parameters “Fc” and “LCRB” specified as “N/A”,

- Add a footnote to “N/A” to clarify that the E-UTRA UL carrier should be configured as carrier agnostic,

- Send an LS explaining to RAN5 how to configure the E-UTRA UL parameters so that the E-UTRA DL carrier is not affected by 2UL IMD interference,

- Exact contents of the LS are to be further discussed. The intention is to provide recommendations on:

  - E-UTRA UL Fc: “RAN4 recommends that the E-UTRA Fc is configured according to the LTE band’s default Tx-Rx frequency separation”,

  - E-UTRA UL RB allocation: “RAN4 recommends that the E-UTRA UL RB allocation is configured according to the UL configuration specified the band’s REFSENS for the corresponding DL CBW”,

  - E-UTRA transmit power:“RAN4 recommends configuring the E-UTRA UL as carrier agnostic”.

Example based on DC_(n)3AA.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)3AA | 3 | N/AX | 15 | N/AX | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. NOTE X: The E-UTRA uplink carrier shall be configured as carrier agnostic. |  |  |  |  |  |  |  |

- Conclusion

This paper proposes to send an LS to RAN5 to clarify the RAN4 meaning of E-UTRA UL parameters specified as ‘N/A” for EN-DC intra-band contiguous MSD requirements verifies with 1UL in the NR band.

Observation 1: EN-DC intra-band contiguous MSD requirements with 1UL carrier in the NR SCG band requires dual-UL connectivity: E-UTRA MCG + NR SCG. It is not clear how RAN5 configures the E-UTRA UL carrier to verify such 1UL MSD requirements for cases where the E-UTRA UL RF parameters “Fc” (carrier frequency) and “LCRB” (RB allocation”) are specified as “N/A”. An LS to RAN5 may be needed.

Proposal 1: For EN-DC intra-band contiguous MSD requirements with 1UL in the NR band:

- Do not change the E-UTRA UL parameters “Fc” and “LCRB” specified as “N/A”,

- Add footnote to the “N/A” to clarify that the E-UTRA UL carrier should be configured as carrier agnostic,

- Send an LS explaining to RAN5 how to configure the E-UTRA UL parameters so that the E-UTRA DL carrier is not affected by 2UL IMD interference,

- Exact contents of the LS are “to be further discussed”. The intention is to provide recommendations on:

  - E-UTRA UL Fc:“RAN4 recommends that the E-UTRA Fc is configured according to the LTE band’s default Tx-Rx frequency separation”

  - E-UTRA UL RB allocation: example: “RAN4 recommends that the E-UTRA UL RB allocation is configured according to the UL configuration specified the band’s REFSENS for the corresponding DL CBW”

  - E-UTRA transmit power:, “RAN4 recommends configuring the E-UTRA UL as carrier agnostic”.

- References

- R4-2311136 CR to TS 38.101-3 Rel-18 Removing triple uplink configurations, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, Skyworks Solutions, inc.

- R4-2311138 CR to TS 38.101-1 Rel-18 Removing triple uplink configurations and corrections to 2UL CA exceptions, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, Skyworks Solutions, inc.