# 3GPP TSG RAN WG4 Meeting#114-bis	R4-2503248

# Wuhan, CN, Apr 7 – Apr 11, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for 6Rx devices

Agenda item:	7.1.3

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, it was agreed that 6-layer performance requirements will be defined for FWA devices. Additionally, RAN4 will introduce 4-layer performance requirements applicable to FWA and handheld UE devices [1]. In this paper, we present our perspectives on the simulation assumptions for defining PDSCH demodulation performance requirements.

2. Simulation Assumptions

2.1 Correlation Across Receive Antennas

FWA devices have a significantly larger form factor compared to handheld UE devices. Consequently, it is natural to assume low correlation across receive antennas for FWA requirements. Additionally, measured correlation values for handheld devices indicate that the correlation across receive antennas has minimal impact on performance requirements. Moreover, standardizing measured correlation for RAN4 requirements is challenging. Therefore, we propose using low correlation for performance requirements across FWA and handheld devices.

Proposal 1: Adopt 3GPP low correlation for 6-layer demodulation performance requirements applicable to FWA devices.

Proposal 2: Adopt 3GPP low correlation for 4-layer demodulation performance requirements applicable to FWA and handheld UE devices.

2.2 Channel Model

For 8Rx devices, RAN4 considered TDLA30-10 as the channel model for both 8-layer and 4-layer requirements. Additionally, TDLA30-10 channel model was used during the 6-layer feasibility study for 6Rx devices. Therefore, we believe that the same channel model can be assumed for demodulation performance requirements.

Proposal 3: Consider TDLA30-10 for both 6-layer and 4-layer demodulation performance requirements for 6Rx devices.

2.3 PDSCH DMRS Configurations

We note that Rel-15 Type1 DMRS supports only four orthogonal DMRS ports with one frontloaded DMRS symbol. In contrast, Rel-18 eType1 DMRS allows configuration of up to eight orthogonal DMRS ports with one frontloaded DMRS symbol. Extensive discussions took place during the 6-layer feasibility study regarding PDSCH DMRS configurations. The DMRS configurations considered were: Rel-15 2+0, Rel-15 2+2, and Rel-18 1+1 for 6-layer performance study.

We note that eType1 DMRS is likely to become more prevalent in advanced 5G use cases due to its reduced overhead and better support for higher-order MIMO. Additionally, RAN4 has already introduced requirements for eType1 DMRS during the Rel-18 timeframe [2]. Additionally, based on the simulation results provided in our contribution for RAN4#114 [3], it is evident that Rel-18 eType1 DMRS offers comparable performance with just one frontloaded DMRS symbol compared to the Rel-15 2+0 case. It also provides robustness in high Doppler scenarios due to the additional DMRS symbol. While the Rel-15 2+2 DMRS configuration could be considered for 6Rx requirements, it is important to note that this configuration significantly increases the DMRS overhead, resulting in lower peak throughput. After evaluating all these options and analyzing their pros and cons, we believe that eType1 DMRS should be considered for 6-layer performance requirements for 6Rx devices.

Proposal 4: Adopt eType1 1+1 DMRS configuration for 6-layer demodulation performance requirements for 6Rx devices.

2.4 Modulation

During the 6-layer feasibility study, we considered 256QAM modulation order (i.e., Table 2). We believe that 256QAM modulation order can also be considered for 6-layer and 4-layer PDSCH demodulation performance requirements. However, selecting the MCS will necessitate conducting a simulation campaign.

Proposal 5: Consider 256QAM modulation order for 6-layer and 4-layer demodulation performance requirements.

3. Conclusions

In this paper, we share our perspectives on the simulation assumptions for the 6Rx PDSCH demodulation performance requirements. The following proposals have been made.

Proposal 1: Adopt 3GPP low correlation for 6-layer demodulation performance requirements applicable to FWA devices.

Proposal 2: Adopt 3GPP low correlation for 4-layer demodulation performance requirements applicable to FWA and handheld UE devices.

Proposal 3: Consider TDLA30-10 for both 6-layer and 4-layer demodulation performance requirements for 6Rx devices.

Proposal 4: Adopt eType1 1+1 DMRS configuration for 6-layer demodulation performance requirements for 6Rx devices.

Proposal 5: Consider 256QAM modulation order for 6-layer and 4-layer demodulation performance requirements.

4. References

[1] R4-2503014, “WF on requirements for 6Rx,” 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, Feb 17 – 21, 2025.

[2] TS 38.101-4, ‘User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)’, v18.6.0, Jan 2025.

[3] R4-2500115, “Discussion on the demodulation performance requirements for 6Rx devices,” 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, Feb 17 – 21, 2025.