3GPP TSG-RAN4 Meeting #114bis	R4-2503255

# Wuhan China, 7th – 11th April, 2025

Source: 	Qualcomm Incorporated

Title: 	CSI Compression: Testability and Interoperability

Agenda item:	7.20.2

Document for:	Discussion

# Introduction

In this paper, we present two types of simulation results:

- Results with fully specified option 3 based on mixed dataset of track 2 with fully specified decoder

- Results with partially specified option 4 based on mixed dataset of track 2 with the reference encoder

We also provide our views regarding option 4, in general.

# Past Agreements

## Model of Encoder-Decoder Pair

Following agreements occurred during the email discussion after RAN4 111 [1].

Based on the two rounds of discussions, the following parameters are proposed: Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. Table 1. CNN model parameters for encoder. Model architecture parameters for encoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 47M (vs. 1.6M for eTypeII). Model size; 448Kbyte. Num of Resnet blocks; 6. Layer after resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,32). Size of message (feature map) passed across layers; (13,32). Table 2. CNN model parameters for decoder. Model architecture parameters for decoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 740M. Model size; 7.2Mbyte. Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,128). Size of message (feature map) passed across layers; (13,32). Other model related parameters: Activation function; ReLU. Latent message size; 32 float values. Payload size, bits; 64 per layer.. Quantization; 2 bits. System parameters: Frequency range; 2GHz (4GHz optional). MIMO Layers; 1 layer (2 layers optional, companies to. Sub-bands; 13. Channel estimation; Ideal. Metrics; SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI.. Companies are also invited to provide the training parameters used.

Following agreements happened a couple of meeting ago [2].

Parameter/Configurations; Configuration for alignment. Channel/data-set related. UE antenna element assumptions; Ideal omni-directional antenna elements following RAN1 assumptions. Sub-band channel averaging; - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected. Any other pre-processing; - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1.. Data collection assumptions; - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs.. Training dataset size; - Aim to use the dataset around 600K. - At least 100K sample to be used.. Dataset quantization; - Float32. Other data-set characteristics/statistics; - Companies to propose. - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII.. Model-related. Quantization of encoder output; - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization. Normalization before quantization in the encoder; - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional.. Use and configuration of drop-out; - Not to use drop-out layers. Training loss function, use of regularization; - Use (1-SGCS) as loss function as reference. Optimizer; - Use Adam. Learning rate. - Fixed learning rate of 10^-4. Batch size. - 256. (Max) Number of epochs. - 150 epochs. Stopping criteria. - No improvement over 25 epochs.. Weight initialization. - Default. Kernel Parameters: - Dilation - Padding Type The value of groups (VG); - Default. Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

RAN4 agreed to the following during the last meeting [3]:

Issue 4-1: Simulation results discussion for Option 3 … Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

## System Parameters

The system parameter assumptions of 38.843 were considered to be baseline for RAN4 performance evaluation.

Table 6.2.1-1: Baseline System Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g., UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz as baseline, optional for 4GHz (if R16 as baseline) FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz (if R17 as baseline) |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R16 as baseline) 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R17 as baseline) |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g., 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | At least, FTP model 1 with packet size 0.5 Mbytes is assumed. Other options are not precluded |
| Traffic load (Resource utilization) |  | 20/50/70%. Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) CSI prediction: 100% outdoor (10, 20, 30, 60, 120 km/h) including outdoor-to-indoor car penetration loss per TR 38.901 if the simulation assumes UEs inside vehicles. No explicit trajectory modeling considered for evaluations. |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline. Up to companies to choose the error modelling method for realistic channel estimation. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation. |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. - For AI/ML based solutions: The above-mentioned "CSI feedback overhead" is calculated as max allowed bits at the given rank. - For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB - Option 2a: The above-mentioned "CSI feedback overhead" is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned "CSI feedback overhead" is calculated as max allowed bits at the given rank Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | For CSI compression: Companies need to report which option is used between: - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. Additional assumptions from R17 TypeII EVM: Same consideration with respect to utilizing angle-delay reciprocity should be considered for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline. Optionally, Type I Codebook (if it outperforms Type II Codebook) can be considered for comparing AI/ML schemes. For CSI-prediction: Both of the following are taken as baseline: - The nearest historical CSI without prediction - Non-AI/ML or AI/ML with collaboration Level x based CSI prediction for which corresponding details would need to be reported Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example. For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison. |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |  |

## Different Options for Fully and Partially Specified Encoeder-Decoder Pair

RAN4 agreed to the following recently [4].

Issue 4-1: Option 3 (fully specified decoder) next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

Issue 1-2: Option 3 next step - decoder(s) selection choose the decoders with Low, medium, high SGCS Agreement: among the decoders submitted by companies Continue the feasibility study on the two tracks below: Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

RAN4 made the following agreement regarding reference encoder decoder pair a couple of meeting ago [4]

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

Issue 1-3: Option 4a(Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures) Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset

Issue 1-4: Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - Step 6: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

# Discussion

## Simulation Results with Option 3

Table 1: Results with Track 2, Option 3 and different Encoder-Decoder pairing

| Enc-Model | Dec-model (frozen) | Train dataset | Test dataset | SGCS | Ref SGCS (Samsung joint training) |
| --- | --- | --- | --- | --- | --- |
| QC-Enc with quantization | Samsung-Dec with quantization | Mixed | Mixed | 0.7005 | 0.7016 |
| QC-Enc without quantization | Samsung-Dec without quantization | Mixed | Mixed | 0.752 | 0.7478 |

The email discussion after RAN4 114 meeting agreed to use Samsung decoder as the frozen decoder for track 2 of the fully specified option. Companies were also requested to try Nokia decoder as the frozen decoder for this track. We have used Samsung decoder as the frozen decoder in this part.

Table 1 shows the SGCS results with track 2, Samsung joint decoder and Qualcomm encoder. Following steps were performed to generate the results of Table 1.

- Samsung’s decoder is obtained from [9] (uploaded after RAN4 114 meeting) and considered as a frozen decoder.

- Qualcomm’s encoder is trained with Samsung’s frozen decoder based on mixed dataset of [10] (generated and uploaded after RAN4 114 meeting)

- Trained Qualcomm encoder from previous step and Samsung’s frozen decoder is tested based on mixed dataset of [10]

Table 1 shows that the SGCS obtained with separate encoder decoder training (QC encoder with Samsung’s frozen decoder) is almost identical to the SGCS obtained with Samsung’s joint encoder-decoder training. Hence, fully specified decoder in track 2 with mixed dataset seems to be a feasible approach.

Observation 1: the SGCS obtained with separate encoder decoder training (QC encoder with Samsung’s frozen decoder) is almost identical to the SGCS obtained with Samsung’s joint encoder-decoder training.

Observation 2: Option 3 (fully specified decoder) leads to good SGCS in, at least, the scenario where encoder is trained with the same dataset as the decoder.

## Simulation Results with Option 4

RAN4 agreed to the following sub-options for option 4 in one of the previous meetings.

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

The following steps were suggested during the email discussion after RAN4 114 to evaluate the feasibility of option 4a:

- Take Samsung encoder and mixed dataset. Use these to “label” the (encoder input, encoder output) training dataset. Train a new decoder using the labelled dataset. Then train an “own” encoder with the newly trained decoder

- Then all companies share their “new” decoders that have been trained with the Samsung labelled dataset

- Test “own” encoder vs. All other companies decoders from step 2. (The other companies decoders here represent different TE vendors...)

We perform step 1 to train our “own” encoder and then test “own” encoder with Samsung decoder. Table 2 and 3 show the results of

Table 2: Results with Partially Specified Approach and encoder-decoder based on quantization  (mixed dataset and reference encoder)

| Step | Encoder model | Decoder model | Train dataset | Test dataset SGCS | Ref SGCS (Samsung joint training) |
| --- | --- | --- | --- | --- | --- |
| 1 | Samsung encoder without quantization (frozen) | QC “nominal” decoder without quantization | Mixed | -- | -- |
| 2 | QC “own” encoder without quantization | QC “nominal” decoder without quantization | Mixed | -- | -- |
| 3 | QC “own” encoder without quantization | Samsung decoder without quantization (frozen) | Test only | 0.7005 | 0.7016 |

Table 3: Results with Partially Specified Approach without quantization (mixed dataset and reference encoder)

| Step | Encoder model | Decoder model | Train dataset | Test dataset SGCS | Ref SGCS (Samsung joint training) |
| --- | --- | --- | --- | --- | --- |
| 1 | Samsung encoder without quantization (frozen) | QC “nominal” decoder without quantization | Mixed | -- | -- |
| 2 | QC “own” encoder without quantization | QC “nominal” decoder without quantization | Mixed | -- | -- |
| 3 | QC “own” encoder without quantization | Samsung decoder without quantization (frozen) | Test only | 0.754 | 0.7478 |

Table 2 and Table 3 show that Qualcomm’s “own” encoder, generated by training with Qualcomm’s “nominal” decoder that was generated based on the labelled (encoder input, encoder output) training dataset, achieves good SGCS values.

Qualcomm’s “own” encoder now needs to be tested with other companies’ “nominal” decoders, that were derived from the labelled training dataset, to evaluate the feasibility of option 4a.

Observation 3: Option 4a leads to good SGCS when Qualcomm’s “own” encoder, derived through a two-step approach from the labelled training dataset, is tested with Samsung decoder and mixed dataset. The feasibility of option 4a will depend on how Qualcomm’s “own” encoder performs other companies’ “nominal” decoders that were generated based on labelled dataset.

      - Note 1: “Labelled” dataset refers to (encoder input, encoder output)

      - Note 2: “Labelled dataset” was generated based on mixed dataset and Samsung reference encoder.

## General Discussion regarding Option 4 (Partially Specified Approach with Reference Encoder)

In option 4b, TE first trains its decoder based on reference encoder. UE is later expected to develop a nominal decoder that works well with the reference encoder and then train its encoder based on the nominal decoder. Later, UE’s developed encoder gets tested with TE’s developed decoder.

Observations from Track 1 results suggest that performance will be poor if TE and UE develop their decoder and encoder respectively with completely different training datasets. Use of same training dataset during encoder and decoder will guarantee good SGCS performance.

Previous section showed that option 4a with labelled dataset achieved good SGCS. However, it required a reference encoder. Hence, training dataset (i.e., channel information) and reference encoder both need to be specified to obtain good SGCS.

If channel information and reference encoder are both specified in option 4b, then option 4b becomes a superset of option 4a where (channel information, encoder output) is specified. Hence, RAN4 doesn’t need to evaluate two approaches in option 4. RAN4 can just evaluate one approach in this option where both training dataset (channel information) and reference encoder are specified.

Eventually, a reference decoder might be useful to ensure good performance in this approach. Specifying a reference decoder will allow UE to train its encoder in one step instead of the current two step process where it first develops a nominal decoder and then trains its own developed encoder based on the nominal decoder.

Observation 4: In option 4b,

- TE is expected to train its decoder based on reference encoder.

- UE is expected to develop a nominal decoder that works well with the reference encoder and then train its encoder based on the nominal decoder.

- There will be mismatch between TE and UE if the training dataset, i.e., channel information, is not specified.

Observation 5: Training dataset, i.e., channel information, and reference encoder both need to be specified in option 4b.

Observation 6: If channel information and reference encoder are specified in option 4b, then option 4b becomes a superset of option 4a where (channel information, encoder output) is specified.

Proposal 1: RAN4 specifies both training dataset (channel information) and reference encoder in option 4 and focuses only on this option for the partially specified approach.

Proposal 2: RAN4 also considers specifying a reference decoder so that UE can derive its encoder through one step and TE get a guideline to implement its decoder during the tests.

# Conclusions

Observation 1: The SGCS obtained with separate encoder decoder training (QC encoder with Samsung’s frozen decoder) in option 3 with track 2 mixed dataset is almost identical to the SGCS obtained with Samsung’s joint encoder-decoder training.

Observation 2: Option 3 (fully specified decoder) leads to good SGCS in, at least, the scenarios where encoder is trained with the same dataset as the decoder.

Observation 3: Option 4a leads to good SGCS when Qualcomm’s “own” encoder, derived through a two-step approach from the labelled training dataset, is tested with Samsung decoder and mixed dataset. The feasibility of option 4a will depend on how Qualcomm’s “own” encoder performs other companies’ “nominal” decoders that were generated based on labelled dataset.

      - Note 1: “Labelled” dataset refers to (encoder input, encoder output)

      - Note 2: “Labelled dataset” was generated based on mixed dataset and Samsung reference encoder.

Observation 4: In option 4b,

- TE is expected to train its decoder based on reference encoder.

- UE is expected to develop a nominal decoder that works well with the reference encoder and then train its encoder based on the nominal decoder.

- There will be mismatch between TE and UE if the training dataset, i.e., channel information, is not specified.

Observation 5: Training dataset, i.e., channel information, and reference encoder both need to be specified in option 4b.

Observation 6: If channel information and reference encoder are specified in option 4b, then option 4b becomes a superset of option 4a where (channel information, encoder output) is specified.

Proposal 1: RAN4 specifies both training dataset (channel information) and reference encoder in option 4 and focuses only on this option for the partially specified approach.

Proposal 2: RAN4 also considers specifying a reference decoder in option 4 so that UE can derive its encoder through one step and TE can get a guideline to implement its decoder during the tests.

# References

[1] R4-[111][133] NR_AIML_air-email discussions after RAN4#111 meeting, https://list.etsi.org/scripts/wa.exe?A2=3GPP_TSG_RAN_WG4;aa5369d9.2406C&S=

[2] R4-2414447, “Alignment of model training and data set parameters for AI/ML based CSI compression”, Nokia,  3GPP TSG-RAN4 Meeting #112.

[3] R4-2413169, “CSI Compression: Testability and Interoperability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.

[4] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112

[5] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #112bis.

[6] R4-2415889, “CSI Compression: Testability and Interoperability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112-bis

[7]https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_113/Inbox/Drafts/%5B113%5D%5B129%5D%20NR_AIML_air_part1/CSI_Compression_Results_Summary/%5B113%5D%5B129%5DAIML_Summary_of_CSI_Compression_Sim_Results_v3_Nokia2_QC2.xlsx

[8]https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_113

[9] https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Models/R4_114

[10] https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_114